3GPP TSG-RAN WG3 Meeting #125	R3-244850

Maastricht, Netherlands, 19 – 23 August, 2024

Title:	(pCR to TR 38.769) Study on solutions for Ambient IoT in NR

Agenda Item:	16.1

Source:	Huawei, CMCC

Document for:	pCR

# 1	Introduction

This contribution provides the baseline pCR to the TR 38.769 from RAN3 perspective, captures the agreed TPs listed in [1] ~ [6].

# 2	Reference

- R3-243962 [TP for TR 38.769] CB:#AIoT1_Architecture, Ericsson (Moderator), Huawei

- R3-243963 Inventory and Command between AIoT CN amd AIoT RAS, Huawei, CMCC, Nokia, Ericsson, ZTE, Xiaomi, Qualcomm Incorporated, Samsung, CATT, Lenovo, LG Electronics, NEC

- R3-243964 (TP for TR 38.769) Locating Ambient-IoT device, CMCC, ZTE, CATT, Xiaomi, LGE, Huawei, Lenovo, NEC

- R3-244837 (TP for TR 38.769) RAN architecture aspects, Huawei

- R3-244838 (TP for TR38.769) NG interface impact for Ambient-IoT, ZTE Corporation, Xiaomi

- R3-244840 (TP for TR 38.769) Locating Ambient-IoT device, CMCC

# 3		Text Proposal to TR 38.769

------------Start of the Change-------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.848: "Study on Ambient IoT (Internet of Things) in RAN".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AIoT	Ambient IoT

AIoT RAN	AIoT Radio Access Network

DO-A	Device-originated autonomous

DO-DTT	Device-originated by device-terminated trigger

DT	Device-terminated

FR	Frequency Range

IoT	Internet of Things

LPWA	Low-power, wide-area

LTE-MTC	Long Term Evolution – Machine Type Communication

NB-IoT	Narrowband IoT

RFID	Radio frequency identification

SFO	Sampling frequency offset

# 4	Evaluation methodology

Editor’s note: These sub-clauses correspond to Objective 1a, 1b, 1d, respectively.

## 4.1	Remaining details of RAN design targets

## 4.2	Evaluation assumptions

## 4.3	Link budget template

# 5	Ambient IoT device architectures

## 5.1	~1 µW devices (Device 1)

## 5.2	≤few hundred µW devices

### 5.2.1	External carrier wave (Device 2a)

### 5.2.2	Internally-generated carrier wave (Device 2b)

# 6	Solutions for ambient IoT

## 6.1	Physical layer

## 6.2	Protocol stack and signalling procedures

Editor’s note: Corresponds to the RAN2 objective in the SID.

## 6.3	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID.

## 6.3	RAN architecture aspects

Editor’s note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary.

This chapter attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

Editor’s Note 2: What functionalities are hosted by the 5GS for AIoT is TBD.

This chapter also attempts to identify a functional split between RAN and CN.

The logical system architecture for AIoT consists of the following architectural elements:

AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2].

Editor’s Note 3: Further details FFS, if any.

AIoT RAN: hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

AIoT radio: radio interface between AIoT device and AIoT RAN node in topology 1 and between AIoT device and AIoT-enabled UE in topology 2.

Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2.

AIoT CN: hosts certain functions for AIoT as of the functional split between RAN and CN.

NOTE: the details of AIoT CN are subject to SA2.

Editor’s Note 6: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface: interface between the AIoT RAN and the AIoT CN on which certain AIoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAN and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAN, or one instance for NG alone.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

AIoT RAN node function: a function that contains e.g. the control of the AIoT radio resources used towards the AIoT device.

Editor’s Note 9: further details are FFS. Note that “control of AIoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

### 6.3.1 Support of Topology 1

Figure 6.3.1-1 depicts a logical system architecture for topology 1, where the Common reader function and AIoT RAN node function are deployed within an AIoT RAN.

Figure 6.3.1-1 Logical system architecture for topology 1

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Figure 6.3.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport:

Figure 6.3.1-2. Protocol Stack for Topology 1

Editor’s Note 1: Figure 6.3.1-2 serves as a starting point for further discussions.

For topology 1, the XXAP is terminated at an AIoT RAN node.

Editor’s Note 2: the signalling transport for XXAP is FFS.

Editor’s Note 3: The protocol stack does not detail how AIoT upper layer information is transported over XXAP, details are pending on SA2 agreements. And the AIoT CN may include AMF and AIoT related functions which is also up to SA2 decision.

Editor’s Note 4: aspects of interaction between upper layer information exchange and XXAP in order to trigger the AIoT RAN node functions are FFS.

### 6.3.2 Support of Topology 2

Figure 6.3.2-1 depicts a logical system architecture for topology 2, wherethe Common reader function is located at anAIoT-enabled UE, and the AIoT RAN node function is located at an AIoT-enabled gNB.

The following definitions apply:

AIoT-enabled gNB: a gNB supporting AIoT RAN node function, which is able to communicate with the AIoT enabled UE via NR Uu interface.

AIoT-enabled UE: a UE supporting Common reader function, which is able to communicate with the AIoT device via the AIoT radio interface.

Figure 6.3.2-1 Logical system architecture for topology 2

Editor’s Note 1:	Figure 6.3.2-1 doesn’t illustrate the protocol between AIoT enabled UE and AIoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

Editor’s Note 2:	In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Editor’s Note 3:	The AIoT CN could include AMF and AIoT related functions. This is up to SA2 decision.

Editor’s Note 4:	The AIoT enabled gNB performs radio resource management for AIoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

#### 6.3.2.1 Solutions for Topology 2

To support Topology 2, the following solutions are to be studied for conveying AIoT upper layer information:

-	RRC based solution. With this solution, AIoT CN applies AIoT upper layer information explicitly over XXAP signaling. AIoT upper layer information is then relayed explicitly to/from the AIoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of AIoT upper layer information at AIoT-enabled gNB. AIoT upper layer information is transmitted over AIoT-enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of AIoT upper layer information at AIoT-enabled gNB. AIoT upper layer information is transmitted as AIoT-enabled UE's user plane data.

Editor’s note 1: how to enable radio resource control, i.e. trigger AIoT RAN node functions for above solutions is FFS.

Editor’s note 2: depiction and further details of the options above are FFS.

## 6.4	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

### 6.4.1	Information exchanged between AIoT CN and AIoT RAN

#### 6.4.1.1	Inventory

Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the AIoT CN to the AIoT RAN, may include the following:

(1)	AIoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: It is FFS whether AIoT RAN needs to interpret/store/process it.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

Multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single Inventory Report.

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

#### 6.4.1.2	Command

Command can be sent by the AIoT CN for a single device.

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether AIoT RAN can remain agnostic of the type of request from the AIoT CN (need to differentiate command and inventory)

### 6.4.2	Signaling and Procedures for Topology 1

#### 6.4.2.1 Candidate procedures for AIoT Inventory for Topology 1

Editor’s note 1: Future discussions on AIoT Inventory will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc.

Figure 6.4.2.1-1: Message flow for AIoT Inventory in Topology 1

### 6.4.3	Signaling and Procedures for Topology 2

#### 6.4.3.1 Candidate procedures for AIoT Inventory for Topology 2

Editor’s note 1: Future discussions on AIoT Inventory will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc.

Figure 6.4.3.1-1: Message flow for AIoT Inventory in Topology 2 (if RRC-based solution is used)

Figure 6.4.3.1-2: Message flow for AIoT Inventory in Topology 2 (if NAS/UP based solution is used)

Editor’s note 2: how and where to depict signalling suitable for triggering AIoT RAN node functions for AIoT radio resource management needs further discussions for direct communication between AIoT CN and AIoT-enabled UE.

## 6.5	Coexistence of ambient IoT and NR/LTE

Editor’s note: Corresponds to the first RAN4 objective in the SID.

## 6.6	RF requirements study

Editor’s note: Corresponds to the second RAN4 objective in the SID.

## 6.7	Characteristics of carrier-wave waveform

## 6.8	Locating ambient IoT devices

Editor’s note 1: Proximity determination may be in a 6.8.x sub-clause, or another arrangement, depending on how the study proceeds.

Editor’s note 2: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

### 6.8.1	General

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by AIoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

For topology 1, AIoT RAN node ID can be considered as a location of AIoT device. For topology 2, UE ID of the AIoT-enabled UE can be considered as a location of AIoT device.

Editor’s Note 1: How to know the “reader” location is FFS. Whether to use more than one “readers” within one AIoT RAN for location purposes is FFS.

Editor’s Note 2: Signalling details on how to provide the location information of the AIoT device to the AIoT CN needs further study.

Editor’s Note 3: Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4: Analysising the gap between the positioning requirements in the SID and the feasibility for AIoT devices.

# 7	Coverage evaluations

# 8	Conclusions and recommendations

------------End of the Change-------------