3GPP TSG-RAN WG3 Meeting #126	R3-247760

Orlando, USA,  18 – 22 November, 2024

Agenda item:	31.2

Source:	Nokia, Ericsson, BT

Title:	Response to R3-247276, R3-247277, R3-247278

Document for:	Response

# 1	Introduction

We here provide response to [1-3].

# 2	Discussion

Concerning observation 1 and proposals 1-1 and 1-2 of [1]:

Observation 1: gNB-DU has the sufficient information to decide using cell DTXDRX or not.

Proposal 1-1: The gNB-DU decides to use cell DTX/DRX and parameters of cell DTX/DRX.

Proposal 1-2: The gNB-DU decides to release cell DTX/DRX.

Although such agreement was not taken explicitly in work item phase, we agree with the above observation and proposals which also reflect the de facto situation, considering that no F1 signalling has been defined for cell DTX/DRX operation.

According to [1], the gNB will activate or deactivate cell DTX/DRX based on cell load. However, due to the impact on the scheduler, we believe that gNB-DU implementations should be left with full freedom in their decisions on cell DTX/DRX pattern (within the limits of constraints imposed by the standard) and activation/deactivation. Factors like respect of QoS constraints and highly dynamic constraints related to served UEs’ radio conditions could therefore be taken into account, combined with the aspect of optimizing energy efficiency. This potentially leads to dynamic activation and deactivation decisions for cell DTX, made possible by RAN1’s decisions on L1 signalling for activation and deactivation using the DCI format 2_9 introduced in Rel-18. This DCI format enables separate activation/deactivation of cell DTX and cell DRX (for the case where both are configured), and cell DTX/DRX capable UEs monitor the DCI format 2_9 in the common search space of the serving cell​.

It is therefore important to ensure that network signalling for cell DTX/DRX doesn’t limit the gNB-DUs possibilities to take autonomous and dynamic decisions on activation and deactivation of cell DTX and cell DRX operation.

Observation 1: It is important that network signalling for cell DTX/DRX doesn’t limit the gNB-DUs possibilities to take autonomous and dynamic decisions on activation and deactivation of cell DTX and cell DRX operation.

Another important point is that 3GPP specifications don’t mandate that cell DTX/DRX operation has to be activated and deactivated for all UEs in a cell. In fact it is decided on the UE groups.

Observation 2: 3GPP specifications don’t mandate that cell DTX/DRX operation has to be activated and deactivated for all UEs in the cell.

In section 2.2 of [1], the following is observed:

Observation 2: gNB-CU can remove the unsuitable UEs when it knows cell DTXDRX is configured and parameters of the cell DTXDRX configuration.

There are here two aspects:

- which cell DTX/DRX information would be needed in the gNB-CU for NES CHO (conditional handover triggered by NES event)

- which cell DTX/DRX information would be needed in the gNB-CU for handling of UEs e.g. not supporting NES CHO or which are ‘unsuitable’ for cell DTX/DRX operation

On the first point, NES CHO is described in TS 38.300 clause 15.4.2.4:

The same principle as described in 9.2.3.4 applies to conditional handover in case the source cell is using a network energy saving solution (e.g., the cell is activating cell DTX/DRX or turning off), unless hereunder specified. In this case, the following additional triggering conditions are supported, upon which UE may use NES-specific CHO event for executing CHO to a candidate cell, as defined in TS 38.331 [12]:

-	The UE may be notified via DCI to enable CHO conditions(s) configured with NES event indication.

As can be seen, NES CHO is configured to a UE before e.g. cell DTX/DRX is activated or before its serving cell turns off. Also, RAN3 clarified the mechanisms behind NES CHO configuration based on different earlier papers [4-6]. We don’t exclude that some network signalling support for this purpose could be considered, e.g. for the case of cell DTX/DRX applied in a capacity cell with coverage entirely ensured by a coverage layer, but the solution submitted in [3] to this meeting is not suitable for this purpose, e.g. as per our observation 2 above. Also, the cell DTX/DRX pattern would not be needed.

Observation 3: Network signalling support for NES CHO submitted to this meeting is not suitable or needed for this purpose.

On the second point, it should first be noticed that cell DTX/DRX is designed as a backwards compatible feature. A cell operating in cell DTX/DRX mode may therefore continue to serve legacy (pre-Rel-18) UEs and other UEs not supporting cell DTX/DRX operation although with reduced energy saving potential. It is also important that the gNB-CU remains free to optimize load according to e.g. radio conditions of the UE which is important to maintain good service, and also has important impact on the energy consumption both for the network and for the UE. So, similarly as for our observation 3 above, one could consider some network support, but the cell DTX/DRX pattern would not be needed for this purpose.

Observation 4: Network signalling support to enable the gNB-CU to “remove unsuitable UEs” submitted to this meeting is not suitable or needed for this purpose.

It is further observed and proposed in [1]:

Observation 3: The information of cell DTX/DRX is also beneficial for gNB-CU to derive the real resource status of the NES cell, or do interference elimination between different cells.

Proposal 2: The gNB-DU sends the gNB-CU the parameters of cell DTX/DRX configuration when cell DTX/DRX is configured or modified, or indicate the gNB-CU that cell DTX/DRX is released via F1AP messages.

[1] claims that “if the cell DTX/DRX information is delivered over F1, the gNB-CU can use it to derive the real PRB number used during the on-duration time of the NES cell in conjunction with the resource status report from the gNB-DU on the NES cell”. We strongly believe that the gNB-CU should not be encouraged to determine any “real” load or available capacity based on the cell DTX/DRX pattern information only. This would either create an overcompensation in the gNB-CU to keep the cell load unnecessarily low (assuming cell DTX/DRX operation 100% of the time), or create a need for the gNB-CU to be continuously updated of the cell DTX/DRX activation status hence resulting in significant signalling load.

A signalling solution including probability indicator as proposed in [7,8] could be used to solve this problem.

The same argument holds for interference mitigation in the scenario of cell DTX/DRX operation in neighbour cells operating at the same frequency. In most deployments each cell has a higher number of neighbours than the number of orthogonal cell DTX/DRX patterns. E.g. if one imagines a cell DTX/DRX duty cycle of ~30%, there would in principle be 3 orthogonal DTX/DRX patterns, while the number of neighbour cells is typically significantly higher. For a workable solution to really mitigate interference, a cell should therefore adapt its own cell DTX/DRX pattern, or usage of cell DTX/DRX, by taking into account the usage probability information received. For example, a neighbour cell indicating close to 100% active duration usage probability in the active duration will create significantly more interference than a cell indicating a lower number e.g. 40%.

Observation 5: Sending cell DTX/DRX activation information over network interfaces will create undesirable over-compensation actions in the gNB-CU or generate significant signalling load over F1 and Xn if gNB-DU flexibility of dynamic cell DTX/DRX decisions is maintained.

Observation 6: A workable solution for interference mitigation requires usage probability information sent together with the activation status.

Proposal: Include usage probability information sent together with the activation status over F1 and Xn.

We include corresponding XnAP and F1AP CRs in annex of this paper.

# 3	Conclusion

We have made the following observations and proposal:

Observation 1: It is important that network signalling for cell DTX/DRX doesn’t limit the gNB-DUs possibilities to take autonomous and dynamic decisions on activation and deactivation of cell DTX and cell DRX operation.

Observation 2: 3GPP specifications don’t mandate that cell DTX/DRX operation has to be activated and deactivated for all UEs in the cell.

Observation 3: Network signalling support for NES CHO configuration submitted to this meeting is not suitable or needed for this purpose.

Observation 4: Network signalling support to enable the gNB-CU to “remove unsuitable UEs” submitted to this meeting is not suitable or needed for this purpose.

Observation 5: Sending cell DTX/DRX activation information over network interfaces will create undesirable over-compensation actions in the gNB-CU or generate significant signalling load over F1 and Xn if gNB-DU flexibility of dynamic cell DTX/DRX decisions is maintained.

Observation 6: A workable solution for interference mitigation requires usage probability information sent together with the activation status.

Proposal: Include usage probability information sent together with the activation status over F1 and Xn.

# References

[1]		R3-247276, Support cell DTXDRX for NES, CATT, ZTE, NEC

[2]		R3-247277, Support Cell DTX/DRX for NES, XnAP CR, CATT, ZTE, NEC

[3]		R3-247278, Support Cell DTX/DRX for NES, F1AP CR, CATT, ZTE, NEC

[4]	R3-240546, (TPs to 38.473 and 38.401) NES CHO awareness in source gNB, Qualcomm Incorporated (RAN3#123)

[5]	R3-241534, Conditional Handover Request for NES, F1AP CR, CATT (RAN3#123bis)

[6]	R3-241904, Correction to 38.473 on NES CHO awareness at source gNB-DU, F1AP CR, Qualcomm Incorporated (RAN3#123bis)

[7]	R3-244467, Introduction of cell DTX/DRX information, XnAP CR, Nokia, Ericsson, BT (RAN3#125)

[8]	R3-244346, Introduction of cell DTX/DRX information, F1AP CR, Ericsson, Nokia, BT (RAN3#125)

# Annex A – XnAP CR

3GPP TSG-RAN WG3 Meeting #126	R3-24xxxx

# Orlando, USA,  18 – 22 November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | xx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of cell DTX/DRX information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson, BT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Cell DTX/DRX is supported for network energy saving in the gNB but inter-node coordination is currently missing. A gNB which receives cell DTX/DRX configuration information from a neighbour gNB may, for example, use the information to adopt non-overlapping cell DTX/DRX patterns if possible, or patterns with reduced overlap, in order to reduce inter-cell interference. Such inter-node coordination also needs to take into account that 3GPP specifications don’t mandate that cell DTX/DRX has to be activated and deactivated for all UEs in the cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added cell DTX/DRX information in the NG-RAN NODE CONFIGURATION UPDATE message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-node coordination for cell DTX/DRX not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2, 9.1.3.4, 9.2.2.x1 (new), 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.4.2	NG-RAN node Configuration Update

#### 8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending the NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the NG-RAN NODE CONFIGURATION UPDATE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells To Add IE and in the Served NR Cells To Modify IE.

If Supplementary Uplink is configured at the NG-RAN node2, the NG-RAN node2 shall include in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells IE if any.

If the TAI Support List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message, the receiving node shall replace the previously provided TAI Support List IE by the received TAI Support List IE.

If the Cell Assistance Information NR IE is present, the NG-RAN node2 shall, if supported, use it to generate the Served NR Cells IE and include the list in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

If the Cell Assistance Information E-UTRA IE is present, the NG-RAN node2 shall, if supported, use it to generate the Served E-UTRA Cells IE and include the list in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

If the Partial List Indicator NR IE is included in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message and set to "partial" the NG-RAN node1 shall, if supported, assume that the Served NR Cells IE in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message includes a partial list of NR cells.

If the Partial List Indicator E-UTRA IE is included in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message and set to "partial" the NG-RAN node1 shall, if supported, assume that the Served E-UTRA Cells IE in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message includes a partial list of NR cells.

If the Cell and Capacity Assistance Information NR IE is present in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message from the candidate NG-RAN node2, the NG-RAN node1 shall, if supported, store the collected information to be used for future NG-RAN node interface management.

If the Cell and Capacity Assistance Information E-UTRA IE is present in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message from the candidate NG-RAN node2, the NG-RAN node1 shall, if supported, store the collected information to be used for future NG-RAN node interface management.

Upon reception of the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall update the information for NG-RAN node1 as follows:

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the NG-RAN NODE CONFIGURATION UPDATE message and the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

If the TNL Configuration Info IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall take this IE into account for IPSec establishment. In case the IP-Sec Transport Layer Address IE within the Extended UP Transport Layer Addresses To Add List IE is present and the GTP Transport Layer Address Info IE within the GTP Transport Layer Addresses To Add List IE is not empty, GTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in the IP-Sec Transport Layer Address IE. In case the IP-Sec Transport Layer Address IE is not present, GTP traffic is terminated at the endpoints given by the list of addresses in the GTP Transport Layer Address Info IE within the GTP Transport Layer Addresses To Add List IE if present.

If the TNL Configuration Info IE is contained in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 shall take this IE into account for IPSec establishment. In case the IP-Sec Transport Layer Address IE within the Extended UP Transport Layer Addresses To Add List IE is present and the GTP Transport Layer Address Info IE within the GTP Transport Layer Addresses To Add List IE is not empty, GTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in the IP-Sec Transport Layer Address IE. In case the IP-Sec Transport Layer Address IE is not present, GTP traffic is terminated at the endpoints given by the list of addresses in the GTP Transport Layer Address Info IE within the GTP Transport Layer Addresses To Add List IE if present.

If the CSI-RS Transmission Indication IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall take this IE into account for neighbour cell’s CSI-RS measurement.

The NG-RAN NODE CONFIGURATION UPDATE message may contain for each cell served by NG-RAN node1 NPN related broadcast information. The NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message may contain for each cell served by NG-RAN node2 NPN related broadcast information.

If the Additional Measurement Timing Configuration List IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall take this IE into account for neighbour cell’s CSI-RS measurement.

If the Local NG-RAN Node Identifier IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, take this into account for future retrieval of the UE contexts from the NG-RAN node1.

If the Local NG-RAN Node Identifier IE is present in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 shall, if supported, take this into account for future retrieval of the UE contexts from the NG-RAN node2.

If the Neighbour NG-RAN Node List IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may take this into account for Local NG-RAN Node Identifier conflict detection.

If the Neighbour NG-RAN Node List IE is present in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 may take this into account for Local NG-RAN Node Identifier conflict detection.

If the Local NG-RAN Node Identifier Removal IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, discard it from its context and not use it for future retrieval of the UE contexts from the NG-RAN node1.

If the Local NG-RAN Node Identifier Removal IE is present in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 shall, if supported, discard it from its context and not use it for future retrieval of the UE contexts from the NG-RAN node2.

If the Served Cell Specific Info Request IE is included in the NG-RAN NODE CONFIGURATION UPDATE message and if the NG-RAN node2 is a gNB, the NG-RAN node2 shall, if supported, include the Additional Measurement Timing Configuration List IE for the requested NR cells in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

If the TAI NSAG Support List IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node shall, if supported, take this IE into account for slice aware cell reselection.

If the TAI Slice Unavailable Cell List IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, take this IE into account to deduce slice resource allocation.

If the Cell DTX/DRX Information IE is included in the NG-RAN NODE CONFIGURATION UPDATE message, the receiving NG-RAN node may take this IE into account to configure neighbour cells’ cell DTX/DRX pattern.

Update of Served Cell Information NR:

-	If Served Cells NR To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall add cell information according to the information in the Served Cell Information NR IE.

-	If Served Cells NR To Modify IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall modify information of cell indicated by Old NR-CGI IE according to the information in the Served Cell Information NR IE.

-	When either served cell information or neighbour information of an existing served cell in NG-RAN node1 need to be updated, the whole list of neighbouring cells, if any, shall be contained in the Neighbour Information NR IE. The NG-RAN node2 shall overwrite the served cell information and the whole list of neighbour cell information for the affected served cell.

-	If the Deactivation Indication IE set to "deactivated" is contained in the Served Cells NR To Modify IE, it indicates that the concerned cell was switched off to lower energy consumption.

-	If Served Cells NR To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall delete information of cell indicated by Old NR-CGI IE.

-	If the Intended TDD DL-UL Configuration NR IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 should take this information into account for cross-link interference management and/or NR-DC power coordination with the NG-RAN node1. The NG-RAN node2 shall consider the received Intended TDD DL-UL Configuration NR IE content valid until reception of a new update of the IE for the same NG-RAN node2.

-	If the NR Cell PRACH Configuration IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use this information for RACH optimisation.

-	If the SFN Offset IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE shall, if supported, use this information to update the SFN0 time offset of the reported cell.

-	If the Supported MBS FSA ID List IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use it according to TS 38.300 [9].

-	If the RedCap Broadcast Information IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

-	If the eRedCap Broadcast Information IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

-	If the Mobile IAB Cell IE is included in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message or the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the receiving NG-RAN node may use it accordingly.

-	If the XR Broadcast Information IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

Update of Served Cell Information E-UTRA:

-	If Served Cells E-UTRA To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall add cell information according to the information in the Served Cell Information E-UTRA IE.

-	If Served Cells E-UTRA To Modify IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall modify information of cell indicated by Old ECGI IE according to the information in the Served Cell Information E-UTRA IE.

-	When either served cell information or neighbour information of an existing served cell in NG-RAN node1 need to be updated, the whole list of neighbouring cells, if any, shall be contained in the Neighbour Information E-UTRA IE. The NG-RAN node2 shall overwrite the served cell information and the whole list of neighbour cell information for the affected served cell.

-	If the Deactivation Indication IE set to "deactivated" is contained in the Served Cells E-UTRA To Modify IE, it indicates that the concerned cell was switched off to lower energy consumption.

-	If the Served Cells E-UTRA To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall delete information of cell indicated by Old ECGI IE.

-	If the Protected E-UTRA Resource Indication IE is included into the NG-RAN NODE CONFIGURATION UPDATE (inside the Served Cell Information E-UTRA IE), the receiving gNB should take this into account for cell-level resource coordination with the ng-eNB. The gNB shall consider the received Protected E-UTRA Resource Indication IE content valid until reception of a new update of the IE for the same ng-eNB. The protected resource pattern indicated in the Protected E-UTRA Resource Indication IE is not valid in subframes indicated by the Reserved Subframes IE (contained in E-UTRA - NR CELL RESOURCE COORDINATION REQUEST messages), as well as in the non-control region of the MBSFN subframes i.e. it is valid only in the control region therein. The size of the control region of MBSFN subframes is indicated in the Protected E-UTRA Resource Indication IE.

-	If the PRACH Configuration IE is contained in the Served Cell Information E-UTRA IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use this information for RACH optimisation.

-	If the NPRACH Configuration IE is contained in the Served Cell Information E-UTRA IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use this information for RACH optimisation.

-	If the SFN Offset IE is contained in Served Cell Information E-UTRA IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE shall, if supported, use this information to update the SFN0 time offset of the reported cell.

Update of TNL addresses for SCTP associations:

If the TNLA To Add List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, use it to establish the TNL association(s) with the NG-RAN node1. If the TNLA To Add List IE does not include the Port Number IE, the NG-RAN node2 shall assume that port number value 38422 is used for the endpoint. The NG-RAN node2 shall report to the NG-RAN node1, in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the successful establishment of the TNL association(s) with the NG-RAN node1 as follows:

-	A list of successfully established TNL associations shall be included in the TNLA Setup List IE;

-	A list of TNL associations that failed to be established shall be included in the TNLA Failed to Setup List IE.

If the TNLA To Remove List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message the NG-RAN node2 shall, if supported, initiate removal of the TNL association(s) indicated by the received Transport Layer information towards the NG-RAN node1.

-	If the received TNLA Transport Layer Address IE includes the Port Number IE, the NG-RAN node1 TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the NG-RAN node1 TNL endpoints correspond to all NG-RAN node1 TNL endpoints identified by the Endpoint IP Address IE and any Port Number(s).

If the TNLA To Update List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message the NG-RAN node2 shall, if supported, update the TNL association(s) indicated by the received Transport Layer information towards the NG-RAN node1.

-	If the received TNLA Transport Layer Address IE includes the Port Number IE, the NG-RAN node1 TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the NG-RAN node1 TNL endpoints correspond to all NG-RAN node1 TNL endpoints identified by the Endpoint IP Address IE and any Port Number(s).

Update of AMF Region Information:

-	If AMF Region Information To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall add the AMF Regions to its AMF Region List.

-	If AMF Region Information To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall remove the AMF Regions from its AMF Region List.

Update of Cell Coverage:

If the Coverage Modification List IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information in the Cell Coverage State IE to identify the cell deployment configuration enabled by the NG-RAN node1 and for configuring the mobility towards the cell(s) indicated by the Global NG-RAN Cell Identity IE, as described in TS 38.300 [9].

-	If the Cell Deployment Status Indicator IE is present in the Coverage Modification List IE, the NG-RAN node2 shall consider the cell deployment configuration of the cell to be modified as the next planned configuration and shall remove any planned configuration stored for this cell.

-	If the Cell Deployment Status Indicator IE is present and the Cell Replacing Info IE contains non-empty cell list, the NG-RAN node2 may use this list to avoid connection or re-establishment failures during the reconfiguration, e.g. consider the cells in the list as possible alternative handover targets.

-	If the Cell Deployment Status Indicator IE is not present, the NG-RAN node2 shall consider the cell deployment configuration of cell to be modified as activated and replace any previous configuration for the cells indicated in the Coverage Modification List IE.

If the SSB Coverage Modification List IE is present in the Coverage Modification List IE, the NG-RAN node2 may use the information in the SSB Coverage State IE to identify the SSB beam deployment configuration enabled by the NG-RAN node1 and for configuring the mobility towards the beam(s) indicated by the SSB Index IE, as described in TS 38.300 [9].

If the Coverage Modification Cause IE set to "coverage" or "cell edge capacity" is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information for deducing the CCO issue detected at NG-RAN node1 and for configuring coverage state of its served cell(s).

If the Coverage Modification Cause IE set to "network energy saving" is present and the SSB Coverage State IE is zero for a set of SSB beams in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information to decide the SSB beam activation for the concerned beams when necessary.

Interactions with other procedures:

If the NG-RAN node1 receives a NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message containing a Local NG-RAN Node Identifier identical to the Local NG-RAN Node Identifier included in the corresponding NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node1 may initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE message a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

If the NG-RAN node1 receives a NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message containing a Local NG-RAN Node Identifier within the Neighbour NG-RAN Node List IE identical to the Local NG-RAN Node Identifier included in the corresponding NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node1 may initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE message a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

#### 8.4.2.3	Unsuccessful Operation

Figure 8.4.2.3-1: NG-RAN node Configuration Update, unsuccessful operation

If the NG-RAN node2 cannot accept the update it shall respond with the NG-RAN NODE CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the NG-RAN NODE CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the NG-RAN node1 shall wait at least for the indicated time before reinitiating the NG-RAN Node Configuration Update procedure towards the same NG-RAN node2. Both nodes shall continue to operate the Xn with their existing configuration data.

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the NG-RAN NODE CONFIGURATION UPDATE message and the NG-RAN NODE CONFIGURATION UPDATE FAILURE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

#### 8.4.2.4	Abnormal Conditions

If the NG-RAN node1 after initiating NG-RAN node Configuration Update procedure receives neither NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message nor NG-RAN NODE CONFIGURATION UPDATE FAILURE message, the NG-RAN node1 may reinitiate the NG-RAN node Configuration Update procedure towards the same NG-RAN node2, provided that the content of the new NG-RAN NODE CONFIGURATION UPDATE message is identical to the content of the previously unacknowledged NG-RAN NODE CONFIGURATION UPDATE message.

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| CHOICE Initiating NodeType | M |  |  |  | YES | ignore |
| >gNB |  |  |  |  |  |  |
| >>Served Cells To Update NR | O |  | 9.2.2.15 |  | YES | ignore |
| >>Cell Assistance Information NR | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA | O |  | 9.2.2.43 |  | YES | ignore |
| >>Served Cell Specific Info Request | O |  | 9.2.2.102 |  | YES | ignore |
| >ng-eNB |  |  |  |  |  |  |
| >>Served Cells to Update E-UTRA | O |  | 9.2.2.16 |  | YES | ignore |
| >>Cell Assistance Information NR | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA | O |  | 9.2.2.43 |  | YES | ignore |
| TNLA To Add List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Add Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >>TNL Association Usage | M |  | 9.2.3.84 |  | – |  |
| TNLA To Update List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Update Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >>TNL Association Usage | O |  | 9.2.3.84 |  | – |  |
| TNLA To Remove List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| Global NG-RAN Node ID | O |  | 9.2.2.3 |  | YES | reject |
| AMF Region Information To Add | O |  | AMF Region Information 9.2.3.83 | List of all added AMF Regions to which the NG-RAN node belongs. | YES | reject |
| AMF Region Information To Delete | O |  | AMF Region Information 9.2.3.83 | List of all deleted AMF Regions to which the NG-RAN node belongs. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  | 9.2.3.96 |  | YES | ignore |
| Coverage Modification List |  | 0 .. 1 |  | List of cells with modified coverage. | GLOBAL | reject |
| >Coverage Modification Item |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | Global Cell Identity 9.2.2.73 | Global Cell Identity of the cell to be modified. In this version of the specification, only a NG-RAN cell identifier can be included. | – |  |
| >>Cell Coverage State | M |  | INTEGER (0..63, …) | Value ‘0’ indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | – |  |
| >>Cell Deployment Status Indicator | O |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration. | – |  |
| >>Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  | – |  |
| >>>Replacing Cells |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>>>Global NG-RAN Cell Identity |  |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of a cell that may replace all or part of the coverage of the cell to be modified. | – |  |
| >>SSB Coverage Modification List |  | 0..1 |  | List of SSB beams with modified coverage. | – |  |
| >>>SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. | – |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value ‘0’ indicates that the SSB beam is inactive. Other values Indicates that the SSB beam is active and also indicates the coverage configuration of the concerned SSB beam. | – |  |
| >>Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the reason for the coverage modification in NG-RAN node1. | YES | ignore |
| Local NG-RAN Node Identifier | O |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Local NG-RAN Node Identifier Removal | O |  | Local NG-RAN Node Identifier 9.2.2.101 |  | YES | Ignore |
| Cell DTX/DRX Information List |  | 0 .. 1 |  | List of cells with modified cell DTX/DRX information. | GLOBAL | ignore |
| >Cell DTX/DRX Information Item |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | Global Cell Identity 9.2.2.73 | Global Cell Identity of the cell to be modified. In this version of the specification, only a NG-RAN cell identifier can be included. | – |  |
| >>Cell DTX/DRX Information | M |  | 9.2.2.x1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the NG RAN nodes. Value is 32. |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |

#### 9.2.2.x1	Cell DTX/DRX Information

This IE indicates cell DTX/DRX information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE On-Duration Timer | M |  |  | See cellDTXDRX-onDurationTimer defined in TS 38.331 [10]. |
| >sub MilliSeconds |  |  |  |  |
| >>Sub MilliSeconds Timer | M |  | INTEGER (1..31) |  |
| >milliSeconds |  |  |  |  |
| >>MilliSeconds Timer | M |  | ENUMERATED (ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200, ms1600, ... ) |  |
| CHOICE Cycle Start Offset | M |  |  | See cellDTXDRX-CycleStartOffset defined in TS 38.331 [10]. |
| >ms10 |  |  |  |  |
| >>Offset ms10 | M |  | INTEGER (0..9) |  |
| >ms20 |  |  |  |  |
| >>Offset ms20 | M |  | INTEGER (0..19) |  |
| >ms32 |  |  |  |  |
| >>Offset ms32 | M |  | INTEGER (0..31) |  |
| >ms40 |  |  |  |  |
| >>Offset ms40 | M |  | INTEGER (0..39) |  |
| >ms60 |  |  |  |  |
| >>Offset ms60 | M |  | INTEGER (0..59) |  |
| >ms64 |  |  |  |  |
| >>Offset ms64 | M |  | INTEGER (0..63) |  |
| >ms70 |  |  |  |  |
| >>Offset ms70 | M |  | INTEGER (0..69) |  |
| >ms80 |  |  |  |  |
| >>Offset ms80 | M |  | INTEGER (0..79) |  |
| >ms128 |  |  |  |  |
| >>Offset ms128 | M |  | INTEGER (0..127) |  |
| >ms160 |  |  |  |  |
| >>Offset ms160 | M |  | INTEGER (0..159) |  |
| >ms256 |  |  |  |  |
| >>Offset ms256 | M |  | INTEGER (0..255) |  |
| >ms320 |  |  |  |  |
| >>Offset ms320 | M |  | INTEGER (0..319) |  |
| >ms512 |  |  |  |  |
| >>Offset ms512 | M |  | INTEGER (0..511) |  |
| >ms640 |  |  |  |  |
| >>Offset ms640 | M |  | INTEGER (0..639) |  |
| >ms1024 |  |  |  |  |
| >>Offset ms1024 | M |  | INTEGER (0..1023) |  |
| >ms1280 |  |  |  |  |
| >>Offset ms1280 | M |  | INTEGER (0..1279) |  |
| >ms2048 |  |  |  |  |
| >>Offset ms2048 | M |  | INTEGER (0..2047) |  |
| >ms5120 |  |  |  |  |
| >>Offset ms5120 | M |  | INTEGER (0..5119) |  |
| >ms10240 |  |  |  |  |
| >>Offset ms10240 | M |  | INTEGER (0..10239) |  |
| Slot Offset | M |  | INTEGER (0..31) | See cellDTXDRX-SlotOffset defined in TS 38.331 [10]. |
| Config Type | M |  | ENUMERATED (dtx, drx, dtxdrx, ...) | See cellDTXDRXconfigType defined in TS 38.331 [10]. |
| Active Duration Usage Probability | M |  | INTEGER (0..100) | Probability in percent that the resource is used during the active duration (see TS 38.300 [9] clause 15.4.2.3). Value 0 corresponds to 0% probability, and value 100 corresponds to 100% probability. |
| Non-Active Duration Usage Probability | M |  | INTEGER (0..100) | Probability in percent that the resource is used during the non-active duration (see TS 38.300 [9] clause 15.4.2.3). Value 0 corresponds to 0% probability, and value 100 corresponds to 100% probability. |

### 9.3.4	PDU Definitions

<<< skip unchanged text >>>

NodeAssociatedInfoResult,

SLPositioning-Ranging-Services-Info,

PDUSessionsListToBeReleased-UPError,

UserPlaneFailureIndication,

CellDTXDRXInfoList

FROM XnAP-IEs

id-ReportingPeriodicityForDataCollection,

id-NodeAssociatedInfoResult,

id-SLPositioning-Ranging-Services-Info,

id-PDUSessionsListToBeReleased-UPError,

id-UserPlaneFailureIndication,

id-CellDTXDRXInfoList,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells,

maxnoofSSBAreas

FROM XnAP-Constants;

<<< skip unchanged text >>>

-- **************************************************************

--

#### -- NG-RAN NODE CONFIGURATION UPDATE

--

-- **************************************************************

NGRANNodeConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ NGRANNodeConfigurationUpdate-IEs}},

...

}

NGRANNodeConfigurationUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-TAISupport-list							CRITICALITY reject	TYPE TAISupport-List									PRESENCE optional }|

{ ID id-ConfigurationUpdateInitiatingNodeChoice	CRITICALITY ignore	TYPE ConfigurationUpdateInitiatingNodeChoice	PRESENCE mandatory}|

{ ID id-TNLA-To-Add-List						CRITICALITY ignore	TYPE TNLA-To-Add-List									PRESENCE optional }|

{ ID id-TNLA-To-Remove-List						CRITICALITY ignore	TYPE TNLA-To-Remove-List								PRESENCE optional }|

{ ID id-TNLA-To-Update-List						CRITICALITY ignore	TYPE TNLA-To-Update-List								PRESENCE optional }|

{ ID id-GlobalNG-RAN-node-ID					CRITICALITY reject	TYPE GlobalNG-RANNode-ID								PRESENCE optional }|

{ ID id-AMF-Region-Information-To-Add			CRITICALITY reject	TYPE AMF-Region-Information							PRESENCE optional }|

{ ID id-AMF-Region-Information-To-Delete		CRITICALITY reject	TYPE AMF-Region-Information							PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication					PRESENCE optional }|

{ ID id-TNLConfigurationInfo					CRITICALITY ignore	TYPE TNLConfigurationInfo								PRESENCE optional }|

{ ID id-Coverage-Modification-List				CRITICALITY reject	TYPE Coverage-Modification-List						PRESENCE optional }|

{ ID id-Local-NG-RAN-Node-Identifier			CRITICALITY ignore	TYPE Local-NG-RAN-Node-Identifier					PRESENCE optional }|

{ ID id-Neighbour-NG-RAN-Node-List				CRITICALITY ignore	TYPE Neighbour-NG-RAN-Node-List						PRESENCE optional }|

{ ID id-Local-NG-RAN-Node-Identifier-Removal	CRITICALITY ignore	TYPE Local-NG-RAN-Node-Identifier					PRESENCE optional }|

{ ID id-CellDTXDRXInfoList						CRITICALITY ignore	TYPE CellDTXDRXInfoList									PRESENCE optional },

...

}

### 9.3.5	Information Element definitions

<<< skip unchanged text >>>

CellDeploymentStatusIndicator ::= ENUMERATED {pre-change-notification, ...}

CellGroupID ::= INTEGER (0..maxnoofSCellGroups)

CellDTXDRXCycleStartOffset ::= CHOICE {

ms10					INTEGER(0..9),

ms20					INTEGER(0..19),

ms32					INTEGER(0..31),

ms40					INTEGER(0..39),

ms60					INTEGER(0..59),

ms64					INTEGER(0..63),

ms70					INTEGER(0..69),

ms80					INTEGER(0..79),

ms128					INTEGER(0..127),

ms160					INTEGER(0..159),

ms256					INTEGER(0..255),

ms320					INTEGER(0..319),

ms512					INTEGER(0..511),

ms640					INTEGER(0..639),

ms1024					INTEGER(0..1023),

ms1280					INTEGER(0..1279),

ms2048					INTEGER(0..2047),

ms2560					INTEGER(0..2559),

ms5120					INTEGER(0..5119),

ms10240					INTEGER(0..10239),

choice-extension		ProtocolIE-Single-Container { {CellDTXDRXCycleStartOffset-ExtIEs} }

}

CellDTXDRXCycleStartOffset-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellDTXDRXInfo ::= SEQUENCE {

onDurationTimer							CellDTXDRXOnDurationTimer,

cycleStartOffset						CellDTXDRXCycleStartOffset,

slotOffset                				INTEGER (0..31),

configType								ENUMERATED {dtx, drx, dtxdrx},

activeDurationUsageProbability			INTEGER (1..100),

nonActiveDurationUsageProbability		INTEGER (1..100),

iE-Extensions							ProtocolExtensionContainer { { CellDTXDRXInfo-ExtIEs} } OPTIONAL,

...

}

CellDTXDRXInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellDTXDRXInfoItem ::= SEQUENCE {

globalNG-RANCell-ID				GlobalCell-ID,

cellDTXDRXInfo					CellDTXDRXInfo,

iE-Extension					ProtocolExtensionContainer { { CellDTXDRXInfoItem-ExtIEs} } OPTIONAL,

...

}

CellDTXDRXInfoItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellDTXDRXInfoList ::= SEQUENCE (SIZE (0..maxnoofCellsinNG-RANnode)) OF CellDTXDRXInfoItem

CellDTXDRXOnDurationTimer ::= CHOICE {

subMilliSeconds 		INTEGER (1..31),

milliSeconds    		ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600,

...},

choice-extension		ProtocolIE-Single-Container { {CellDTXDRXOnDurationTimer-ExtIEs} }

}

CellDTXDRXOnDurationTimer-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellMeasurementResult ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellMeasurementResult-Item

### 9.3.7	Constant definitions

<<< skip unchanged text >>>

id-XR-Bcast-Information					 															ProtocolIE-ID ::= 465

id-PDUSessionsListToBeReleased-UPError																ProtocolIE-ID ::= 466

id-MaximumDataBurstVolume																			ProtocolIE-ID ::= 467

id-CPAC-Preparation-Type																			ProtocolIE-ID ::= 468

id-UserPlaneFailureIndication																		ProtocolIE-ID ::= 469

id-MN-only-MDT-collection																			ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-CellDTXDRXInfoList																				ProtocolIE-ID ::= xxx

END

-- ASN1STOP

# Annex B – F1AP CR

3GPP TSG-RAN WG3 Meeting #126	R3-24xxxx

# Orlando, USA,  18 – 22 November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | xx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of cell DTX/DRX information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, BT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Cell DTX/DRX is supported for network energy saving in the gNB but currently the information is not transferred from gNB-DU to gNB-CU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added cell DTXDRX information in F1 Setup and gNB-DU Configuration Update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for transferring Cell DTX/DRX information in the NG-RAN split architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.2, 8.2.4.2, 9.2.1.4, 9.2.1.7, 9.3.1.x, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR xx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

The gNB-DU initiates the procedure by sending a F1 SETUP REQUEST message including the appropriate data to the gNB-CU. The gNB-CU responds with a F1 SETUP RESPONSE message including the appropriate data.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the F1 interface is operational and other F1 messages may be exchanged.

If the F1 SETUP REQUEST message contains the gNB-DU Name IE, the gNB-CU may use this IE as a human readable name of the gNB-DU. If the F1 SETUP REQUEST message contains the Extended gNB-DU Name IE, the gNB-CU may use this IE as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if included.

If the F1 SETUP RESPONSE message contains the gNB-CU Name IE, the gNB-DU may use this IE as a human readable name of the gNB-CU. If the F1 SETUP RESPONSE message contains the Extended gNB-CU Name IE, the gNB-DU may use this IE as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if included.

If the F1 SETUP REQUEST message contains the gNB-DU Served Cells List IE, the gNB-CU shall take into account as specified in TS 38.401 [4].

For NG-RAN, the gNB-DU shall include the gNB-DU System Information IE and the TAI Slice Support List IE in the F1 SETUP REQUEST message.

The gNB-CU may include the Cells to be Activated List IE in the F1 SETUP RESPONSE message. The Cells to be Activated List IE includes a list of cells that the gNB-CU requests the gNB-DU to activate. The gNB-DU shall activate the cells included in the Cells to be Activated List IE and reconfigure the physical cell identity for cells for which the NR PCI IE is included.

If Cells to be Activated List Item IE is included in the F1 SETUP RESPONSE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the F1 SETUP RESPONSE message.

For NG-RAN, the gNB-DU may include the RAN Area Code IE in the F1 SETUP REQUEST message. The gNB-CU may use it according to TS 38.300 [6].

For NG-RAN, the gNB-DU may include Supported MBS FSA ID List IE in the Served Cell Information IE in the F1 SETUP REQUEST message. The gNB-CU may use it according to TS 38.300 [6].

For NG-RAN, the gNB-CU may include Available PLMN List IE, and optionally also Extended Available PLMN List IE in the F1 SETUP RESPONSE message, if the available PLMN(s) are different from what gNB-DU has provided in F1 SETUP REQUEST message, gNB-DU shall take this into account and only broadcast the PLMN(s) included in the received Available PLMN list(s).

For NG-RAN, the gNB-CU may include Available SNPN ID List IE in the F1 SETUP RESPONSE message. If the available SNPN(s) are different from what gNB-DU has provided in F1 SETUP REQUEST message, gNB-DU shall take this into account and only broadcast the SNPN(s) included in the received Available SNPN ID list.

The Latest RRC Version Enhanced IE shall be included in the F1 SETUP REQUEST message and in the F1 SETUP RESPONSE message.

If in F1 SETUP REQUEST message, the Cell Direction IE is present, the gNB-CU should use it to understand whether the cell is for UL or DL only. If in F1 SETUP REQUEST message, the Cell Direction IE is omitted in the Served Cell Information IE it shall be interpreted as that the Cell Direction is Bi-directional.

If the Intended TDD DL-UL Configuration IE is present in the F1 SETUP REQUEST message, the receiving gNB-CU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-CU may merge the Intended TDD DL-UL Configuration information received from two or more gNB-DUs. The gNB-CU shall consider the received Intended TDD DL-UL Configuration IE content valid until reception of an update of the IE for the same cell(s).

If the Aggressor gNB Set ID IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, take it into account.

If the Victim gNB Set ID IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, take it into account.

If the F1 SETUP REQUEST message contains the Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the SFN Offset IE is contained in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information to deduce the SFN0 offset of the reported cell.

If the F1 SETUP RESPONSE message contains the Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the F1 SETUP RESPONSE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the BAP Address IE is included in the F1 SETUP REQUEST message, without an accompanying RRC Terminating IAB-Donor gNB-ID IE, the receiving gNB-CU shall, if supported, consider the information therein for discovering the co-location of an IAB-DU and an IAB-MT.

If the F1 SETUP REQUEST message is received from an IAB-donor-DU, the gNB-CU shall, if supported, include the BAP Address IE in the F1 SETUP RESPONSE message.

NOTE:	How to identify the IAB-donor-DU is up to gNB-CU implementation.

If the F1 SETUP RESPONSE message contains the BAP Address IE, the gNB-DU shall, if supported, store the received BAP address and use it as specified in TS 38.340 [30].

If the NR PRACH Configuration List IE is included in the Served Cell Information IE contained in the F1 SETUP REQUEST message, the gNB-CU may store the information, and forward it to other RAN nodes for RACH optimisation. If the L139 Info IE included in the NR PRACH Configuration List IE is present, it shall contain the Root Sequence Index IE.

If the RedCap Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the eRedCap Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If both the RRC Terminating IAB-Donor gNB-ID IE and the BAP Address IE are included in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, consider that the BAP address indicated by the BAP Address IE is assigned by the gNB-CU of the RRC-terminating IAB-donor indicated by the RRC Terminating IAB-Donor gNB-ID IE, and use this BAP address and gNB-ID for the subsequent IAB Transport Migration Management procedure towards the RRC-terminating IAB-donor of the mobile IAB-node, as specified in TS 38.423 [28].

If the F1 SETUP REQUEST message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the NCGI to be Updated List IE is included in the F1 SETUP RESPONSE message, the gNB-DU shall, if supported, change the NCGI of the cell indicated by the Old NCGI IE to the NCGI indicated by the New NCGI IE.

If the Cell DTX/DRX Information IE is included in Cell DTX/DRX Information List IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, take the information into account when configuring cell DTX/DRX patterns.

#### 8.2.3.3	Unsuccessful Operation

Figure 8.2.3.3-1: F1 Setup procedure: Unsuccessful Operation

If the gNB-CU cannot accept the setup, it should respond with a F1 SETUP FAILURE and appropriate cause value.

If the F1 SETUP FAILURE message includes the Time To Wait IE, the gNB-DU shall wait at least for the indicated time before reinitiating the F1 setup towards the same gNB-CU.

#### 8.2.3.4	Abnormal Conditions

Not applicable.

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

The gNB-DU initiates the procedure by sending a GNB-DU CONFIGURATION UPDATE message to the gNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU responds with GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If gNB-DU ID IE is contained in the GNB-DU CONFIGURATION UPDATE message for a newly established SCTP association, the gNB-CU will associate this association with the related gNB-DU.

If Served Cells To Add Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall add cell information according to the information in the Served Cell Information IE. For NG-RAN, the gNB-DU shall include the gNB-DU System Information IE.

If Served Cells To Modify Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall modify information of cell indicated by Old NR CGI IE according to the information in the Served Cell Information IE and overwrite the served cell information for the affected served cell. Further, if the gNB-DU System Information IE is present the gNB-CU shall store and replace any previous information received.

If Served Cells To Delete Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall delete information of cell indicated by Old NR CGI IE.

If Cells Status Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall update the information about the cells, as described in TS 38.401 [4]. If if the Switching Off Ongoing IE is present in the Cells Status Item IE, contained in the GNB-DU CONFIGURATION UPDATE message, and the corresponding Service State IE is set to "Out-of-Service", the gNB-CU shall ignore the Switching Off Ongoing IE.

If Cells to be Activated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for cells for which the NR PCI IE is included.

If Cells to be Activated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If Cells to be Deactivated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall deactivate all the cells with NR CGI listed in the IE.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

For NG-RAN, the gNB-DU may include the RAN Area Code IE in the GNB-DU CONFIGURATION UPDATE message. The gNB-CU shall store and replace any previously provided RAN Area Code IE by the received RAN Area Code IE.

For NG-RAN, the gNB-DU may include the Supported MBS FSA ID List IE in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message. The gNB-CU shall store and replace any previously provided MBS FSA ID list IE by the received MBS FSA ID list IE.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

If Available SNPN ID List IE is contained in GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall overwrite the whole available SNPN ID list and update the corresponding system information.

If in GNB-DU CONFIGURATION UPDATE message, the Cell Direction IE is present, the gNB-CU should use it to understand whether the cell is for UL or DL only. If in GNB-DU CONFIGURATION UPDATE message, the Cell Direction IE is omitted in the Served Cell Information IE it shall be interpreted as that the Cell Direction is Bi-directional.

If the GNB-DU CONFIGURATION UPDATE message includes gNB-DU TNL Association To Remove List IE, the gNB-CU shall, if supported, initiate removal of the TNL association(s) indicated by gNB-DU TNL endpoint(s) and gNB-CU TNL endpoint(s) if the TNL Association Transport Layer Address gNB-CU IE is present, or the TNL association(s) indicated by gNB-DU TNL endpoint(s) if the TNL Association Transport Layer Address gNB-CU IE is absent:

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the gNB-DU TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-DU TNL endpoints correspond to all gNB-DU TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

- 	if the received TNL Association Transport Layer Address gNB-CU IE includes the Port Number IE, the gNB-CU TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-CU TNL endpoints correspond to all gNB-CU TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If the Intended TDD DL-UL Configuration IE is present in the GNB-DU CONFIGURATION UPDATE message, the receiving gNB-CU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-CU may merge the Intended TDD DL-UL Configuration information received from two or more gNB-DUs. The gNB-CU shall consider the received Intended TDD DL-UL Configuration IE content valid until reception of an update of the IE for the same cell(s).

If the Aggressor gNB Set ID IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account.

If the Victim gNB Set ID IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account.

If the GNB-DU CONFIGURATION UPDATE message includes Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message includes Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the SFN Offset IE is contained in the Served Cell Information IE in GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information to deduce the SFN0 offset of the reported cell.

If the NR PRACH Configuration List IE is included in the Served Cell Information IE contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information, and forward it to other RAN nodes for RACH optimisation. If the L139 Info IE included in the NR PRACH Configuration List IE is present, it shall contain the Root Sequence Index IE.

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message contains the BAP Address IE, the gNB-DU shall, if supported, store the received BAP address and use it as specified in TS 38.340 [30].

If the Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization and network energy saving. If the Coverage Modification Cause IE is set to the "network energy saving", gNB-CU may consider those deactivated SSB beams are due to network energy saving.

If the Cells for SON IE is present in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU may store or update this information and behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the RedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the eRedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If the gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU. If the Extended gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if also included.

If the RRC Terminating IAB-Donor Related Info IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider that the BAP address indicated by the Mobile IAB-MT BAP Address IE is assigned by the gNB-CU of the RRC-terminating IAB-donor indicated  by the RRC Terminating IAB-Donor gNB-ID IE, and it shall use this  BAP address and gNB ID for the subsequent IAB Transport Migration Management procedure towards the RRC-terminating IAB-donor of the mobile IAB-node as needed, as specified in TS 38.423 [28].

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the Cell DTX/DRX Information IE is included in Cell DTX/DRX Information List  IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take the information into account when configuring cell DTX/DRX patterns.

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: gNB-DU Configuration Update procedure: Unsuccessful Operation

If the gNB-CU cannot accept the update, it shall respond with a GNB-DU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-DU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-DU shall wait at least for the indicated time before reinitiating the GNB-DU CONFIGURATION UPDATE message towards the same gNB-CU.

#### 8.2.4.4	Abnormal Conditions

Not applicable.

***********************

Skip to the Next Change

***********************

#### 9.2.1.4	F1 SETUP REQUEST

This message is sent by the gNB-DU to transfer information associated to an F1-C interface instance.

NOTE:	If a TNL association is shared among several F1-C interface instances, several F1 Setup procedures are issued via the same TNL association after that TNL association has become operational.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-DU ID | M |  | 9.3.1.9 |  | YES | reject |
| gNB-DU Name | O |  | PrintableString(SIZE(1..150,...)) |  | YES | ignore |
| gNB-DU Served Cells List |  | 0.. 1 |  | List of cells configured in the gNB-DU | YES | reject |
| >gNB-DU Served Cells Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| gNB-DU RRC version | M |  | RRC version 9.3.1.70 |  | YES | reject |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-node. | YES | ignore |
| Extended gNB-DU Name | O |  | 9.3.1.205 |  | YES | ignore |
| RRC Terminating IAB-Donor gNB-ID | O |  | Global gNB ID 9.3.1.305 | The Global gNB ID of a mobile IAB-node’s RRC-terminating IAB donor. This IE is only present if the mobile IAB-node’s RRC terminating IAB-donor-CU is different from the gNB-CU receiving this message. | YES | reject |
| Mobile IAB-MT User Location Information | O |  | 9.3.1.307 |  | YES | ignore |
| Cell DTX/DRX Information List |  | 0 .. 1 |  | List of cells with cell DTX/DRX information configured. | YES | ignore |
| >Cell DTX/DRX Information Item |  | 0 .. < maxCellingNBDU > |  |  | Each | ignore |
| >>NR Cell Identity | M |  | BIT STRING (36) |  | – |  |
| >>Cell DTX/DRX Information | M |  | 9.3.1.x |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

***********************

Skip to the Next Change

***********************

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Served Cells To Add List |  | 0..1 |  | Complete list of added cells served by the gNB-DU | YES | reject |
| >Served Cells To Add Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| Served Cells To Modify List |  | 0..1 |  | Complete list of modified cells served by the gNB-DU | YES | reject |
| >Served Cells To Modify Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| Served Cells To Delete List |  | 0..1 |  | Complete list of deleted cells served by the gNB-DU | YES | reject |
| >Served Cells To Delete Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| Cells Status List |  | 0..1 |  | Complete list of active cells | YES | reject |
| >Cells Status Item |  | 0 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Service Status | M |  | 9.3.1.68 |  | - |  |
| Dedicated SI Delivery Needed UE List |  | 0..1 |  | List of UEs unable to receive system information from broadcast | YES | ignore |
| >Dedicated SI Delivery Needed UE Item |  | 1 .. <maxnoofUEIDs> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-DU ID | O |  | 9.3.1.9 |  | YES | reject |
| gNB-DU TNL Association To Remove List |  | 0..1 |  |  | YES | reject |
| >gNB-DU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | reject |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-DU. | - |  |
| >>TNL Association Transport Layer Address gNB-CU | O |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU | - |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Coverage Modification Notification | O |  | 9.3.1.213 |  | YES | ignore |
| gNB-DU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-DU. | YES | ignore |
| Extended gNB-DU Name | O |  | 9.3.1.205 |  | YES | ignore |
| RRC Terminating IAB-Donor Related Info | O |  | 9.3.1.306 | Indicates the information related to a mobile IAB-node’s RRC-terminating IAB-donor. | YES | reject |
| Mobile IAB-MT User Location Information | O |  | 9.3.1.307 |  | YES | ignore |
| Cell DTX/DRX Information List |  | 0 .. 1 |  | List of cells with cell DTX/DRX information configured modified. | YES | ignore |
| >Cell DTX/DRX Information Item |  | 0 .. < maxCellingNBDU > |  |  | Each | ignore |
| >>NR Cell Identity | M |  | BIT STRING (36) |  | – |  |
| >>Cell DTX/DRX Information | M |  | 9.3.1.x |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |

***********************

Skip to the Next Change

***********************

#### 9.3.1.x	Cell DTXDRX Information

This IE indicates cell DTX/DRX information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE On-Duration Timer | M |  |  | See cellDTXDRX-onDurationTimer defined in TS 38.331 [10]. |
| >sub MilliSeconds |  |  |  |  |
| >>Sub MilliSeconds Timer | M |  | INTEGER (1..31) |  |
| >milliSeconds |  |  |  |  |
| >>MilliSeconds Timer | M |  | ENUMERATED (ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200, ms1600, ... ) |  |
| CHOICE Cycle Start Offset | M |  |  | See cellDTXDRX-CycleStartOffset defined in TS 38.331 [10]. |
| >ms10 |  |  |  |  |
| >>Offset ms10 | M |  | INTEGER (0..9) |  |
| >ms20 |  |  |  |  |
| >>Offset ms20 | M |  | INTEGER (0..19) |  |
| >ms32 |  |  |  |  |
| >>Offset ms32 | M |  | INTEGER (0..31) |  |
| >ms40 |  |  |  |  |
| >>Offset ms40 | M |  | INTEGER (0..39) |  |
| >ms60 |  |  |  |  |
| >>Offset ms60 | M |  | INTEGER (0..59) |  |
| >ms64 |  |  |  |  |
| >>Offset ms64 | M |  | INTEGER (0..63) |  |
| >ms70 |  |  |  |  |
| >>Offset ms70 | M |  | INTEGER (0..69) |  |
| >ms80 |  |  |  |  |
| >>Offset ms80 | M |  | INTEGER (0..79) |  |
| >ms128 |  |  |  |  |
| >>Offset ms128 | M |  | INTEGER (0..127) |  |
| >ms160 |  |  |  |  |
| >>Offset ms160 | M |  | INTEGER (0..159) |  |
| >ms256 |  |  |  |  |
| >>Offset ms256 | M |  | INTEGER (0..255) |  |
| >ms320 |  |  |  |  |
| >>Offset ms320 | M |  | INTEGER (0..319) |  |
| >ms512 |  |  |  |  |
| >>Offset ms512 | M |  | INTEGER (0..511) |  |
| >ms640 |  |  |  |  |
| >>Offset ms640 | M |  | INTEGER (0..639) |  |
| >ms1024 |  |  |  |  |
| >>Offset ms1024 | M |  | INTEGER (0..1023) |  |
| >ms1280 |  |  |  |  |
| >>Offset ms1280 | M |  | INTEGER (0..1279) |  |
| >ms2048 |  |  |  |  |
| >>Offset ms2048 | M |  | INTEGER (0..2047) |  |
| >ms5120 |  |  |  |  |
| >>Offset ms5120 | M |  | INTEGER (0..5119) |  |
| >ms10240 |  |  |  |  |
| >>Offset ms10240 | M |  | INTEGER (0..10239) |  |
| Slot Offset | M |  | INTEGER (0..31) | See cellDTXDRX-SlotOffset defined in TS 38.331 [10]. |
| Config Type | M |  | ENUMERATED (dtx, drx, dtxdrx, ...) | See cellDTXDRXconfigType defined in TS 38.331 [10]. |
| Active Duration Usage Probability | M |  | INTEGER (0..100) | Probability in percent that the resource is used during the active duration (see TS 38.300 [9] clause 15.4.2.3). Value 0 corresponds to 0% probability, and value 100 corresponds to 100% probability. |
| Non-Active Duration Usage Probability | M |  | INTEGER (0..100) | Probability in percent that the resource is used during the non-active duration (see TS 38.300 [9] clause 15.4.2.3). Value 0 corresponds to 0% probability, and value 100 corresponds to 100% probability. |

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

BroadcastMRBs-FailedToBeModified-Item,

BroadcastMRBs-FailedToBeSetup-Item,

BroadcastMRBs-FailedToBeSetupMod-Item,

BroadcastMRBs-Modified-Item,

BroadcastMRBs-Setup-Item,

BroadcastMRBs-SetupMod-Item,

BroadcastMRBs-ToBeModified-Item,

BroadcastMRBs-ToBeReleased-Item,

BroadcastMRBs-ToBeSetup-Item,

BroadcastMRBs-ToBeSetupMod-Item,

Candidate-SpCell-Item,

Cause,

Cells-Allowed-to-be-Deactivated-List-Item,

CellDTXDRXInfoList,

Cells-Failed-to-be-Activated-List-Item,

Cells-Status-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

CellULConfigured,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRB-Activity-Item,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRB-Notify-Item,

***********************

Skip to the Next Change

***********************

id-BroadcastMRBs-ToBeSetup-Item,

id-BroadcastMRBs-ToBeSetupMod-List,

id-BroadcastMRBs-ToBeSetupMod-Item,

id-Candidate-SpCell-Item,

id-Candidate-SpCell-List,

id-Cause,

id-Cancel-all-Warning-Messages-Indicator,

id-CellDTXDRXInfoList,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-Status-Item,

id-Cells-Status-List,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-Cells-Allowed-to-be-Deactivated-List,

id-Cells-Allowed-to-be-Deactivated-List-Item,

id-Cells-With-SSBs-Activated-List,

id-Recommended-SSBs-for-Paging-List,

id-ConfirmedUEID,

id-CriticalityDiagnostics,

id-C-RNTI,

***********************

Skip to the Next Change

***********************

-- **************************************************************

--

#### -- F1 SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- F1 Setup Request

--

-- **************************************************************

F1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupRequestIEs} },

...

}

F1SetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-DU-ID						CRITICALITY reject	TYPE GNB-DU-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-Name						CRITICALITY ignore	TYPE GNB-DU-Name						PRESENCE optional	}|

{ ID id-gNB-DU-Served-Cells-List		CRITICALITY reject	TYPE GNB-DU-Served-Cells-List			PRESENCE optional	}|

{ ID id-GNB-DU-RRC-Version				CRITICALITY reject	TYPE RRC-Version						PRESENCE mandatory	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info		PRESENCE optional	}|

{ ID id-BAPAddress						CRITICALITY ignore	TYPE BAPAddress							PRESENCE optional	}|

{ ID id-Extended-GNB-DU-Name			CRITICALITY ignore	TYPE Extended-GNB-DU-Name				PRESENCE optional	}|

{ ID id-RRC-Terminating-IAB-Donor-gNB-ID	CRITICALITY reject	TYPE GlobalGNB-ID					PRESENCE optional }|

{ ID id-Mobile-IAB-MTUserLocationInformation	CRITICALITY ignore	TYPE Mobile-IAB-MTUserLocationInformation 		PRESENCE optional	}|

{ ID id-CellDTXDRXInfoList						CRITICALITY ignore	TYPE CellDTXDRXInfoList									PRESENCE optional },

...

}

GNB-DU-Served-Cells-List 	::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { GNB-DU-Served-Cells-ItemIEs } }

GNB-DU-Served-Cells-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-GNB-DU-Served-Cells-Item		CRITICALITY reject	TYPE		GNB-DU-Served-Cells-Item	PRESENCE mandatory	},

...

}

***********************

Skip to the Next Change

***********************

-- **************************************************************

--

#### -- GNB-DU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-DU CONFIGURATION UPDATE

--

-- **************************************************************

GNBDUConfigurationUpdate::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateIEs} },

...

}

GNBDUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID												PRESENCE mandatory	}|

{ ID id-Served-Cells-To-Add-List				CRITICALITY reject	TYPE Served-Cells-To-Add-List								PRESENCE optional	}|

{ ID id-Served-Cells-To-Modify-List				CRITICALITY reject	TYPE Served-Cells-To-Modify-List							PRESENCE optional	}|

{ ID id-Served-Cells-To-Delete-List				CRITICALITY reject	TYPE Served-Cells-To-Delete-List							PRESENCE optional	}|

{ ID id-Cells-Status-List						CRITICALITY reject	TYPE Cells-Status-List											PRESENCE optional	}|

{ ID id-Dedicated-SIDelivery-NeededUE-List		CRITICALITY ignore	TYPE Dedicated-SIDelivery-NeededUE-List					PRESENCE optional	}|

{ ID id-gNB-DU-ID								CRITICALITY reject	TYPE GNB-DU-ID													PRESENCE optional	}|

{ ID id-GNB-DU-TNL-Association-To-Remove-List	CRITICALITY reject	TYPE GNB-DU-TNL-Association-To-Remove-List				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE Transport-Layer-Address-Info							PRESENCE optional	}|

{ ID id-Coverage-Modification-Notification		CRITICALITY ignore	TYPE Coverage-Modification-Notification					PRESENCE optional	}|

{ ID id-gNB-DU-Name								CRITICALITY ignore	TYPE GNB-DU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-DU-Name					CRITICALITY ignore	TYPE Extended-GNB-DU-Name										PRESENCE optional	}|

{ ID id-RRC-Terminating-IAB-Donor-Related-Info	CRITICALITY reject	TYPE RRC-Terminating-IAB-Donor-Related-Info	         PRESENCE optional }|

{ ID id-Mobile-IAB-MTUserLocationInformation    CRITICALITY ignore	TYPE Mobile-IAB-MTUserLocationInformation 		      PRESENCE optional	}|

{ ID id-CellDTXDRXInfoList						CRITICALITY ignore	TYPE CellDTXDRXInfoList											PRESENCE optional },

...

}

Served-Cells-To-Add-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Add-ItemIEs } }

Served-Cells-To-Modify-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Modify-ItemIEs } }

Served-Cells-To-Delete-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Delete-ItemIEs } }

Cells-Status-List	::= SEQUENCE (SIZE(0.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Status-ItemIEs } }

Dedicated-SIDelivery-NeededUE-List::= SEQUENCE (SIZE(1.. maxnoofUEIDs))	OF ProtocolIE-SingleContainer { { Dedicated-SIDelivery-NeededUE-ItemIEs } }

GNB-DU-TNL-Association-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-DU-TNL-Association-To-Remove-ItemIEs } }

Served-Cells-To-Add-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Add-Item		CRITICALITY reject	TYPE	Served-Cells-To-Add-Item				PRESENCE mandatory	},

...

}

Served-Cells-To-Modify-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Modify-Item			CRITICALITY reject	TYPE		Served-Cells-To-Modify-Item							PRESENCE mandatory	},

...

}

Served-Cells-To-Delete-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Delete-Item				CRITICALITY reject	TYPE		Served-Cells-To-Delete-Item					PRESENCE mandatory	},

...

}

Cells-Status-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Status-Item				CRITICALITY reject	TYPE		Cells-Status-Item					PRESENCE mandatory	},

...

}

Dedicated-SIDelivery-NeededUE-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Dedicated-SIDelivery-NeededUE-Item		CRITICALITY ignore	TYPE	Dedicated-SIDelivery-NeededUE-Item				PRESENCE mandatory	},

...

}

GNB-DU-TNL-Association-To-Remove-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-DU-TNL-Association-To-Remove-Item		CRITICALITY reject	TYPE	 GNB-DU-TNL-Association-To-Remove-Item			PRESENCE mandatory	},

...

}

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Coverage-Modification-Cause,

id-Cell-Direction,

***********************

Skip to the Next Change

***********************

#### -- C

CAGID ::= BIT STRING (SIZE(32))

Cancel-all-Warning-Messages-Indicator ::= ENUMERATED {true, ...}

Candidate-SpCell-Item ::= SEQUENCE {

candidate-SpCell-ID			NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { Candidate-SpCell-ItemExtIEs } }	OPTIONAL,

...

}

Candidate-SpCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CapacityValue::= SEQUENCE {

capacityValue				INTEGER (0..100),

sSBAreaCapacityValueList	SSBAreaCapacityValueList		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { CapacityValue-ExtIEs} } OPTIONAL

}

CapacityValue-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

***********************

Skip to the Next Change

***********************

CellGroupConfig ::= OCTET STRING

CellCapacityClassValue ::= INTEGER (1..100,...)

Cell-Direction ::= ENUMERATED {dl-only, ul-only}

CellMeasurementResultList ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF CellMeasurementResultItem

CellMeasurementResultItem ::= SEQUENCE {

cellID							NRCGI,

radioResourceStatus				RadioResourceStatus 			OPTIONAL,

compositeAvailableCapacityGroup	CompositeAvailableCapacityGroup	OPTIONAL,

sliceAvailableCapacity			SliceAvailableCapacity 			OPTIONAL,

numberofActiveUEs 				NumberofActiveUEs				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CellMeasurementResultItem-ExtIEs} } OPTIONAL

}

CellMeasurementResultItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NR-U-Channel-List	CRITICALITY ignore	EXTENSION NR-U-Channel-List PRESENCE optional },

...

}

CellDTXDRXCycleStartOffset ::= CHOICE {

ms10					INTEGER(0..9),

ms20					INTEGER(0..19),

ms32					INTEGER(0..31),

ms40					INTEGER(0..39),

ms60					INTEGER(0..59),

ms64					INTEGER(0..63),

ms70					INTEGER(0..69),

ms80					INTEGER(0..79),

ms128					INTEGER(0..127),

ms160					INTEGER(0..159),

ms256					INTEGER(0..255),

ms320					INTEGER(0..319),

ms512					INTEGER(0..511),

ms640					INTEGER(0..639),

ms1024					INTEGER(0..1023),

ms1280					INTEGER(0..1279),

ms2048					INTEGER(0..2047),

ms2560					INTEGER(0..2559),

ms5120					INTEGER(0..5119),

ms10240					INTEGER(0..10239),

choice-extension		ProtocolIE-Single-Container { {CellDTXDRXCycleStartOffset-ExtIEs} }

}

CellDTXDRXCycleStartOffset-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellDTXDRXInfo ::= SEQUENCE {

onDurationTimer							CellDTXDRXOnDurationTimer,

cycleStartOffset						CellDTXDRXCycleStartOffset,

slotOffset                				INTEGER (0..31),

configType								ENUMERATED {dtx, drx, dtxdrx},

activeDurationUsageProbability			INTEGER (1..100),

nonActiveDurationUsageProbability		INTEGER (1..100),

iE-Extensions							ProtocolExtensionContainer { { CellDTXDRXInfo-ExtIEs} } OPTIONAL,

...

}

CellDTXDRXInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellDTXDRXInfoItem ::= SEQUENCE {

globalNG-RANCell-ID				GlobalCell-ID,

cellDTXDRXInfo					CellDTXDRXInfo,

iE-Extension					ProtocolExtensionContainer { { CellDTXDRXInfoItem-ExtIEs} } OPTIONAL,

...

}

CellDTXDRXInfoItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellDTXDRXInfoList ::= SEQUENCE (SIZE (0..maxnoofCellsinNG-RANnode)) OF CellDTXDRXInfoItem

CellDTXDRXOnDurationTimer ::= CHOICE {

subMilliSeconds 		INTEGER (1..31),

milliSeconds    		ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600,

...},

choice-extension		ProtocolIE-Single-Container { {CellDTXDRXOnDurationTimer-ExtIEs} }

}

CellDTXDRXOnDurationTimer-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

***********************

Skip to the Next Change

***********************

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

***********************

Skip to the Next Change

***********************

id-EarlySyncServingCellInformation					ProtocolIE-ID ::= 843

id-RANSharingAssistanceInformation					ProtocolIE-ID ::= 844

id-LTMCFRAResourceConfig-List						ProtocolIE-ID ::= 845

id-F1U-PathFailure									ProtocolIE-ID ::= 846

id-MeasBasedOnAggregatedResources					ProtocolIE-ID ::= 847

id-SIB23-message									ProtocolIE-ID ::= 848

id-BarringExemptionforEmerCallInfo					ProtocolIE-ID ::= 849

id-SIB17bis-message									ProtocolIE-ID ::= 850

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-CellDTXDRXInfoList								ProtocolIE-ID ::= xxx

END

-- ASN1STOP