3GPP TSG-RAN WG3 Meeting #126	R3-247762

Orlando, Florida, USA, November 18th – 22nd, 2024

Agenda item:	13.2

Source: 	LG Electronics Inc, ZTE, Fujitsu, KDDI, KT Corp., ITRI, Rakuten, ETRI, SK Telecom, LG Uplus,

Title: 	Response to R3-247111 and R3-247112

Document for:	Discussion and Decision

# Introduction

RAN2 liaised RAN3 [1] (R2-2409378/R3-247111) to inform the RAN2 agreements about Rel-19 Inter-CU LTM, one of which is re-using the reference configuration handling from Rel-18 LTM:

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

For this, R3-247112 [2] provided the well-summarized analysis of the potential challenges and impacts with the single reference configuration from RAN3 point of view, requesting RAN2 to revisit the agreement:

Proposal 1: RAN3 to discuss the need of multiple reference configuration from RAN3 signalling point of view. In our view, supporting on single reference configuration in case of inter-CU LTM brings more complexity in RAN3 signaling than benefits. We list below the challenges that RAN3 will face in designing an inter-CU handover with single reference configuration. Challenge #1: First, it needs to be decided in RAN3 who generates the reference configuration. Is it the source CU or one of the candidate target CU? Challenge # 2: If one of the candidate target CUs generates the reference configuration, it incurs additional signaling to first fetch the reference configuration at source CU and then use it to get delta configurations of other candidate cells. This would cause additional signaling overhead over F1 and Xn. Challenge # 3: If source CU generates the reference configuration based on its own cells, it may not be the optimum choice as source CU may only contain one cell and all other candidate cells may belong to different CU. This would result in sub-optimum use of reference configuration with no clear benefit of reducing signaling overhead. Challenge # 4: Commonality between cell configurations belonging to different CUs may be limited, especially in case of multi-vendor CUs and different frequency ranges. Hence, this makes the reference configuration feature not useful and also not feasible for actual deployments. Challenge # 5: If RAN3 decides to go in the path that one CU can generate reference configuration based on its own cell configurations but other cells belonging to different CU use complete configuration, this results in additional complexity in RRC signaling and structure from both RAN2 and RAN3 point of view. Challenge #6: Mechanism for updating reference each time a a new candidate cell or candidate CU is prepared. Observation 1: It is observed that the use of single reference configuration brings more challenges in RAN3 than actual advantages. Proposal 2: RAN3 to discuss the above-mentioned challenges with respect to the use of single reference configurations. We believe that supporting multiple reference configurations for inter-CU LTM is a straightforward solution for the above challenges and with clear advantages. In addition, it is a simple extension with reduced effort from RAN3 point of view. The justification of this is as follows: Justification # 1: Cell configurations have the maximum similarities when they belong to same CU (and/or frequency ranges). Hence, generating one reference configuration per CU makes more sense. Justification # 2: Reference configuration per CU will results in reduced signaling from RAN3 point of view. Meaning, every CU can generate reference configuration and provide it as part of HANDOVER REQUEST ACKNOWLEDGEMENT message. Justification # 3: It gives the network flexibility to configure the number of reference configurations which provides the maximum benefits (subject to UE capability). Justification # 4: It helps the modularity in RRC structure design where LTM Config is generated per CU (subject to further RAN2 discussions). Justification #5: Addition and removal of candidate gNBs does not require an update procedure related to the reference configuration. Based on the above, we propose the following. Proposal 3: RAN3 to agree that multiple reference configurations provide more benefits compared to single reference configuration from RAN3 signaling and standardization effort point of view. Proposal 4: Send an LS to RAN2 requesting to revisit the agreement on reference configuration considering RAN3 view.

In this response paper, we review the background and design principle of “reference configuration” concept, based on which, we provide additional analysis and reasons why the Rel-18 Intra-CU LTM decision of having the single reference configuration should not be simply replayed onto Rel-19 Inter-CU LTM.

The draft LS reply to RAN2 for requesting to revisit the agreement is also provided in Annex.

# Discussion

What is the reference configuration? and Why should we reconsider?

The recent concept of “reference configuration” in RRC signalling was introduced to enable delta RRC signalling for candidate cell configurations seamlessly during consecutive cell switches (a.k.a. subsequent mobility). In NR, mobility mechanisms have evolved (in one track) to enhance robustness, by allowing NW to prepare and preconfigure multiple candidate cells in advance among which only one is selected for cell switch. Until CPAC, mobility scenarios were limited to only one-time cell switch where all the prepared configurations are released in the UE and NW after the successful execution. For this one-time switch, the legacy delta / full RRC signalling model was still feasible to rely on the current configuration at the source cell as a basis for generating delta RRC signalling.

However, with the introduction of subsequent mobility scenarios (S-CPAC, LTM), where UEs can move through a series of consecutive cell switches based on pre-configurations from NW upfront, the limitation of this legacy delta / full model became outstanding. The existing model could not be applied for consecutive cell switches and changes of cell configurations over time, because, without knowing the exact sequence of next cell switches in advance, NW was impossible to prepare deltas for candidate cells upfront where the basis for delta (i.e. current configuration of a UE) will keep changing over the course of cell switches. This challenge was addressed by introducing a stable “reference” configuration that remains unchanged as UEs move from cell to cell, allowing NW to prepare and preconfigure delta signalling across multiple candidate cells, and save RRC signalling overhead.

In sum, the reference configuration concept serves as a foundational mechanism to overcome the RRC signaling challenges in subsequent mobility scenarios. It enables NW to prepare delta configurations upfront, that was not feasible in the legacy delta / full RRC signalling model. Without this “reference” concept, RRC signalling optimization for multiple candidate cell configurations was never possible in subsequent mobility scenarios. It also offers flexibility as it relaxed the limitation of the legacy model that had to be based only on the “current configuration” as a basis for delta at any point of time. That is, it freed the NW from the legacy requirement to use only the current configuration as the “reference”. The concept of reference configuration inherently imposes the flexibility for RRC signalling optimization.

From these senses, simply following the Rel-18 approach of having only one reference without full understanding of its design concept could go against the RRC signalling efficiency and flexibility that this elegant concept provides. The flexibility inherent in the reference configuration design should guide toward the RRC signaling optimization, not restrict it to a single reference.

Moreover, Rel-19 is different to Rel-18, which involves multiple CUs connected via Xn, and thus more careful considerations are required than Rel-18. As pointed out in [2], the similarity between cell configurations belonging to different CUs (especially in case of multi-vendor CUs) is not expected to be large. This aspect can also be observed from the fact that every handover in legacy was not solely based on delta RRC signalling. Full configuration (originally devised for resetting RRC configuration for re-synch between UE and NW) was extended to mobility when the target doesn’t comprehend the RRC configuration at the source side. If the target side is disaggregated between CU and DU, the situation is more complex: even if the target CU comprehended the CU part of the configuration, still handover has to be based on full configuration if the target DU doesn’t comprehend the source’s CellGroupConfig. Supporting delta RRC signalling during mobility across eNBs/gNBs has never been easy in real deployments.

If only a single reference is chosen based from either the source CU or one of candidate CU(s), it is not hard to imagine that other candidate cells belonging to different CU may end up requiring complete configurations, losing the inherent benefits of reference configuration for saving RRC signalling overhead.

Figure 1: Example comparison of single reference configuration vs multiple reference configurations

Therefore, we believe that, at least in Rel-19, specifications should allow each involved CU to independently decide and establish the most suitable reference configuration for its own candidate cells from the beginning.

Why RAN3? and Why now?

Another motivation for this contribution is due to the discussions resulting in the agreement. Two solutions were on the table at RAN2-127 (Maastricht) and the agreement toward the single reference configuration was made based on show of hands:

Single or multiple reference configuration(s)? P1 in R2-2406694 (Apple) Proposal 1: Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations are supported. - P1 in R2-2406694 is agreed. P1 in R2-2407269 (LG) Proposal 1. Multiple reference configurations are provided for inter-CU LTM, where each reference configuration is CU specific. [Samsung]: Multiple reference configuration gives more flexibility. NW still can achieve single reference configuration, which is up to NW. [Huawei]: Assume it does not happen often and saved bits are not significant. It is considered as optimization for some cases, which is not essential. [ZTE]: With multiple reference configurations, it simplifies inter-CU signalling. [Xiaomi]: With multiple reference configurations, it increases UE complexity and it requires more UE memory capability. [Ericsson]: Single reference configuration works as baseline. [Nokia]: With single reference configuration, the reference configuration setup procedure needs to be repeated whenever any CU reconfigures some configuration. [Session chair]: Check companies’ positions. If there is no clear majority companies, we can stick to what we already have (single reference configuration). - Multiple reference configurations: LG, Samsung, CMCC, Nokia, Fujitsu, Rakuten, ITRI, KDDI, ZTE, BT, IDC, KT, ETRI (13) - Single reference configuration: Google, Honor, CATT, Lenovo, Vivo, Apple, OPPO, Spredtrum, Xiaomi, NEC, Intel, MediaTek, Qualcomm, China Telecom, Ericsson, Sharp (16)

RAN3 is not in a position and does not intend to dictate how RAN2 should conduct discussions. Each WG has its own traditions and practices of making decisions and progresses, for which other WGs shall respect.

But our concerns lie in the fact that both solution candidates for down-selection are essentially impacting XnAP designs within RAN3’s domain. Given that most of RAN3 experts are unable to physically attend RAN2 sessions for providing technical clarities and insights for judging solution candidates, if there is no clear consensus, or solutions on the table require in-depth evaluation, it is generally expected to send an LS to the responsible group(s) to get feedbacks and additional technical clarities. This formal procedure helps involving cross-WG expertise to best decide the right solution and maintain high standard of specifications. However, we observe that no such LS was sent for RAN3’s consultation before reaching the agreement.

For Rel-18 reference configuration, it is true that RAN2’s decision for Intra-CU LTM was made without formal consultation from RAN3. But that was less of a concern at that time given that only a single CU is involved, fully controlling and managing one or more underlying DUs for LTM candidate cell configurations. We also observe that Rel-18 S-CPAC (involving multiple SN entities) was decided in RAN2 to follow the “single” reference configuration mirrored from Intra-CU LTM, mainly for RRC signaling harmonization and sake of progress:

R2-2302934 Report of [Post121][044][eMob] SCG Selective Activation in NR-DC Signalling interaction Qualcomm Incorporated discussion Rel-18 DISCUSSION P1 - Nokia think that the procedure can be the same but the actual configuration will be different. VDF think we can look at the differences later. Chair think the P1 includes what Nokia thinks. For the reference configuration for SCG Selective Activation, aim at following similar design as LTM. For inter-SN SCG Selective Activation, the RRC reconfiguration message containing the Rel-18 CPC configurations provided to the UE is in MN format. For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC. FFS on the following options for subsequent CPC: Option 1: Source MN generates the execution conditions for all subsequent CPC. Option 2: Candidate SN may generate execution conditions for subsequent CPC. For SN initiated inter-SN SCG selective activation, source SN generates the execution conditions for the initial CPC. FFS if Candidate SN may generate/modify execution conditions for subsequent CPC Assume for now that there is only one reference configuration.

In fact, there were many enhancements under-went for Rel-18 mobility features, and companies were busy completing them in time. RAN3 had been also busy finishing up stage-2/3 for those mobility features to meet the Rel-18 schedule. These situations are understandable, but we think that the decision for S-CPAC was not made with careful and full considerations of its broader impacts.

Therefore, for Rel-19 (and beyond), we believe that we should not merely follow the past design decision on the reference configuration handling. From its design principle and nature that enables the RRC signalling efficiency and flexibility for subsequent mobility scenarios, and also considering involvements across multiple CUs, we believe the right direction in Rel-19 is in allowing each involved CU to independently decide the most suitable reference configuration for its own candidate cells.

Proposal 1: For Rel-19 Inter-CU LTM, allow multiple reference configurations, so that each involved CU can independently decide the most suitable reference configuration for its own candidate cells.

Proposal 2: Send an LS to RAN2 to revisit the agreement on reference configuration considering the RAN3’s perspective.

The draft LS reply is provided in Annex.

# Conclusion

In the present contribution we make the following observations:

What is the reference configuration? and Why should we reconsider?

Observation 1: “Reference configuration” was introduced to enable delta RRC signalling for subsequent mobility scenarios (S-CPAC, LTM), as the traditional delta/full RRC signalling model could not be applied when NW preconfigures multiple candidate cell configurations upfront.

Observation 2: The legacy delta/full model relies on a single, “current configuration” as the basis for deltas, but in consecutive mobility scenarios, NW is impossible to preconfigure deltas as the current configuration of a UE will keep changing over the course of cell switches and unpredictable.

Observation 3: By providing stable baseline that remains unchanged as UEs move from cell to cell, “Reference configuration” enabled preconfigured delta signaling for multiple candidate cells, and thus enabled RRC signaling overhead reduction for subsequent mobility scenarios.

Observation 4: Moreover, the reference concept inherently offers flexibility, allowing NW to optimize RRC signaling without being restricted to using only “current configuration” as for delta basis.

Observation 5: Rel-19, with multiple CUs connected via Xn and potentially involving multi-vendor CUs, requires more careful considerations, as the similarity between cell configurations belonging to different CUs (especially in case of multi-vendor CUs) is not expected to be large.

Observation 6: Supporting delta RRC signalling during mobility across eNBs/gNBs has never been easy – that’s where every handover in legacy was not solely based on delta RRC signalling, and “full configuration” was allowed when a target doesn’t comprehend RRC configuration at the source side.

Observation 7: In a disaggregated CU-DU environment, even if the target CU could interpret source configurations, full configuration was still necessary if the target DU could not comprehend the source’s CellGroupConfig, which complicated delta RRC signaling.

Observation 8: Limiting Rel-19’s design to a single reference configuration, as per Rel-18's approach, would go against the flexibility and signalling efficiency that this concept was designed to enable. For optimal RRC signaling efficiency in Rel-19, each CU should be allowed to independently decide and define the most suitable reference configuration for its own candidate cells.

Why RAN3? and Why now?

Observation 9: The decision for a single reference configuration for Rel-19 Inter-CU LTM in RAN2 was made by a show of hands, with almost equal support for each candidate solution.

Observation 10: RAN3 respects the decision-making practices of RAN2, but the candidate solutions on the table both affect XnAP designs under RAN3’s domain. It is generally expected to send an LS to get feedbacks and technical clarities from relevant WGs for in-depth evaluation, but there was no formal consultation from RAN3 before RAN2 reaching the agreement.

Observation 11: For Rel-18 Intra-CU LTM, RAN2’s decision on the single reference configuration was made without formal consultation from RAN3. But that was less of a concern at that time since only a single CU is involved, controlling/managing underlying DUs for LTM candidate cell configurations.

Observation 12: The RAN2’s decision for Rel-18 S-CPAC, involving multiple SN entities, was made without careful and full considerations of its broader impacts, as it mirrored the Intra-CU LTM decision of single reference configuration for RRC signalling harmonization and sake of progress.

Observation 13: For Rel-19 (and beyond), we should not merely follow the past decision on reference configuration, given the complexity of Rel-19 (involving multiple CUs over Xn) and considering its design concept and nature that provides flexibility and enables RRC signalling overhead reduction for subsequent mobility scenarios.

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: For Rel-19 Inter-CU LTM, allow multiple reference configurations, so that each involved CU can independently decide the most suitable reference configuration for its own candidate cells.

Proposal 2: Send an LS to RAN2 to revisit the agreement on reference configuration considering the RAN3’s perspective.

The draft LS reply is provided in Annex.

# Reference

[1] R3-247111/R2-2409378, LS on RAN2 agreements for inter-CU LTM, RAN2

[2] R3-247112, “Support of Multiple Reference Configurations”, Nokia. KDDI, LG Electronics, ZTE, Rakuten, Fujitsu

# Annex: [DRAFT] LS to RAN2

Title:	[Draft] RAN3 feedbacks on Rel-19 Inter-CU LTM reference configuration

Response to:	R3-247111/R2-2409378 LS on RAN2 agreements for inter-CU LTM

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	LG Electronics Inc. [To be RAN3]

To:	RAN2

Cc:

Contact Person:

Name: 	Jaemin Han

E-mail Address: 	jaeminh.han@lge.com

Send any reply LS to:           3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks RAN2 to inform the RAN2 agreements so far achieved for Rel-19 Inter-CU LTM. Regarding the following agreement on single reference configuration, RAN3 would like to provide the following feedbacks:

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

- Considering the design concept and nature of reference configuration which enables delta RRC signalling for subsequent mobility scenarios, RAN3 sees that limiting Rel-19 to a single reference, as per Rel-18, restricts the flexibility inherent in this concept and goes against its purpose for RRC signalling efficiency. RAN3 sees it is critical that each CU should be able to independently decide the most suitable reference configuration for its own candidate cells.

- Unlike Rel-18 Intra-CU LTM, Rel-19 aims LTM with multiple CUs connected via Xn, potentially involving multi-vendor CUs with limited configuration similarity. RAN3 sees that more careful consideration is necessary, as the single reference chosen based from one CU may end up complete configurations of candidate cells belonging to different CU(s) and losing the inherent benefits of reference configuration for saving RRC signalling overhead in Rel-19 Inter-CU LTM.

- RAN3 observes that supporting delta RRC signalling during mobility across eNBs/gNBs has never been easy – that’s where every handover in legacy was not solely based on delta RRC signalling. “Full configuration” was allowed when a target doesn’t comprehend RRC configuration at the source side. Moreover, in a disaggregated CU-DU environment, even if the target CU could interpret source configurations, full configuration was still necessary if the target DU could not comprehend the source’s CellGroupConfig, which complicated delta RRC signaling.

- RAN3 observes that, for Rel-18 Intra-CU LTM, RAN2’s decision on the single reference configuration was made without formal consultation from RAN3. But that was less of a concern from RAN3 at that time as only a single CU is involved, fully controlling underlying DUs for LTM candidate cell configurations. It was also observed that RAN2’s decision for S-CPAC, involving multiple SN entities, was made without careful considerations of its broader impacts, as it mirrored the Intra-CU LTM decision of single reference configuration for RRC signalling harmonization and sake of progress.

- For Rel-19 (and beyond), RAN3 believes that we should not merely follow the past decision on reference configuration, given the complexity of Rel-19 (involving multiple CUs over Xn) and considering its design concept and nature that provides flexibility and enables RRC signalling overhead reduction on candidate cells configuarations in subsequent mobility scenarios.

RAN3 respectively requests RAN2 to take the above information and feedbacks from RAN3 into account and to discuss and revisit the agreement on reference configuration for Rel-19 Inter-CU LTM.

2. Actions:

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to take the above information and revisit the agreement on reference configuration in Rel-19 Inter-CU LTM.

3. Date of Next TSG-RAN WG3 Meetings:

TSG RAN3 Meeting #127 	February 17th – 21st, 2025 			Athens, Greece

TSG RAN3 Meeting #127bis	April 7th – 11th, 2025 					China (TBC)