3GPP TSG-RAN WG3 Meeting #1286		R3-247774

Orlando, USA, 18th-22nd November 2024

Title: Offline discussion on the support of L3 based measurement  LTM handover

Moderator: Alexey Kulakov; Vodafone

Questions to discuss:

Proposal 1: To confirm that RAN WG3 intends to support L3 based measurement LTM handover in rel 18

- Qualcomm: Agree with the proposal, but wants to agree on details first.

- Ericsson: Agree we need to discuss details first.

- CATT: Reasonable to agree on the proposal as RAN2 and RAN4 supports

- Samsung: Agree with the CATT.

- NEC: Need to agree on details first

- LG:  Agree with the proposal

- No conclusion

Proposal 2: To decide if the decision to execute L3 measurement based handover is done by CU:

- Nokia: LS from RAN2 is clear, that this is an intended behavior.

- Qualcomm If you agree CU decides, that LTM triggers the switch how do you know the beam info? ZTE: beam level information can also be provided by L3 measurements and supports the proposal. Also asked about early synch support, which is intended to be supported.

- Qualcomm: Beam level information is optional and normally supported by UEs if they support L1 measurements.

- ZTE: It is controlled by the NW

- Qualcomm: Qualcomm states that RAN4 requirements are agreed with the assumption that there is only one algorithm to take a decision and that’s why Qualcomm propose an LS to RAN4. Qualcomm is worried that RAN4 requirements can not be met once the decision is done by CU

- Ericsson: Disagree with Qualcomm and think RAN4 requirements are NW nodes agnostic and so no need for LS.

- Huawei: Similar understanding with E///. Clear requirements are in RAN4 spec and they should be applicable to any scenario (DU or CU makes a decision). Qualcomm confirms that their concern is that only DU knows when to send PDCCH order and/or MAC CE and so requirements from RAN4 might not be met.

- ZTE agrees with E/// and Huawei.

- LG: agrees with ZTE.

- Google: Who makes the config, this node should also make a decision. Agrees with CU approach.

- Jio: supports Qualcomm

- Ericsson: there is no signaling overhead. Forwarding the measurements between the nodes might interfere with event based LTM in R19.

- Qualcomm think that even in FR1 there are multiple beams.

Working assumption:

- Qualcomm suggest to standardize both

- NEC has strong concerns to standardize both

- Ericsson: Need to down select.

- CATT: don’t need both.

Solution 1:The decision to execute L3 measurement based handover is done by CU:

10 (Ericsson, Nokia, Huawei, ZTE, CATT, Samsung, Google, LG, Docomo, Vodafone)

Solution 2:The decision to execute L3 measurement based handover is done by DU

2 (Qualcomm and Jio)

Solution 3: The decision to execute L3 measurement based handover can be done by CU or DU

1 (Qualcomm)

To go with a majority is not possible as Qualcomm objects and believe that RAN4 requirements cannot be met and L1/L2 based decisions should be by DU (not based on CU triggers)

Conclusion: Moderator would like to propose to go with the majority during the official RAN3#126 meeting, noting that there are objections from 2 companies.

Solution 1: The decision to execute L3 measurement based handover is done by CU

- Supported by majority (see above)

Solution 2: The decision to execute L3 measurement based handover is done by DU

Solution 3: The decision to execute L3 measurement based handover can be done by CU or DU

3. Date of Next RAN2 Meetings:

TSG-RAN3 #127	February  17th to 21st, 2025	Athens, Greece