3GPP TSG-RAN WG3 Meeting #126	R3-247797

Orlando, USA, 18th – 22th Nov 2024

Agenda Item:	16.4

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CB: # AIoT3_LocationReport

Document for:	Approval

# Introduction

CB: # AIoT3_LocationReport

- Discuss how to support to report reader information as device location to CN?

- Capture the agreement to TP

(moderator - Nok)

Summary of offline disc R3-247797

# For the Chairman’s Notes

Topology 1

For topology 1, Reader ID is included in the Inventory report and Command report message in addition of AIoT RAN node ID.

For topology 1, only one reader ID included in Inventory report and Command report message. If AIoT RAN node would receive multiple responses for the same AIoT device it can send multiple inventory report messages.

For topology 1, no need to include an AIoT Device location in the Inventory report and Command report message.

Whether to have awareness of reader location in AIoT CN needs further study.

Topology 2

For topology 2, the AIOT CN may get UE reader location via ULI in NGAP messages or via legacy LCS mechsnism.

# Discussion-First round

RAN3 already agreed in the TR 38.769:

A-IoT device location information may be used for the following purposes:

(a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the A-IoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

## Topology 1

The current TR 38.769 currently agreed the following granularity of location.

For topology 1, A-IoT RAN node ID can be considered as a location of A-IoT device. For topology 2, UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device.

However, at last RAN3 meeting the following agreement was also taken:

The following principle applies to Topology1 and Topology2:

For both purpose a) and b), identifier of the reader may be reported to AIoT CN in the Inventory Report message.

This means that even for purpose a) the reader ID can be reported in the Inventory report.

It was also agreed that an AIoT RAN node may support multiple readers i.e. multiple TRPs. In that case the reader ID is the TRP ID. This means that for topology 1 the “reader granularity” will be finer than AIoT RAN node ID.

Proposal 1: confirm that Reader ID is included in the Inventory report and Command report message.

Proposal 1 agreed.

------------------------------------

To achieve purpose b) the AIoT CN would need to estimate the location of the “reader ID” received in inventory report.

The following options would be possible.

- Option 1: the AIoT CN may be configured by O&M about the readers supported by an AIoT RAN node and their location.

- Option 2: the AIoT CN may receive the list of readers supported by an AIoT RAN node and their location in an XXAP or NGAP Setup/Configuration Update message received beforehand.

This question however relates to the open issue of whether to have awareness of reader location on the AIoT CN.

Another issue is that even if AIoT CN gets reader location awareness, it may not be enough to cover all use cases of purpose b).  No agreement to standardize option 2.

------------------------------------

In topology 1 tdoc R3-247492 proposes that the AIoT device location is also optionally reported in the inventory report. This is for the case that the AIoT RAN node manages to get device location by some implementation means.

It was commented that the case of multiple “activation” readers is out of scope. In case multiple “receiving readers” receive the AIoT device report, this should be rare and in that case multiple inventory report messages can be sent from AIoT RAN node to AIoT CN. Also, in that case, we don’t consider the case of  an AIoT RAN node uses an algorithm to compute itself some device location : this is because it would be part of positioning which is out of scope.

This leads to following conclusions:

- Only single reader ID in inventory report message;

- No AIoT device location IE in inventory report message.

## Topology 2

At RAN3#126 the following was proposed for purpose b)

In topology 2, report ULI as the reader location information in the legacy NGAP UE associated signalling or the Inventory Report and Command Report messages?

The following options apply:

Option1: include ULI in the inventory report and command report messages.

Option 2: no need for AIoT CN to receive ULI because anyway not enough to achieve purpose b)

Agreed that AIOT CN may get UE reader location via ULI in NGAP or via legacy LCS mechsnism.

# Conclusion, Recommendations

# References