3GPP TSG-RAN WG3#126	R3-247836

Orlando, USA, 18-22 Nov 2024

Agenda Item:	8.1

Source:	Qualcomm Inc

Title:	Summary of Discussion on XR for dynamaic PSI activation/deactivation

Document for:	Discussions & Approval

# Introduction

- Reply to R3-247009 : Reply to SA2 LS on multi-modality awareness

RAN3 reply to SA2 and RAN2 to confirm the usage of MMSID, while traffic synchronization can be further checked in RAN3 based on progress in RAN2, SA2 and SA4.

- Reply to R3-247113 : LS for PDU Set Information Marking Support without QoS parameters

Whether the NG-RAN node need to inform SMF on activating/deactivating the PDU Set Information marking or inform SMF about support of the PDU Set Information marking?

Based on above Incoming LSes discussion, this email discussion handles following discussion.

CB: # 1_XRIncomingLS

- Check the open issue as above

- Provide the reply LS towards R3-247009 and R3-247113

(moderator - QC)

# For Chairman’s notes

<TBD>

Summary for Chair Notes:

9 companies provide response. 4 companies supportive, 4 companies not supportive and 1 company said wait for RAN2 discussion .

Based on this,

It is proposed to send reply LS indicating that some companies believe it is beneficial to support dynamic PSI activation/deactication from RAN to 5GC. Some companies think, dynamic frequenet PSI activation/deactivation indication from RAN to 5GC is not needed, which adds signaling overhead and instead RAN can ignore PSI header if not used by RAN.

Send reply LS  to SA2 with above information.

# 3	Discussion

## 3.1 Dynamic Indication from RAN to 5GC for PSI activation/deactivation

Chair Note: Whether the NG-RAN node need to inform SMF on activating/deactivating the PDU Set Information marking or inform SMF about support of the PDU Set Information marking?

SA2 discussed about dynamic PSI activation/deactivation from UPF to RAN for the purpose of assisting UPF whether and when to process RTP header information (received over N6 interface) and sending PSI via GTP-U header towards RAN. UPF makes decision to dynamically activate/deactivate PSI processing based on RAN requesting to activate/deactivate PSI Information. Within RAN, without receiving any PDU Set QoS information, RAN may use PSI information to make decision about whether and which PDU sets to dropped during the congeation situation.

This is new feature introduced by SA2 and its motivation is not same as R18 XR feature where PDU Set QoS Information is provided to RAN during PDU Session Resource setup/modification procedure and if RAN indicates its support and UPF starts sending PSI information to RAN via GTP-U header. RAN will use PSI information for the purpose of DL PDU Set discarding when there is congestion.

In RAN3, some companies are thinking that based on new indicator received from AMF (i.e, 5GC will provide PSI information to RAN without providing PDU Set QoS parameters), RAN can just re-use R18 XR introduced IE to indicate whether RAN supports PSI handling or not and no need to introduce NG-AP signaling changes to dynamically indicate activation/deactivation (when congestion about to start -> activation needed, when congestion is ended -> deactivation needed) of PSI. Even if RAN gets PSI information all the time, if there is no congestion then RAN can just ignore this PSI information received from UPF.

From moderator understanding, this is not aligned with the SA2 motivation of introducing PSI without PDU Set QoS Information and does not help UPF to reduce unnecessary PSI information processing.

Question 1 : Do companies agree with SA2 motivation (as described above) of introducing PSI information without providing PDU Set QoS Information to RAN ?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | SA2 motivation was to provide PSI information to RAN only when needed based on dynamic activation/deactivation request received from RAN during congestion scenario. Main goal was to reduce UPF processing overload when RAN does not want to use PSI. |
| Huawei | Partially Yes. | We agree with part of the clarified motivation. The motivation of introducing PDU Set information without providing PDU Set QoS Information to RAN is “RAN may use PSI information to make decision about whether and which PDU sets to dropped during the congeation situation.” But not “UPF makes decision to dynamically activate/deactivate PSI processing based on RAN requesting to activate/deactivate PSI Information.” So, based on our understanding, the key point for supporting this feature is RAN can indicate CN that the PDU set information (maybe only the PSI) is still useful for RAN even if without PDU set QoS information. We do not see the strong motivation for dynamic activation/deactivation of PDU set marking at UPF. If the RAN sends the activation/deactivation indicator according to the congestion information, such interaction between RAN and CN will be frequently and the signaling overhead over NG will be increased, since the radio interface status may be changed dynamically. Moreover, if this dynamic activation/deactivation of PDU set information marking based on RAN congestion status is really beneficial for UPF, it is also possible that the CN (e.g., UPF) can make the decision by itself according to the received information about congestion, and this is up to implementation. Regarding this question, we have a further question for clarification: The moderator use “introducing PSI information without providing PDU Set QoS Information to RAN”, does it mean that for such scenario, the UPF only need to provide PSI to RAN? |
| CATT |  | Maybe we should discuss usage first. The PDU Set Information marking can be benefit for PSI based discard and it may also benefit for scheduling e.g., network can prioritize the PDU set of which 80% PDU already has been successful transmitted or the PSI is high. In general, how to use PDU Set Information marking for improving performance is implementation. If NG-RAN supports PDU Set Information marking, considering it will benefit for PSI discard and scheduling at least, there is no reason to deactivate it by NG-RAN. |
| Ericsson |  | We agree with Huawei’s analysis. Please note that reusing the existing PDU Set handling indication IE is anyway needed, when the CN does not send any PSQP to RAN, but would like to know if RAN supports PDU Set handling before instructing the UPF to do any marking. So there is no need to create a duplicate solution. Further, how the RAN uses PDU Set Importance (PSI) information to decide which PDU sets to be dropped during the congestion is implementation dependent decision, as described in TS 38.300/23.501. Each RAN may have different implementation algorithm on how to handle local congestion depending on radio donidtions, thus this dynamic indication would create different level of signalling over NW nodes (from DU->CU->SMF), and SMF may be updated too frequently or too little and give wrong assumption. This will lead to an IOT issue. Finally, in legacy, the PSI-based discarding mechanism was already in place, and we did not have any dynamic indication over CP about RAN congestion level. Based on the reasons above, we think the reasons for introducing this dynamic notification motivated by only some companies in SA2 are moot. |
| Nokia | Partially (with comments) | It is agreeable for RAN to inform CN that it can make use of the PDU Set Information Marking. It should also be possible to perform and use PDU Set Information Marking even without PDU set QoS parameters. But SA2 does not have a conclusion on whether this RAN to CN indication is sent at any time. This is why SA2 asked RAN3 to provide feedback on whether “NG-RAN could dynamically request the SMF…. at any time with an NG-RAN initiated procedure”. There is no need for at any time with an NG-RAN initiated procedure based on following reasons: - When a QoS flow is established/modified, the gNB know whether it could perform PSI-based discard later. So this indication can be provided to CN as a response to CN-initiated QoS flow setup/modification. - In case “at any time with an NG-RAN initiated procedure”, it may be inefficient, since it may be too late to activate the marking when the congestion happens. The gNB may still receive some un-marked pakcets which just make the congestion even worse. It is more efficient to receive the marked packets in advance, and up to gNB to decide whether use it or not, e.g. to enable the gNB take quick action in case a congestion happens. The “at any time” approach adds more signaing load and complexity in both gNB and CN nodes. There is no clear benefit as we explained above. |
| ZTE | YES | Share the similar view with QC. In addition, from our understanding, act/deact mechanism may also helpful in the future for not only per node basis, but also for per PDU session basis. Finer granularity control for the PDU set handling function can always bring more flexibility for the NW in different cases. |
| SS |  | We still suggest to wait RAN2’s conclusion first, after all RAN2 will give the feedback on if the PDU Set based handling without PDU set QoS parameters is useful or not. It is unnecessary to send the reply LS so urgent. |
| CMCC | Yes | From RAN perspective, it is useful for the NG-RAN to be aware of the capability of PDU Set Information Marking supported in the UPF, a new PDU Set Information Marking Support Indicator from the UPF is needed if PDU Set QoS parameters is absence. Whether the NG-RAN can activate/deactivate the PDU Set marking at the UPF, we think it is benefitial. Under the uncongested status, especially for those NG-RAN nodes that have significantly less traffic during night time, the NG-RAN node could deactivate the PDU Set marking at the UPF, given this PDU Set information is likely not be used. Consequently, the UPF could spend less resources on PDU Set marking process. On the other hand, we agree with some other company’s view, such activation/deactivation should not be frequent to cause RAN to CN control signalling overhead. |
| Vivo | Yes | RAN2 already concluded that it is useful on the PDU Set based handling without PDU set QoS parameters. The typical scenario for PDU set based handling without PDU set QoS parameters is for PSI based SDU discard in RAN congestion case as analyzed above, which is different than legacy PDU Set QoS related handling. As a result, in case of no congestion happened in RAN, there is no need to handle such PDU Set based marking information in NG-RAN Node. Thus, SA2 also provides a solution that allow NG-RAN dynamically to request the SMF to activate or deactivate the PDU Set Information marking at any time NG-RAN needed. And PDU SESSION RESOURCE MODIFY INDICATION procedure could be used for example. In this case, NG-RAN dynamically to request the SMF to activate or deactivate the PDU Set Information marking when there is need, e.g. when there is congestion in RAN. |

Question 2 : If RAN does not provide any dynamic activation/deactivation of PSI information, how it will help UPF to reduce PSI processing overhead ? please explain .

| Company | Does it meet SA2 design motivation ? |
| --- | --- |
| Qualcomm | No, It does not help to fulfill SA2 design motivation. |
| Huawei | As we answered for the previous question, if the dynamic activation/deactivation of PDU set information marking based on RAN congestion status is really beneficial for UPF, it is also possible that the CN (e.g., UPF) can make the decision by itself according to the received information about congestion, and this is up to implementation. |
| CATT | Even no congestion, the UPF can still provide PDU set information to achieve a better performance for XR e.g., scheduling, this is not meaningless overhead in NG-U. |
| Ericsson | As stated above, not needed. We already have L4S, which is a better way for congestion notification. |
| Nokia | The question is not valid, since this is not the exact SA2 requirement. Please refer to the SA2 LS. SA2 has no conclusion whether need this RAN indication at any time to help UPF to reduce processing overhead. The requirement is about enabling PDU set marking and handling even without PDU set QoS parameters. There is no requirement to help UPF to reduce PSI processing overhead. So please focus the discussion on the SA2 requirement. |
| ZTE | No, It does not help to fulfill SA2 design motivation. |
| SS | Dynamic activation/deactivation by RAN is not needed. |
| CMCC | No, It does not help to fulfill SA2 design motivation. |
| vivo | No, It does not help to fulfill SA2 design motivation. |

Summary :

Summary : 4 companies are supportive to support dynamic dynamic PSI activation indication from RAN to UPF to address both RAN congestion and UPF overhead processing reduction.

1 company suggested to wait for RAN2 discussion and no need to reply LS at all until RAN2 concludes. From moderator perspective, there is no need to wait for RAN2 conclusion.

4 companies are not supportive for dynamic PSI activation indication from RAN to UPF expressing concern about signaling overhead between RAN and CN.

There is no majority in any direction based on this discussion.

Moderator proposes to send reply LS indicating that some companies believe it is beneficial to support dynamic PSI activation/deactication from RAN to 5GC. Some companies think, dynamic frequenet PSI activation/deactivation indication from RAN to 5GC is not needed, which adds signaling overhead and instead RAN can ignore PSI header if not used by RAN.

Send reply LS  to SA2 with above information.