3GPP TSG-RAN3 Meeting #126	R3-24xxxx

Orlando, USA, 18th - 22nd Nov 2024

- Title:	(TP to 38.300) Architecture of NR Femto

- Source:	ZTE Corporation

- Agenda Item:	12.3

- Document for:	Discussion, Decision

# TP for NR femto

- 4.X	Support of NR Femtos

- 4.X.1	Architecture

Figure 4.X.1-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto to the 5GC.

Figure 4.X.1-1: NR Femto Logical Architecture

NOTE: the SeGW is out of RAN scope.

The NR Femto may directly connect to the 5GC. The NG-RAN architecture may also deploy an NR Femto Gateway (NR Femto GW) to allow the concentration of the NG-C interface between the NR Femto and the 5GC. Based on implementation, the transport of NG-U between the NR Femto and the 5GC may be optionally concentrated in the NR Femto GW. NR Femto may serve more than one cell.

For NR Femto, the NG-C interface is defined as the interface:

-	Between the NR Femto GW and the Core Network;

-	Between the NR Femto and the NR Femto GW;

-	Between the NR Femto and the Core Network;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto as an AMF. The NG interface between the NR Femto and the 5GC is the same regardless whether the NR Femto is connected to the 5GC via an NR Femto GW or not.

The NR Femto shall only connect to a single NR Femto GW at one time when the NR Femto connects via the NR Femto GW. The NR Femto GW supports NG-Flex configuration and can simultaneously connect to multiple AMFs.

The functions supported by the NR Femto shall be the same as those supported by a gNB (with possible exceptions e.g. NNSF when the NR Femto connects via the NR Femto GW) and the procedures run between an NR Femto and the 5GC shall be the same as those between a gNB and the 5GC.

Xn-connectivity is supported between NR Femtos and between NR Femtos and gNBs, independent of whether any of the involved NR Femtos is connected to an NR Femto GW.