3GPP TSG-RAN WG3 #126	draft R3-247809

Orlando, FL, USA, 18-22 November 2024

Agenda Item:	12.3

Source:	Ericsson

Title:	Access Control with NR Femto – Stage 2 TP

Document for:	Discussion

# Text Proposal for TR 38.300 BL CR (R3-247030)

START OF CHANGES

### 4.x.4	Access Control

- Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription.

NOTE: The NR Femto node may use the CAG mechanism for PNI NPN (see clause 16.7.4) as follows:

-	The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	The NR Femto may activate a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

END OF CHANGES