3GPP TSG-RAN WG3 Meeting #127                                                              R3-250102

Athens, Greece, February 17 – 21, 2025

Agenda item: 		12.2

Source: 		Qualcomm Incorporated

Title: 	(TPs to BL CR 38.401, 38.413) WAB architecture and mobility

Document for:		Discussion

# 1	Introduction

This contribution discusses remaining issue of WAB related to:

- Authorization of WAB-node

- Preventability of multi-hop WAB during WAB-MT network access

- Preventability of inter-WAB-gNB Xn establishment

- Handling of UE’s AMF change

- Handling of IP address change for WAB-gNB

- PCI collision avoidance

- Additional ULI to be included with UE’s ULI

The discussion aims to make further progress on these issues.

# 2	Discussion

## 2.1	Authorization of WAB-node

SA2 has defined multiple aspects of WAB-node authorization in TS 23.501 clause 5.49.3. Several of these aspects are related to RAN. RAN3 should capture these RAN-related aspects on stage-2 together with a reference to TS 23.501.

Proposal 1: TS 38.401 to capture RAN-related aspects of WAB-node authorization as defined in TS 23.501.

## 2.2	Preventability of multi-hop WAB during WAB-MT network access

Last meeting, RAN3 agreed on the preventability of multi-hop WAB during WAB-MT mobility [1]:

For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling.

It remains to be discussed on how multi-hop WAB can be prevented during WAB-MT access. Based on offline discussion in last meeting, it would be desirable to support a solution that does not require WAB-specific enhancements to the UE stack, since such enhancement might be economically viable for a rollout of a small number of WAB-nodes.

The following alternatives can be considered:

Alternative 1: WAB-MT is prevented to perform cell (re)selection of a WAB-gNB cell

Alt 1a: Using dedicated CAG ID for WAB: The WAB-MT is configured to only connect to cells broadcasting a WAB-specific CAG ID. WAB-gNBs are not configured with this WAB-specific CAG ID. The BH RAN is configured to broadcast the WAB-specific CAG ID.

Alt 1b: Using forbidden TAC list: The WAB-MT is configured with a list of forbidden TACs, which contains all TACs used by WAB-gNBs. The BH RAN is configured to broadcast TACs that are different from those used by WAB-gNBs.

Alternative 2: WAB-MT is handed over to a BH RAN node when accessing a WAB-gNB cell.

When the WAB-MT connects to a WAB-gNB cell, the WAB-gNB conducts a handover of the WAB-MT to a BH RAN node. The WAB-gNB identifies the WAB-MT by the WAB-specific S-NSSAI included in Msg. 5.

All of the above alternatives use legacy UE functionality for the WAB-MT. They all can be supported based on implementation. There is no need to define other solutions, which define a new behaviour or stage-3 enhancements for the WAB-MT.

For Alt.1a, the WAB-MT needs to support for PNI-NTN, which is an optional feature for UEs. Alt.1b and Alt.2, the WAB-MT does not have to support any optional UE feature.

In the last meeting, one operator raised concerns that allocating additional identifier space for WAB would overly restrict deployments. For Alt1a, one CAG ID needs to be allocated for WAB out of a 4x109 CAG ID space. For Alt1b, it is sufficient to allocate one TAC for WAB out of 4x106 TAC space. Such allocations should not pose any constraint on actual deployments. For Alt2, no additional identifier is needed for the prevention of multi-hop WAB.

Proposal 2: Multi-hop WAB during WAB-MT access is preventable via implementation, e.g., by using one of the following alternatives:

- Alt. 1a: WAB-MT only connects to cells broadcasting WAB-specific CAG ID, which is not broadcasted by WAB-gNBs,

- Alt. 1b: WAB-gNB broadcasts WAB-specific TACs, which are contained in WAB-MT’s forbidden TAC list,

- Alt. 2: In case WAB-MT connects to WAB-gNB, it is handed over to BH RAN.

## 2.3	Preventability of inter-WAB-gNB Xn establishment

According to the WID for RAN’s WI on Additional Topological Enhancements for NR, RAN3 should define the support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs [2].

To prevent the Xn establishment between WAB-gNBs, a WAB-gNB needs to be made aware whether a neighbour node is a WAB-gNB or another RAN node. For this purpose, WAB-gNBs can be configured with a reserved range of PCI values, gNB-ID values, NR-CGI values and/or TAC values that are not used by the BH RAN in the relevant area of the WAB-nodes’ operation. When a WAB-gNB initiates the Xn Setup procedure toward a peer WAB-node, the peer WAB-node can identify that the request was initiated by a WAB-node since the served cells use some of these WAB-reserved parameters.

This mechanism for preventability of inter-WAB-gNB Xn establishment does not require any specification enhancement.

Proposal 3: Xn establishment between WAB-gNBs is preventable via implementation, e.g., by having the WAB-gNB reject the Xn Setup request from peer gNB that uses WAB-reserved ranges of PCIs, gNB-IDs, NR CGIs and/or TACs.

## 2.4	Handling of UE’s AMF change

Last meeting, RAN3 agreed on the handling fo the UE’s AMF change [1]:

The “two logical gNB solution” can support UE’s AMF change during WAB-gNB mobility.

RAN3 further sent an LS to SA2 to describe solutions where the UE’s AMF change via use of a single logical WAB-gNB [3]. In this LS, RAN3 describes two solutions:

Solution 1: Using single WAB-gNB with single cell and changing TAC

Solution 2: Using single WAB-gNB with two cells and different TACs

SA2 discussed this LS in the last meeting in January 2025 and agreed CRs S2-2501319 and S2-2501320 to TS 23.501 that address aspects raised in this LS.

S2-2501319 states:

When it is required to change AMF for the UEs served by the MWAB-gNB upon mobility of a MWAB into a new region, the TAC(s) and Cell ID(s) the MWAB-gNB announces also change. The MWAB-gNB, for UEs in RRC_CONNECTED mode, behaves as specified in TS 38.401 [42].

S2-2501320 states:

- During mobility, a MWAB-gNB can obtain or apply a new configuration as specified in TS 38.401 [42] from the OAM server of MWAB Broadcasted PLMN and, if any new AMF was provided as part of the new configuration, the MWAB-gNB sets up the N2 connection with the new AMF and it releases the N2 connection with any old AMF removed from the MWAB-gNB configuration NOTE: It is expected the MWAB-gNB removes the old configuration after the MWAB-gNB has completed the handover of the UEs in RRC_CONNECTED state.

This implies that SA2 has uphold its prior decision that the TAC(s) and Cell ID(s) change during AMF-change. Therefore, SA2 has implicitly precluded solution 1.

SA2 has further sent a Reply LS to RAN3 in R3/250073/S2-2501326, where it states:

SA2 Answer on the two logical gNB solution: SA is happy to endorse the two logical gNB solutions as there is no impact to existing behaviour in the CN and systemwide. SA2 Answer on the single gNB solutions: There is no consensus on the support of a single logical gNB solution in SA2.

Based on SA2’s prior agreements and Reply LS, RAN3 should preclude single-gNB solutions.

Proposal 4-1: Based on SA2 Reply LS and specification, single gNB solutions for AMF change to be precluded.

RAN3 to capture the two logical gNB solution on stage 2.

Proposal 4-2: RAN3 to capture the two logical gNB solution on stage 2.

## 2.5	Handling of WAB-gNB’s traffic during PDU session change

According to the WID for RAN’s WI on Additional Topological Enhancements, RAN3 should define the handling of the WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes [2].

The only critical scenario to be considered is when the WAB-MT’s PDU session changes. In this case, the IP address(es) of the WAB-MT change(s), too. NG-, Xn- and OAM traffic need to cope with the change of IP addresses.

The following solutions can be considered:

- IP address change for NG-C: According to TS 38.412, the WAB-gNB can establish a new SCTP association with the AMF using one of the new IP addresses and conduct the RAN CONFIGURATION UPDATE procedure as the first procedure via this SCTP association.

- IP address change for Xn-C: According to TS 38.422, the WAB-gNB can establish a new SCTP association with the peer gNB using one of the new IP addresses and conduct the RAN CONFIGURATION UPDATE procedure as the first procedure via this SCTP association.

- IP address change for NG-U: According to TS 38.413, the WAB-gNB can migrate the NG-U GTP-U tunnels to the new IP address(es) via the NGAP PDU session Resource Modify Indication procedure.

- IP address change for Xn-U: Xn-U is used for data forwarding during handover and for DC (split bearers, MN-terminated SCG bearer, SN-terminated MCG bearer). Data forwarding during handover usually covers only a short time frame. In case data forwarding is interrupted due to change of tunnel end point IP addresses, it may be easier to retransmit the packets on upper layers. For DC, the GTP-U tunnels can be migrated via the S-NG-RAN node MODIFICATION PROCEDURES.

- IP address change for OAM traffic: The impact of IP address change on OAM traffic is out-of-scope. Also, an equivalent scenario was already supported for IAB, where OAM traffic can be exchanged via the backhaul, whose IP addresses can change.

Proposal 5: RAN3 to capture on stage-2 that when WAB-gNB changes IP address due to WAB-node mobility, the WAB-gNB’s traffic can be handled in the following manner:

- NG-C and Xn-C can be migrated to new IP address via legacy procedures defined in TS 38.412 and TS 38.422, respectively.

- NG-U GTP-U tunnels can be migrated via the NGAP PDU session Resource Modify Indication procedure.

- Xn-U GTP-U tunnels used for DC can be migrated via the Xn S-NG-RAN NODE MODIFICATION PROCEDURES.

- Xn-U GTP-U tunnels used during UE handover do not need to be migrated since short-lived.

- The migration of OAM traffic to the new IP address(es) is out of scope.

## 2.6	PCI configuration and collision avoidance

According to the WID for RAN’s WI on Additional Topological Enhancements for NR, RAN3 should define the handling of PCI collision avoidance [2]. The PCI configuration for gNB cells is defined in TS 38.401, clause 7.8. This clause further defines the following procedure for PCI collision avoidance for mobile IAB:

For mobile IAB deployments, the legacy mechanisms can be reused for PCI collision detection. The PCI space can be partitioned between mobile IAB cells and stationary cells by implementation.

Based on NOTE 1 in WID [1], these procedures should also be applied to handle PCI configuration and collision avoidance of WAB-gNB cells.

Proposal 6: PCI configuration and PCI collision avoidance to follow the same procedure as defined for mobile IAB in TS 38.401, clause 7.8.

## 2.7	Additional ULI to be included with UE’s ULI

Last meeting, RAN3 achieved the following agreement, which was liaised with SA2:

WA: Additional ULI for WAB consists of TAI and NR CGI, which are determined by the WAB-node, reflecting the WAB-node’s physical location. This solution is compliant with Opton1 and Option3. It is up to SA2 to support one of Opton1 and Option3 or both

SA2 has sent a Reply LS in R3-250071/S2-2501324, which includes on this specific issue:

SA2 discussed the RAN3 Reply to issue-2 and has agreed 23.501 CR 5968.

CR 5968 to TS 23.501 in S2-2501322 states:

The MWAB-gNB shall also provide the Additional User Location Information (Additional ULI) that reflects the location of the MWAB-UE to the AMF when it sends the User Location Information (ULI) over N2 messages. In the Additional ULI, the MWAB-gNB provides the TAI/NR CGI of the MWAB Broadcasted PLMN/SNPN with which the UE is registered as specified in TS 38.401 [42]. NOTE: For the case of MWAB broadcasted PLMN/SNPN being different from the BH PLMN, MWAB-gNB formulates the Additional ULI as specified in TS 38.401 [42] based on MWABs geo-location. The MWAB can determine its geo-location using GNSS or via a MWAB-UE initiated 5GC-MO-LR procedures as specified in TS 23.273 [87]. If the MWAB broadcasted PLMN/SNPN is the same as the BH PLMN, then the TAI/NR CGI of the cell serving the MWAB-UE is included in the Additional ULI as specified in TS 38.401 [42].

RAN3 should capture the additional ULI to the User Location Information IE in TS 38.413 in alignment with this CR.

Proposal 7-1: In alignment with SA2’s CR 5968 to TS 23.501, include Additional ULI to User Location Information IE in TS 38.413 (TP to BL CR provided in the Annex).

In alignment with this CR, RAN3 should also capture the determination of the additional ULI in TS 38.401.

TS 23.501 clause 5.49.4 further defines the WAB-gNB’s behaviour in case the additional ULI changes due to WAB-node mobility:

If serving cell change report is required by AMF as specified in clause 4.10 of TS 23.502 [3], the MWAB-gNB shall report Additional ULI (and ULI) for the UE even when only serving cell of the MWAB-UE changes.

RAN3 should capture in TS 38.401 that the WAB-gNB may have to send User Location Reports when the additional ULI has changed due to WAB-node mobility.

Proposal 7-2: TS 38.401 to include the WAB-gNB’s determination of the additional ULI and the need for User Location Reports in case the additional ULI has changed due to WAB-node mobility.

In the last meeting, RAN3 further identified the following open issues:

It is FFS whether there are issues with applying the solution to MT access via NTN. It is FFS whether further information needs to be included in the additional ULI

In case the backhaul uses NTN, the WAB-MT’s OTA served cell ID does not reflect the physical location of the WAB-MT. Therefore, concerns were raised that the WAB-MT’s served cell ID could not be used as the base for the additional ULI to be included with the UE’s ULI.

SA2’s CR has addressed this issue: In case the backhauling uses NTN, the WAB-node first determines its geolocation and then derives the additional ULI from this geolocation information. This implies that the WAB-MT’s OTA served cell ID is not used for the additional ULI. Therefore, further discussion is not needed.

Proposal 7-3: RAN3 to not further discuss the determination of the additional ULI in case of NTN backhauling, since SA2’s CR 5968 to TS 23.501 has defined a solution for this matter.

## 2.8	Stage-2 specification

In one of the last meetings, RAN3 discussed on how to split stage-2 definitions of WAB between TS 38.300 and TS 38.401 [3].

Since the WI on Additional Topological Enhancements for NR includes WAB and NR Femto, they both should be captured in the same stage-2 spec(s). Presently, RAN3 has endorsed CRs to TS 38.300 for NR Femto. Therefore, TS 38.300 should also include an introduction to WAB. All the details of WAB (authorization, network integration, WAB-node mobility, etc.) can then be captured in TS 38.401. This approach is also well aligned with the specification for mobile IAB.

Proposal 8-1: As for NR Femto, the introduction of WAB to be included in TS 38.300.

A BL draft CR to 38.300 has been submitted separated.

In the Annex below, please find a TP for the BL CR to TS 38.401.

Proposal 8-2: RAN3 to approve TP for BL CR to S 38.401 in the Annex.

# 3	Conclusion

This contribution discussed the aspects of WAB. The following observations and proposals have been made:

Authorization of WAB-node

Proposal 1: TS 38.401 to capture RAN-related aspects of WAB-node authorization as defined in TS 23.501.

Preventability of multi-hop WAB during WAB-MT network access

Proposal 2: Multi-hop WAB during WAB-MT access is preventable via implementation, e.g., by using one of the following alternatives:

- Alt. 1a: WAB-MT only connects to cells broadcasting WAB-specific CAG ID, which is not broadcasted by WAB-gNBs,

- Alt. 1b: WAB-gNB broadcasts WAB-specific TACs, which are contained in WAB_MT’s forbidden TAC list,

- Alt. 2: In case WAB-MT connects to WAB-gNB, it is handed over to BH RAN.

Preventability of inter-WAB-gNB Xn establishment

Proposal 3: Xn establishment between WAB-gNBs is preventable via implementation, e.g., by having the WAB-gNB reject the Xn Setup request from peer gNB that uses WAB-reserved ranges of PCIs, gNB-IDs, NR CGIs and/or TACs.

Handling of UE’s AMF change

Proposal 4-1: Based on SA2 Reply LS and specification, single gNB solutions for AMF change to be precluded.

Proposal 4-2: RAN3 to capture the two logical gNB solution on stage 2.

Handling of WAB-gNB’s traffic during PDU session change

Proposal 5: RAN3 to capture on stage-2 that when WAB-gNB changes IP address due to WAB-node mobility, the WAB-gNB’s traffic can be handled in the following manner:

- NG-C and Xn-C can be migrated to new IP address via legacy procedures defined in TS 38.412 and TS 38.422, respectively.

- NG-U GTP-U tunnels can be migrated via the NGAP PDU session Resource Modify Indication procedure.

- Xn-U GTP-U tunnels used for DC can be migrated via the Xn S-NG-RAN NODE MODIFICATION PROCEDURES.

- Xn-U GTP-U tunnels used during UE handover do not need to be migrated since short-lived.

- The migration of OAM traffic to the new IP address(es) is out of scope.

PCI configuration and collision avoidance

Proposal 6: PCI configuration and PCI collision avoidance to follow the same procedure as defined for mobile IAB in TS 38.401, clause 7.8.

Additional ULI to be included with UE’s ULI

Proposal 7-1: In alignment with SA2’s CR 5968 to TS 23.501, include Additional ULI to User Location Information IE in TS 38.413 (TP to BL CR provided in the Annex).

Proposal 7-2: TS 38.401 to include the WAB-gNB’s determination of the additional ULI and the need for User Location Reports in case the additional ULI has changed due to WAB-node mobility.

Proposal 7-2: RAN3 to not further discuss the determination of the additional ULI in case of NTN backhauling, since SA2’s CR 5968 to TS 23.501 has defined a solution for this matter.

Stage-2 specification

Proposal 8-1: As for NR Femto, the introduction of WAB to be included in TS 38.300.

Proposal 8-2: RAN3 to approve TP for BL CR to S 38.401 in the Annex.

# Reference

[1]  Chair notes, 3GPP TSG RAN WG3 Meeting #126bis, Orlando, Florida, USA, November 2024

[2]  RP-242395, New WID on additional topological enhancements for NR, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 2024

[3]  RAN3-247910, Reply LS on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility), 3GPP TSG RAN  WG3, Meeting #126, Orlando, Florida, USA, November 2024

# Annex 1: TP for BL CR to 38.401

START OF CHANGE

### 6.1.x	Wireless Access Backhaul architecture

A WAB-node consists of a WAB-gNB and a WAB-MT. The WAB-gNB is based on the gNB functionality specified in TS 38.300 [2] and serves UEs by means of a terrestrial NR Uu radio link.

The WAB-MT is served by a BH-RAN-node. The WAB-gNB traffic, including NG, Xn and OAM traffic is transported via backhaul PDU session(s) of the WAB-MT.

NOTE: The use of other types of backhaul, e.g. non-3GPP backhaul, is up to implementation.

The WAB-gNB and the WAB-MT may connect to the same PLMN or to different PLMNs.

Figure 6.1.x-1 shows the WAB architecture for 5GS.

Figure 6.1.x-1: The WAB architecture

In in-band scenarios, backhaul and access of the WAB-node use terrestrial radio links. In out-of-band scenarios, the backhaul can use a terrestrial or a non-terrestrial radio link, while the access uses terrestrial radio link.

The WAB-MT may connect to a public PLMN or an SNPN.

The WAB-gNB may connect to a public PLMN or an SNPN.

Figure 6.1.x-2 shows protocol stacks for NG Control plane and NG User plane transport via the wireless backhaul.

Figure 6.1.x-2: Protocol stacks for NG Control plane and NG User plane transport

Figure 6.1.x-3 shows protocol stacks for Xn Control plane and Xn User plane transport for WAB-node.

The WAB-gNB can establish Xn connections with the BH gNB and/or with other surrounding RAN nodes. The WAB-gNB can use legacy procedures to discover a neighbour RAN-node and resolves its TNL address.

The WAB-gNB can prevent establishment of Xn connections with other WAB-gNBs by rejecting the Xn Setup Request message received from peer WAB-gNBs. The WAB-gNB detects that the Xn Setup Request originates from a peer WAB-gNB based on the service cell information carried in this message, which may carry WAB-specific values for cell frequency, PCI, NR-CGI, or TAC.

= = = = > SKIP

## X.Y	WAB-node authorization

The WAB-MT needs to be authorized to support PDU sessions for backhauling. This authorization is based on WAB-specific S-NSSAI(s) and DNN as defined in TS 23.501[3]. This authorization can be time- and location-dependent.

The WAB-gNB needs to be authorized to provide services to UEs. This authorization is based on OAM configuration as defined in TS 23.501[3].

When the WAB-gNB’s authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB should be handed over to other RAN nodes or released. Then, the WAB-gNB’s NG and Xn connection(s) should be removed.

When the WAB-gNB’s authorization status changes from “not authorized” to “authorized”, the WAB-gNB can reestablish NG connection and potentially Xn connection(s), and then start serving UEs.

The authorization of WAB-gNB and collocated WAB-MT should be time-aligned so that the WAB-MT can provide backhaul support while the WAB-gNB has backhaul connections established and is serving UEs. This time alignment is achieved via implementation as defined in TS 23.501[3].

## X.Y+1	WAB-node mobility

### X.Y+1.Z	WAB-MT mobility procedures

The WAB-MT can use all mobility procedures defined for legacy UEs.

### X.Y+1.Z+1	Change of the AMF(s) for UEs served by the WAB-node

Due to WAB-node movement, the AMF(s) of the UE(s) served by the WAB-node may have to be changed, e.g., based on the WAB-node’s current location and/or additional criteria.

The procedure contains the following steps:

1.	The WAB-node instantiates a new logical WAB-gNB, and it establishes NG connection(s) towards one or more new AMF(s). The new logical WAB-gNB may also establish Xn connection(s) with the BH RAN or with other surrounding RAN nodes. The WAB-node may obtain the configuration parameters needed to establish the connection to the UE’s new AMF(s) via OAM.

2.	The new logical WAB-gNB may activate one or more new cells with cell configuration parameters related to the WAB-gNB’s current location. The TAC broadcasted by the new cell(s) is different from the TAC broadcasted by the old cell(s).

3.	The UEs are handled as follows:

-	A UE in RRC_CONNECTED state is handed over between an old cell served by the old logical WAB-gNB and a new cell served by the new logical WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502 [7]. After all UEs served by the WAB-node have been handed over, the old cell(s) are removed from service.

-	When the old cell(s) are removed from service, a UE in RRC_IDLE or RRC_INACTIVE state that has been camping on an old cell reselects a new cell and performs a Mobility Registration Update procedure as defined in TS 23.502 [7]. The Mobility Registration Update is triggered by the change of TAC between old and new cell.

4.	The NG connection(s) between the old logical WAB-gNB and the initial AMF(s), and all Xn connection(s) between the old logical WAB-gNB and the BH RAN and/or other RAN nodes are removed, and the WAB-gNB is removed from service.

### X.Y+1.Z+2 Handling of WAB-gNB’s traffic during PDU session change

When the WAB-MT’s PDU session used for backhauling changes, the WAB-gNB needs to update the IP addresses used for its backhaul traffic. The following procedures can be used:

- IP address change for NG-C: According to TS 38.412, the WAB-gNB can establish a new SCTP association with the AMF using one of the new IP addresses and conduct the RAN CONFIGURATION UPDATE procedure as the first procedure via this SCTP association.

- IP address change for Xn-C: According to TS 38.422, the WAB-gNB can establish a new SCTP association with the peer gNB using one of the new IP addresses and conduct the RAN CONFIGURATION UPDATE procedure as the first procedure via this SCTP association.

- IP address change for NG-U: According to TS 38.413, the WAB-gNB can migrate the NG-U GTP-U tunnels to the new IP address(es) via the NGAP PDU SESSION RESOURCE MODIFY INDICATION procedure.

- IP address change for Xn-U: Xn-U is used for data forwarding during handover and for DC (split bearers, MN-terminated SCG bearer, SN-terminated MCG bearer). Data forwarding during handover usually covers only a short time frame. In case data forwarding is interrupted due to change of tunnel end point IP addresses, it may be easier to retransmit the packets on upper layers. For DC, the GTP-U tunnels can be migrated via the S-NG-RAN NODE MODIFICATION procedures.

- IP address change for OAM traffic: The impact of IP address change on OAM traffic is out-of-scope. Also, an equivalent scenario was already supported for IAB, where OAM traffic can be exchanged via the backhaul, whose IP addresses can change.

### X.Y+1.Z+3 ULI for UEs camping on WAB-gNBs

When the WAB-gNB sends a User Location Report for a UE to the AMF, it includes an additional ULI, which represents the WAB-MT’s physical location [xx].

In case the PLMN ID used by the UE at the WAB-gNB cell is the same as the PLMN ID used by the WAB-MT at the BH RAN, the WAB-gNB includes the ULI of the WAB-MT’s served cell.

In case the PLMN ID used by the UE at the WAB-gNB is different from the PLMN ID used by the WAB-MT at the BH RAN, the WAB-gNB determines the WAB-MT’s geolocation and derives from this geolocation the additional ULI as defined in 23.501 [3].

## X.Y+2	Preventing formation of multi-hop WAB topologies

The multi-hop WAB topology can be formed when a WAB-MT connects to a non-collocated WAB-gNB. This release does not support enhancements for the support of multi-hop WAB topologies.

The formation of multi-hop WAB topologies can occur under the following circumstances:

- A WAB-MT connects to a non-collocated WAB-gNB during initial network access.

- A WAB-MT is handed over to a peer WAB-gNB.

The formation of a multi-hop WAB topology can be prevented via implementation, e.g., by using the following mechanisms:

- The WAB-MT is configured to only connect to cells broadcasting a WAB-specific CAG, and WAB-gNB cells are not configured with this CAG.

- The WAB-MT is configured with a list of forbidden TACs which are exclusively used by WAB-gNB cells.

- In case a WAB-MT connects to a WAB-gNB, the WAB-gNB hands the WAB-MT over to a BH RAN cell. The WAB-gNB recognizes the WAB-MT based on the WAB-specific S-NSSAI included in the RRC Setup Complete message.

- In case a BH RAN node requests a handover to a WAB-gNB, the WAB-gNB rejects the handover request. The WAB-gNB recognizes the WAB-MT based on the WAB-specific S-NSSAI included in the Xn Handover Request message or NG Handover Required message.

= = = = > SKIP

## 7.8	PCI Optimisation Function

The PCI Optimization Function in non-split gNB case is specified in TS 38.300 [2].

In split gNB architecture, the OAM configures a PCI for each NR cell to the gNB-DU.

For centralized PCI assignment in split gNB architecture, the gNB-CU detects PCI conflict of NR cells and reports the NR cells suffering PCI conflict to OAM directly. The OAM is in charge of reassigning a new PCI for the NR cell subject to PCI conflict.

For distributed PCI assignment in split gNB architecture, the OAM assigns a list of PCIs for each NR cell and sends the configured PCI list to the gNB-CU. If the gNB-CU detects PCI conflict, the gNB-CU may select a new PCI value from the preconfigured PCI list for the NR cell and send it to the gNB-DU by either F1 Setup procedure or gNB-CU configuration update procedure.

For mobile IAB and/or WAB deployments, the legacy mechanisms can be reused for PCI collision detection. The PCI space can be partitioned between mobile IAB cells, WAB cells and stationary cells by implementation.

END OF CHANGE

# Annex 2: TP for BL CR to 38.413

START OF CHANGE

#### 9.3.1.16	User Location Information

This IE is used to provide location information of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE User Location Information | M |  |  |  | - |  |
| >E-UTRA user location information |  |  |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >NR user location information |  |  |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI | M |  | 9.3.3.11 | This IE is ignored if the NR NTN TAI Information IE is present. | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >>NID | O |  | 9.3.3.42 |  | YES | reject |
| >>NR NTN TAI Information | O |  | 9.3.3.53 |  | YES | ignore |
| >>Mobile IAB-MT User Location Information | O |  | 9.3.1.260 | Indicates the user location information of a mobile IAB-MT, which is co-located with the mobile IAB-DU which serves the UE. | YES | ignore |
| >>Additional User Location Information | O |  | 9.3.1.x | Indicates the user location information of the WAB-MT, which is co-located with the WAB-gNB serving the UE. | YES | ignore |
| >N3IWF user location information |  |  |  |  |  |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the N3IWF | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TNGF user location information |  |  |  |  | YES | ignore |
| >>TNAP ID | M |  | OCTET STRING | TNAP Identifier used to identify the TNAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the TNGF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TWIF user location information |  |  |  |  | YES | ignore |
| >>TWAP ID | M |  | OCTET STRING | TWAP Identifier used to identify the TWAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | Non-5G-Capable over WLAN device's local IP address used to reach the TWIF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >W-AGF user location information |  |  |  | Indicates the location information via wireline access as specified in TS 23.316 [34]. | YES | ignore |
| >>W-AGF user location information | M |  | 9.3.1.164 |  | - |  |

NEXT CHANGE

#### 9.3.1.X	Additional User Location Information for WAB-MT

This IE contains the user location information for the WAB-MT that is co-located with the WAB-gNB serving the UE.

In case the UE served by the WAB-gNB uses the same PLMN as the co-located WAB-MT, this IE contains the NR CGI and the TAI of the WAB-MT’s served cell. In case the UE uses a different PLMN than the WAB-MT, the NR CGI and TAI are derived from the WAB-MT’s geolocation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.7 | In case UE and WAB-MT are served by the same PLMN, the NR CGI of the cell, which is the serving cell of the WAB-MT. In case UE and WAB-MT are served by different PLMNs, the NR CGI is derived from the WAB-MT’s geolocation. |
| TAI | M |  | 9.3.3.11 | In case UE and WAB-MT are served by same PLMN, the TAI supported by the cell, which is the serving cell of the WAB-MT. In case UE and WAB-MT are served by different PLMNs, the TAI is derived from the WAB-MT’s geolocation. |

END CHANGEs