3GPP TSG-RAN WG3 Meeting #127                                                              R3-250104

Athens, Greece, February 17 – 21, 2025

Agenda item: 		17.3

Source: 		Qualcomm Incorporated

Title: 	Aspects of on-demand SIB1 for NES enhancements

Document for:		Discussion

# 1	Introduction

In last meeting, RAN3 agreed on aspects related to OD SIB1:

The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2. The procedure uses non UE-associated signaling. Cell A stores the UL WUS configuration information after it has received it. A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. The procedure uses non UE-associated signalling. Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

This contribution aims to make further progress on the support for on-demand SIB1.

# 2	Discussion

## 2.1	Stage-2 procedures for OD SIB1

So far, RAN3 has discussed new signalling messages to be introduced on Xn for the support of OD SIB1. RAN3 has not yet discussed the overall stage-2 procedures to enable OD SIB1 operation across NES-cell gNB-DU, NES-cell gNB-CU, Cell-A gNB-CU and Cell-A gNB-DU.

The following stage-2 procedures need to be considered:

- Initiation of OD SIB1 support for NES cell.

- Revocation of OD SIB1 support for NES cell.

- Reinstatement of OD SIB1 support for NES cell.

- Update of UL WUS configuration transmission status by Cell-A gNB.

These procedures are discussed in the following subsections.

Proposal 2-1: RAN3 to define the stage-2 procedures across NES-cell gNB-DU, NES-cell gNB-CU, Cell-A gNB-CU and Cell-A gNB-DU for the (1) initiation, (2) revocation and (3) reestablishment of OD SIB1 support for a NES cell.

Proposal 2-2: RAN3 to define the stage-2 procedure across Cell-A gNB-DU, Cell-A gNB-CU, NES-cell gNB-CU, and NES-cell gNB-DU for the status update of the UL WUS transmission by a Cell-A.

### 2.2.1	Initiation of OD SIB1 support by NES gNB

The initiation of the support for OD SIB1 operation is invoked by the NES gNB when it launches a new NES cell, or when the UL WUS configuration of a NES cell changes. The high-level procedure is shown in Figure 1.

Figure 1: NES gNB initiates OD SIB1 support for NES cells

The procedure has the following steps:

- The NES gNB-DU sends the UL WUS configuration and the corresponding NES cells to the NES gNB-CU with the (potentially implicit) indication that it wishes to support OD SIB1 operation mode.

- The NES gNB-CU selects potential Anchor gNBs for the transmission of the UL WUS configurations.

- The NES gNB-CU invokes the Xn UL WUS CONFIGURATION PROVISIONING procedure toward the Anchor gNB and includes the request for transmission of the UL WUS configuration, the UL WUS configuration itself, an UL WUS configuration ID, and the corresponding NES cells that use this UL WUS configuration.

- The Anchor gNB-CU selects Anchor cells based on this request. It may consider inter-cell overlap in the selection of Anchor cells, which it may have derived, e.g., from UE measurement reports.

- The Anchor gNB-CU sends to the Anchor gNB-DU a request for transmission of the UL WUS configuration, the UL WUS configuration itself, an UL WUS configuration ID, and the corresponding Anchors cells.

- The Anchor gNB-DU decides whether it can transmit the UL WUS configurations on the respective Anchor cells.

- The Anchor gNB-DU informs the gNB-CU with the Anchor cells that can transmit UL WUS configuration(s) together with the UL WUS configuration ID(s).

- The Anchor gNB CU sends the Anchor cells that transmit UL WUS configuration(s) together with the UL WUS configuration ID(s) to the NES gNB in Xn UL WUS CONFIGURATION PROVISIONING REPLY.

- Based on the reply from Anchor gNB(s), the NES gNB-CU decides whether the NES cell can operate in OD SIB1 mode.

- The NES gNB-CU informs the NES gNB-DU about the NES cells that are permitted to operate in OD SIB1 mode. When the NES gNB-CU has provided permission for a NES cell to operate in OD SIB1 mode, the NES gNB-DU can enter OD SIB1 operation mode.

### 2.2.2	Revocation of OD SIB1 support by NES gNB

This NES gNB-DU can decide that it wants to revoke the support of OD SIB1 operation. The NES gNB should inform the Anchor gNB about the revocation of OD SIB1 support for a NES cell so that the affected Anchor gNBs can suspend the transmission of the corresponding UL WUS configuration.

The NES gNB should include the list of NES cells with the revocation indication so that the Anchor gNB can perform an educated decision on whether transmission of the UL WUS configuration may still be necessary for other NES cells, which did not revoke OD SIB1 support.

The Anchor gNBs should provide feedback to the NES gNB on which of the Anchor cells have ceased transmission of the corresponding UL WUS configuration.

Since the end-to-end procedure for the revocation of OD SIB1 support involves an information flow from NES gNB-DU via NES gNB-CU and Anchor gNB-CU to Anchor gNB-DU and an reply flowing reverse direction, it can use the same F1 and Xn signaling procedures as used for the initiation of OD SIB1 support.

### 2.2.4	Reinstatement of OD SIB1 support by NES gNB

After the NES gNB revoked the support of OD SIB1 operation for a NES cell as discussed in section 2.2.3, it may decide to reinstate the OD SIB1 support for this NES cell. The reinstatement of OD SIB1 support may follow the same procedure as defined for initiation of OD SIB1 support by NES gNB.

### 2.2.3	Update of UL WUS configuration transmission status by Anchor gNB

The Anchor gNB-DU can decide that it wants to update the transmission status of an UL WUS configuration for one or multiple Anchor cells. This update may occur in response to the revocation indication of OD SIB1 support discussed in section 2.2.2, or for other reasons, e.g., Anchor cell overload conditions. The update of the UL WUS configuration transmission status should be conveyed to the NES gNB, so that the NES gNB can discontinue OD SIB1 operation, if needed. The high-level procedure is shown in Figure 2.

Figure 2: Anchor gNB updates transmission status for UL WUS configuration

The procedure has the following steps:

- The Anchor gNB-DU decides on the status update of UL WUS configuration transmissions for a set of Anchor cells.

- The Anchor gNB-DU informs the Anchor gNB-CU about the status update of the transmission status, and it includes the UL WUS config ID(s) and the associated anchors cells for each UL WUS configuration.

- The Anchor gNB-CU sends the update to the NES gNB-CU that have requested transmission of these UL WUS configurations before. It includes the UL WUS configuration IDs and the associated anchors cells for each UL WUS configuration.

- Based on the information received from Anchor gNB(s), the NES gNB-CU decides whether the NES cell can operate in OD SIB1 mode.

- The NES gNB-CU updates the NES gNB-DU about the NES cells that are permitted to operate in OD SIB1 mode, as needed.

## 2.3	Necessary Xn signaling enhancements

In the last meeting, RAN3 introduced the Xn UL WUS CONFIGURATION PROVISIONING procedure. This procedure can be used for initiation, revocation and reestablishment of OD SIB1 support by a NES cell.

Proposal 3-1: The Xn UL WUS CONFIGURATION PROVISIONING procedure to be used for the initiation, revocation and reestablishment of OD SIB1 support for a NES cell.

The Xn UL WUS CONFIGURATION PROVISIONING REQUEST needs to include for each UL WUS configuration to associated NES cells that use this configuration. This allows the Anchor gNB to select the Anchor cells that should transmit this configuration.

Proposal 3-2: The Xn UL WUS CONFIGURATION PROVISIONING REQUEST to include for each UL WUS configuration the associated NES cells so that the Anchor gNB can select the appropriate Anchor cells for UL WUS configuration transmission.

The Xn UL WUS CONFIGURATION PROVISIONING RESPONSE needs to include for each UL WUS configuration the Anchor cells that transmit the configuration so that the NES gNB can decide whether the NES cell has sufficient Anchor cell coverage to move to OD SIB1 operation mode.

Proposal 3-3: The Xn UL WUS CONFIGURATION PROVISIONING RESPONSE to include for each UL WUS configuration the Anchor cells that transmit this configuration so that the NES gNB can decide whether a NES cell can use OD SIB1 operation mode.

In the last meeting, RAN3 introduced the Xn UL WUS CONFIGURATION TRANSMISSION STATUS procedure. This procedure can be used by the Anchor gNB for a proactive update about the UL WUS configuration transmission status by an anchor cell.

Proposal 4-1: The Xn UL WUS CONFIGURATION TRANSMISSION STATUS procedure to be used by the Anchor gNB for a proactive update about the UL WUS configuration transmission status by an anchor cell.

The Xn UL WUS CONFIGURATION TRANSMISSION STATUS needs to include for each UL WUS configuration the Anchor cells for which the UL WUS configuration transmission status has changed, so that the NES gNB can decide whether the NES cell has sufficient Anchor cell coverage to move to OD SIB1 operation mode.

Proposal 4-2: The Xn UL WUS CONFIGURATION TRANSMISSION STATUS procedure to include for each UL WUS configuration the affected Anchor cells, whose transmission status has changed.

## 2.4	Necessary F1 signaling enhancements

As discussed in the end-to-end procedures above, the NES gNB-DU needs to inform the NES gNB-CU about the NES cells that can support OD SIB1 and those, for which NES support is revoked. This allows the NES gNB to ensure proper support by Anchor cells, and to ask Anchor gNBs to revoke UL WUS configuration transmission in case it is not needed anymore.

Proposal 5-1: The NES gNB-DU to inform the NES gNB-CU about the NES cells that can support OD SIB1 operation and about the NES cells, for whom OD-SIB1 support is revoked.

The NES gNB-CU should decide, which NES cell can conduct OD-SIB1 operation based on the Anchor cells that transmit the corresponding UL WUS configuration. It should then inform the NES gNB-DU about the NES cells that are permitted to operate in OD SIB1 mode.

Proposal 5-2: The NES gNB-CU to inform the NES gNB-DU about the NES cells, which are permitted to operate in OD SIB1 mode.

The Anchor gNB-CU should request the Anchor gNB-DU to conduct transmission of a UL WUS configurations for specific Anchor cells. It should also be able to request the Anchor gNB-DU to discontinue transmission of a UL WUS configuration  discontinuation for specific cells.

Proposal 6-1: The Anchor gNB-CU to request the Anchor-DU to conduct or discontinue transmission of WUS configurations for an Anchor cells.

The Anchor gNB-DU should inform the Anchor-CU whether the transmission status of UL WUS configuration has changed for an Anchor cell. This includes confirmation/rejection of UL WUS configuration transmissions requested by the Anchor gNB-CU before.

Proposal: 6-2: The Anchor gNB-DU to inform the Anchor-CU proactively or in response to a request, whether the transmission status of UL WUS configuration has changed for an Anchor cell.

Since any changes related to OD-SIB1 support and transmission of UL WUS configurations are expected to occur on semi-statically, all F1 signaling on NES gNB and Anchor gNB related to OD-SIB1 can reuse legacy procedures.

Proposal 7: All OD-SIB1 related signaling on NES gNB and Anchor gNB to reuse legacy procedures.

# 3	Conclusion

This contribution aimed to make further progress on the support for on-demand SIB1. The following observations and proposals have been made:

Terminology

Proposal 1-1: Introduce the following terminology for OD SIB: NES cell, Anchor cell, NES gNB and Anchor gNB.

Proposal 1-2: Liaise RAN1 and RAN2 on the agreed terminology.

Stage-2 procedures for OD SIB1

Proposal 2-1: RAN3 to define stage-2 procedures across NES gNB-DU, NES gNB-CU, Anchor gNB-CU and Anchor gNB-DU for the (1) initiation, (2) revocation and (3) reestablishment of OD SIB1 support for a NES cell.

Proposal 2-2: RAN3 to define an stage-2 procedure across Anchor gNB-DU, Anchor gNB-CU, NES gNB-CU, and NES gNB-DU for the status update of the UL WUS transmission by an Anchor cell.

Necessary Xn signaling enhancements

Proposal 3-1: The Xn UL WUS CONFIGURATION PROVISIONING procedure to be used for the initiation, revocation and reestablishment of OD SIB1 support for a NES cell.

Proposal 3-2: The Xn UL WUS CONFIGURATION PROVISIONING REQUEST to include for each UL WUS configuration the associated NES cells so that the Anchor gNB can select the appropriate Anchor cells for UL WUS configuration transmission.

Proposal 3-3: The Xn UL WUS CONFIGURATION PROVISIONING RESPONSE to include for each UL WUS configuration the Anchor cells that transmit this configuration so that the NES gNB can decide whether a NES cell can use OD SIB1 operation mode.

Proposal 4-1: The Xn UL WUS CONFIGURATION TRANSMISSION STATUS procedure to be used by the Anchor gNB for a proactive update about the UL WUS configuration transmission status by an anchor cell.

Proposal 4-2: The Xn UL WUS CONFIGURATION TRANSMISSION STATUS procedure to include for each UL WUS configuration the affected Anchor cells, whose transmission status has changed.

Necessary F1 signaling enhancements

Proposal 5-1: The NES gNB-DU to inform the NES gNB-CU about the NES cells that can support OD SIB1 operation and about the NES cells, for whom OD-SIB1 support is revoked.

Proposal 5-2: The NES gNB-CU to inform the NES gNB-DU about the NES cells, which are permitted to operate in OD SIB1 mode.

Proposal 6-1: The Anchor gNB-CU to request the Anchor-DU to conduct or discontinue transmission of WUS configurations for an Anchor cells.

Proposal: 6-2: The Anchor gNB-DU to inform the Anchor-CU proactively or in response to a request, whether the transmission status of UL WUS configuration has changed for an Anchor cell.

Proposal 7: All OD-SIB1 related signaling on NES gNB and Anchor gNB to reuse legacy procedures.

# Reference

[1]  Chair notes, RAN WG3 meeting #126, Orlando, Florida, November 2024