3GPP TSG-RAN WG3 Meeting #127		R3-250106

Athens, GR, 17th – 21st, Feb, 2025

Source:	CATT

Title:	(TP to BL CR for TS 38.413) Support of MBS broadcast service for NR NTN

Agenda Item:	14.2

Document for:	Discussion and Decision

# 1. Introduction

For MBS broadcast service topic, the NR NTN Phase 3 WID [1] content is as follows:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

The previous RAN3 meeting has agreed that geographical service area information can also be transmitted from CN with the same format as defined in Uu. In this contribution, we will continue to discuss the issues related to MBS broadcast service topic.

# 2. Discussion

The previous RAN3 meeting has agreed that the intended service area provided to the gNB can be Mapped Cell ID(s), TAI(s), or geographical service area information and send LS to SA2, RAN2 and CT4. RAN2 replied LS and the conclusions are as follows [2]:

1. The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. 2. The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. 3. The IntendedServiceAreaInfo is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

RAN2 has already discussed the name and encoding of geographical service area information, and reached the above conclusion. And RAN2 has already agreed that the IntendedServiceAreaInfo is considered as the IE name of the geographical area. The previous RAN2 meeting has already agreed that the intended service area can cover the area of more than one NTN cells (or portions thereof). The existing IE name .i.e.Small Area included in BLCR of 38.413 may cause ambiguity and confusion.

Observation 1: The intended MBS broadcast area may be a “Small Area”, may be not a “Small Area”, which may cover the area of more than one NTN cells (or portions thereof).

Therefore, it’s better to update the IE name Small Area to Intended Service Area, to align with RAN2 temorology, and avoid ambiguity and confusion.

Proposal 1: Update the IE name Small Area to Intended Service Area, to keep alignment with RAN2, and avoid ambiguity and confusion.

And in our opinion, RAN3 could keep the existing IE type i.e. OCTECT STRING and the encoding is in the same format as the IntendedServiceAreaInfo IE as defined in TS 38.331.

Proposal 2: For the IE encoding of the “Small Area”, keep the existing IE type i.e. OCTECT STRING, add semantics description with reference to the IntendedServiceAreaInfo IE as defined in TS 38.331.

As above, the corresponding TP for 38.413 BL CR is provided in appendix. RAN3 is requested to discuss and agree the TP.

Proposal 3: It is proposed to discuss and agree the corresponding TP for 38.413 BL CR in appendix.

# 3. Conclusion

According to the analysis in section 2, the following observations and proposals are provided:

Observation 1: The intended MBS broadcast area may be a “Small Area”, may be not a “Small Area”, which may cover the area of more than one NTN cells (or portions thereof).

Proposal 1: Update the IE name Small Area to Intended Service Area, to keep alignment with RAN2, and avoid ambiguity and confusion.

Proposal 2: For the IE encoding of the “Small Area”, keep the existing IE type i.e. OCTECT STRING, add semantics description with reference to the IntendedServiceAreaInfo IE as defined in TS 38.331.

Proposal 3: It is proposed to discuss and agree the corresponding TP for 38.413 BL CR in appendix.

# 4. Reference

- RP-240775 Rel-19 Moderator WID NR NTN phase 3RAN3_126_agenda_20241122_EOM1

- R3-250011 Reply LS on supporting MBS broadcast service for NR NTN

# Annex 1. TP for BLCR on 38.413 Support of MBS Broadcast service

-----------------Start of the Changes-------------------

8.17.1	Broadcast Session Setup

8.17.1.1	General

The purpose of the Broadcast Session Setup procedure is to request the NG-RAN node to setup MBS session resources for a broadcast MBS session. The procedure uses non-UE associated signalling.

8.17.1.2	Successful Operation

Figure 8.17.1.2-1: Broadcast Session Setup, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION SETUP REQUEST message to the NG-RAN node. If the NG-RAN node accepts all the MBS QoS flows in the MBS session at least in one of its cells, the NG-RAN node responds with the BROADCAST SESSION SETUP RESPONSE message.

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44], and TS 38.300 [8] for NTN operation.

If the MBS Session FSA ID List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account to determine cells/frequencies within the MBS service area to broadcast MBS session data as specified in TS 23.247 [44].

If the Supported UE Type List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the Associated Session ID IE is included in the BROADCAST SESSION SETUP REQUEST message the NG-RAN node shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.300 [8].

-----------------Next Change-------------------

8.17.2	Broadcast Session Modification

8.17.2.1	General

The purpose of the Broadcast Session Modification procedure is to request the NG-RAN node to update the MBS session resources or the area related to a previously established broadcast MBS session. The procedure uses non-UE associated signalling.

8.17.2.2	Successful Operation

Figure 8.17.2.2-1: Broadcast Session Modification, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUEST message to the NG-RAN node.

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area and send the BROADCAST SESSION MODIFICATION RESPONSE message. For NTN operation, the NG-RAN node shall, if supported, update the intended service area as specified in TS 38.300 [8].

If the MBS Session Modification Request Transfer IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall replace the previously provided information by the newly received one and update the MBS session resources and area as requested and send the BROADCAST SESSION MODIFICATION RESPONSE message.

If the Supported UE Type List IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the MBS NG-U Failure Indication IE is included in the BROADCAST SESSION MODIFICATION REQUEST message within the MBS Session Setup or Modification Request Transfer IE and set to "N3mb path failure", the NG-RAN node may provide new NG-U transport layer information to replace the failed one(s) or switch the data delivery to another 5GC as per the Broadcast MBS session restoration procedure for N3mb Path Failure specified in TS 23.527 [57].

-----------------Next Change-------------------

#### 9.3.1.209	MBS Service Area Information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |
| MBS Service Area Intended Service Area List |  | 0..<maxnoof IntendedServiceAreasforMBS> |  |  |
| >Intended Service Area | M |  | 9.3.1.x |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 8192. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 1024. |
| maxnoofIntendedServiceAreasIforMBS | Maximum no. of Intended Service areas allowed within one MBS Service Area. Value is 65536. |

-----------------Next Change-------------------

9.3.1.x	Intended Service Area

This IE provides information on the Intended Service Area for MBS Broadcast Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Intended Service Area | M |  | OCTET STRING | Encoded in the same format as the IntendedServiceAreaInfo IE as defined in TS 38.331 [18]. |

-----------------Next Change-------------------

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

UNCHANGED PART OMITTED

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-AssistanceInformation,

id-MBS-ServiceAreaIntendedServiceAreaList,

id-MBS-SessionTNLInfo5GC,

id-MBS-SupportIndicator,

id-MBSSessionFailedtoSetupList,

id-MBSSessionFailedtoSetuporModifyList,

id-MBSSessionSetupResponseList,

id-MBSSessionSetuporModifyResponseList,

id-MBSSessionToReleaseList,

id-MBSSessionSetupRequestList,

id-MBSSessionSetuporModifyRequestList,

UNCHANGED PART OMITTED

maxnoofCandidateRelayUEs,

maxnoofSuccessfulPSCellChangeReports,

maxnoofCellsTSS,

maxnoofPeriodicities,

maxnoofPartiallyAllowedS-NSSAIs,

maxnoofRSPPQoSFlows,

maxnoofIntendedServiceAreasforMBS

FROM NGAP-Constants

UNCHANGED PART OMITTED

MBS-ServiceArea ::= CHOICE {

locationindependent		MBS-ServiceAreaInformation,

locationdependent		MBS-ServiceAreaInformationList,

choice-Extensions		ProtocolIE-SingleContainer { {MBS-ServiceArea-ExtIEs} }

}

MBS-ServiceArea-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

MBS-ServiceAreaInformationList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-ServiceAreaInformationItem

MBS-ServiceAreaInformationItem ::= SEQUENCE {

mBS-AreaSessionID				MBS-AreaSessionID,

mBS-ServiceAreaInformation		MBS-ServiceAreaInformation,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformationItem-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCellList		MBS-ServiceAreaCellList																OPTIONAL,

mBS-ServiceAreaTAIList		MBS-ServiceAreaTAIList																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-ServiceAreaIntendedServiceAreaList		CRITICALITY ignore	EXTENSION MBS-ServiceAreaIntendedServiceAreaList	PRESENCE optional },

...

}

MBS-ServiceAreaCellList ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NR-CGI

MBS-ServiceAreaTAIList ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF TAI

MBS-ServiceAreaIntendedServiceAreaList ::= SEQUENCE (SIZE(1..maxnoofIntendedServiceAreasforMBS)) OF IntendedServiceArea

IntendedServiceArea ::= OCTET STRING

MBS-SessionID ::= SEQUENCE {

tMGI								TMGI,

nID								NID																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-SessionID-ExtIEs} } 		OPTIONAL,

...

}

MBS-SessionID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UNCHANGED PART OMITTED

-----------------Next Change-------------------

9.4.7	Constant Definitions

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

UNCHANGED PART OMITTED

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

maxnoofESNPNs								INTEGER ::= 15

maxnoofCandidateRelayUEs					INTEGER ::= 32

maxnoofSuccessfulPSCellChangeReports       	INTEGER ::= 64

maxnoofCellsTSS								INTEGER ::= 16384

maxnoofPeriodicities						INTEGER ::= 8

maxnoofCAGforMDT							INTEGER ::= 256

maxnoofMDTSNPNs								INTEGER ::= 16

maxnoofPartiallyAllowedS-NSSAIs				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofIntendedServiceAreasforMBS						INTEGER ::= 65536

UNCHANGED PART OMITTED

id-PDUSessionListMTCommHReq								ProtocolIE-ID ::= 431

id-MaximumDataBurstVolume 								ProtocolIE-ID ::= 432

id-MN-only-MDT-collection								ProtocolIE-ID ::= 433

id-MBS-NGUFailureIndication								ProtocolIE-ID ::= 434

id-UserPlaneFailureIndication							ProtocolIE-ID ::= 435

id-UserPlaneFailureIndicationReport						ProtocolIE-ID ::= 436

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-MBS-ServiceAreaIntendedServiceAreaList							ProtocolIE-ID ::= x

-----------------End of the Change-------------------