3GPP TSG-RAN WG3 Meeting #127		R3-250107

Athens, GR, 17th – 21st, Feb, 2025

Source:	CATT

Title:	(TP to NTN BL CRs) Support of regenerative payload

Agenda Item:	14.3

Document for:	Discussion and Decision

- Introduction

In Rel-19 NR NTN WID [1], Regenerative Payload with full gNB on board is to be supported, detail objectives could be found below:

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

And in last RAN3 meeting, we further discussed the relevant issues on support of regenerative payload for NR NTN. According to the discussion, some agreements are achieved [2], as below:

RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

In this contribution, we will continue discuss the potential issues on regenerative payload for NR NTN, our views will be provided accordingly.

- Discussion

## 2.1.	NG connection management

The previous RAN3 meeting has already agreed that NG Removal/Setup can be used for Regenerative Payload NG interface management. And there are different views on whether to introduce a suspend/resume mechanism.

The above NG suspend/resume mechanism is actually the TNLA suspend/resume. When the on-board gNB coverage leaves the AMF service area, the NG connection could be suspended, by indicating the AMF that the current NG connection is temporarily unavailable, and the logical NG interface still keeps. When the on-board gNB come back to the AMF service area, the NG connection could be resumed quickly. Due to the mobility of on board gNB, the TNL over the NG interface will frequently disconnect and re-connect. Solution#2 in TS 23.700-29 mentioned that:

The solution proposes to suspend the TNL between the RAN node and the CN if the RAN node attempts to disconnect the backhaul connection towards the AMF/MME in a PLMN. When the RAN node decides that it cannot serve the current PLMN any longer, it should indicate the AMF/MME to suspend the N2/S1-AP connection and store the context for resuming the N2/S1-AP connection in the future. Both the RAN node and the AMF/MME store the RAN node context information for resuming the N2/S1-AP connection. The RAN node and the 5GC should further adjust the heartbeat timer of the SCTP over N2/S1-AP interface to avoid heartbeat time out.

In this solution, on board gNB needs to store the non-UE context information of the NGAP. While some companies express concerns about limited storage space on satellites, we believe that the above non-UE context information is only the basic context of NGAP and the amount of data is not large. Similarly, in TN system, a gNB can establish NGAP connections with all AMFs in one or more AMF sets and store the corresponding context. Therefore, we do not see any issue for on board gNB to store multiple NGAPs contexts for NGAP suspend/resume.

Observation 1: No big issue for on board gNB to store multiple NGAP contexts for NGAP suspend/resume.

NG suspend and resume solution can provides faster NG connection establishment and reduce the frequent TNL disconnection and re-connection. Therefore, we think that both NG Removal/Setup and NG suspend/resume can be supported, and the two mechanisms can be used flexibly or can be used in combination. For example, if on board gNB may not return to the service area of an AMF for a long time in the future, it can choose NG Removal procedure. If on board gNB will return to the service area of an AMF in a shorter period of time, it can choose NG suspend procedure.

On support of NG Suspend/Resume, it’s possible to reuse the RAN CONFIGURATION UPDATE procedure with necessary enhancement, e.g. add the suspend/resume indicator. For example, if the on-board gNB leaves the service area of an AMF and decides to suspend the NG interface, the gNB stores the corresponding NGAP context and sends RAN CONFIGURATION UPDATE message with a NG suspend indicator. Upon receiving the above message, AMF stores the corresponding NG context information and indicates the transport layer the NGAP is suspended. With the indicator, the lower layer could adjust the heartbeat timer of SCTP by itself.

And if the on-board gNB is reconnecting the feeder link towards the AMF, the gNB may decide to resume the NG connection towards some AMF(s), then it sends RAN CONFIGURATION UPDATE message with a NG resume indicator towards the AMF(s). Upon receiving the above message, the AMF may transfer the suspend indicator to the lower layer, the lower layer updates the heartbeat timer to a normal level.

As above, the corresponding stage 3 TP for 38.413 BL CR is provided in section 7. RAN3 is requested to discuss and agree the TP.

Proposal 1: Besides NG Removal/Setup, it is proposed to support NG suspend/resume for NR NTN NG management.

Proposal 2: RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators. Corresponding TP for 38.413 BL CR is provided in appendix 7.

  - 2.2.	Feeder link switch

The RAN3 meeting has already agreed that the current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch. And in the previous RAN3 meeting, a few companies mentioned how to solve the packet loss problem in hard feeder link switch scenarios. As for DL NG-U packet, when the old feeder link is disconnected, the AMF informs the corresponding SMF(s) and the SMF(s) controls the UPF(s) to buffer the DL data. When the on-board gNB performs feeder link switchover to the new NTN Gateway, NG-U channel may also changes. As for NG-U, it is per user device and per PDU session. The on-board gNB should notify the AMF about the DL NG-U TNL Information. The on-board gNB could use Path Switch Request or PDU Session Resource Modify Indication.

As above, the corresponding stage 2 TP for 38.300 BL CR is provided in section 5. RAN3 is requested to discuss and agree the TP.

Proposal 3: When the on-board gNB performs feeder link switchover to the new NTN Gateway, gNB could notifies the AMF about the new DL NG-U TNL Information through Path Switch Request or PDU Session Resource Modify Indication message. Corresponding TP for 38.300 BL CR is provided in appendix 5.

  - 2.4.	Support of RRC_INACTIVE UE

The last RAN3 meeting has discussed how to support RRC_INACTIVE UE in NTN regenerative payload and the candidate solutions are as follows：

- Option 1: UE context is forwarded to the next satellites

- Option 2: UE context is kept on a fixed node (on-ground gNB or AMF)

- Option 3: UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

For the option 1, the time for the next satellite to serve the same geographic area may be within a few minutes, such as LEO satellite, so the transfer of UE context is very frequent. The above operations are very complicated and this solution defeats the purpose of RRC_INACTIVE state. In addition, the above solution may have security risk. The anchor gNB transfers the UE context with AS security context to the receiving gNB. When the INACTIVE UE resuming connection, the receiving gNB executes the UE’s verification using the security key Krrcint. Both receiving gNB and anchor gNB use the same security key, but normally the key of a gNB cannot be transferred to another gNB. Therefore, the above solution may have security risk, if agreed, SA5 needs to be confirmed.

Observation 2: For the option 1, the transfer of UE context may be very frequent and this solution defeats the purpose of RRC_INACTIVE state.

For the option 2, UE context is kept on a fixed node (on-ground gNB or AMF). If UE context is kept on on-ground gNB, same security issues as Option1. If UE context is kept on AMF, there may still be security issues.The security key has been used in the last serving gNB. When the UE resumes in the new gNB, the new gNB requests the UE context from CN, and the AMF needs to verify the UE context, which is also inconsistent with the security policy.

Observation 3: Both option 1and option 2 may have security risk, if agreed, SA5 needs to be confirmed.

For the option 3, when UE resumes, the current serving gNB initiate NG-based UE context retrieve procedure to retrieve the UE context from last serving gNB. But in current procedure, when the on-board gNB coverage leaves the AMF service area, the NG connection may be removed and the receiving gNB may not be able to fetch UE context from the last serving gNB based on NG signaling. So the ablove solution may need to maintain NG connection. For example, on-board gNB needs to maintain the NG connection to the global AMFs and store multiple UE contexts of RRC_INACTIVE UE for NG-based UE context retrieve, which may bring big challenge on storage space of the satellites.

Observation 4: Solution 3 requires an on board gNB to maintain the NG interfaces with the AMFs, even if it leaves the AMF service area.

Observation 5: Solution 3 requires an on board gNB to store the UE context for the RRC_INACTIVE UEs, which may bring a significant challenge to the satellite on the stoarage capability.

In our opinion, if there is no problem storing UE context, then there is no problem storing non UE context. Therefore we think NG suspend/resume could be considered to provide a faster NG connection to support NG-based UE context retrieve procedure.

Proposal 4: We are open to the option3 and we think NG suspend/resume could be considered to provide a faster NG connection to support NG-based UE context retrieve procedure.

  - 2.5.	Reply LS to SA2 and SA5 on OAM requirements to support regenerative

In pevious meeting, RAN3 has already send LS to SA2 and SA5 on OAM requirement for supported TAIs. And SA2 and SA5 replied LS about the above issue. The LS of SA2 needs to be confirmed with RAN3 as follows[4]:

SA2 also understands the NG Removal procedure is defined to remove interface between a gNB and an AMF. SA2 believes after receiving a NG Removal message, the AMF will release the information of supported TAIs and it is the gNB to determine the time to initiate the NG Removal procedure based on the AMFs serving area (e.g., the serving PLMN allowed operation area). Therefore the number of TAIs and related updates will be bounded by those which can be covered by a satellite while using a NTN gateway. SA2 would like confirm these understanding with RAN3. - SA2 discussed whether the OAM solution would have impacts to the existing procedures related to paging and sequencing of OAM updates and RAN3 updates towards the AMF, e.g. in NGAP Setup/configuration update etc. However, it was unclear to SA2 how the OAM solution would work based on the description in the incoming LS to be able to determine whether there are impacts. - The frequency of updates by OAM was not clear to SA2 and whether the information provided to the AMF is a timetable of which TAIs the gNB will support at what time, or whether the updates are provided frequently as the satellite moves updating the AMF each time a change occurs. SA2 would like to confirm whether RAN3 and SA5 are aligned on the frequency and contents of the OAM updates to the AMF. Some companies in SA2 have concerns with the solutions agreed by RAN3, which changes the functional split between RAN and the core network.

SA2 mentioned that the number of TAIs and related updates will be bounded by those which can be covered by a satellite while using a NTN gateway in LS. For transparent payload, gNB is connected to the gateway on the ground and the supported TAI list of gNB connected to the gateway could be fixed. So the above understanding of SA2 is accurate for transparent payload.

But for regenerative payload, the movement of the NGSO satellites may cause the coverage of the on board gNB change over time. Accordingly the supported TAIs list also changes over time. Existing procedures .i.e. NGAP Setup/ RAN CONFIGURATION UPDATE can be reused to provide the supported TAI list of the on-board gNB to the AMF, but it will cause a large number of NG interface signaling interactions, which is not efficient. Therefore RAN3 prefers OAM based solution. For the sequencing of OAM and legacy procedure, RAN3 understands that if OAM based solution is adopted, NG-RAN will not provide the supported TAI list over the RAN CONFIGURATION UPDATE. If Satellite Operator(s) would not like to use OAM based solution, NG-RAN could also be configured to report the supported TAI list as legacy. To be simple, if both OAM based solution and Signalling based solution are adopted, AMF could adopt the latest supported TAI list provided by OAM or legacy procedure.

For the frequency of updates by OAM, this relies on the implementation of OAM, RAN3 believes that the design of OAM could avoid the frequent configuration, e.g. based on time window. And the LS of SA2 is as follows [5]:

3GPP SA5 thanks RAN3 for the incoming LS on OAM requirements to support regenerative payload. SA5 would like to inform RAN3 that SA5 Rel 19 study “SP-231733 New SID on Management Aspects of NTN Phase 2” has investigated and agreed the potential solutions of providing the information of supported TAIs for NTN gNB to AMF in TR 28.874. Further, SA5 will consider the solution in normative work in Rel-19.

The potential solutions in TS 23.700-29 are as follows:

- Potential solution #<1>: Timing window based TAList update

- Potential solution #<2>: Pre-configuration based on single time window

The design of OAM update frequency and content depends on SA5. As above, the corresponding draft reply LS is provided in appendix 6. RAN3 is requested to discuss and agree it.

Proposal 5: The detail design of OAM is pending to SA5 and it is proposed to discuss the above issues and agree the related draft reply LS in appendix 6.

  - 2.6.	Location based HO over XnAP/NGAP

R18 has already enhanced the Xn interface to support Time based CHO. Handover with Time-based trigger condition and Location-based trigger condition are specified in Stage-2 TS 38.300. In the previous RAN3 meeting, some companies proposed to enhance location based HO. For inter-gNB mobility (between on board gNBs), it is proposed that RAN3 consider whether and how to better support location based HO via the Xn interface. For example, source NG-RAN can transfer the coarse UE location information or the condition of location based CHO to target NG-RAN in the Handover Request. It is not necessary to transfer the above information to time window, and the above information can be more clearly indicated. Thus the target NG-RAN can decide when to reserve resources based on this information.

We can focus on XnAP first, and adopt it to NG if needed. As above, the corresponding stage 3 TP for 38.423 BL CR is provided in section 8. RAN3 is requested to discuss and agree the TP.

Proposal 6: For location based CHO, it is proposed to transfer some information from the source NG-RAN node towards the target NG-RAN node, e.g. the coarse location of the UE, for better resource handling. Discuss and agree the TP for BLCR on TS 38.423 in appendix 8

- Conclusion

Based on the discussion in section 2, we have the following observations and proposals:

NG connection management

Observation 1: No big issue for on board gNB to store multiple NGAP contexts for NGAP suspend/resume.

Proposal 1: Besides NG Removal/Setup, it is proposed to support NG suspend/resume for NR NTN NG management.

Proposal 2: RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators. Corresponding TP for 38.413 BL CR is provided in appendix 7.

Feeder link switch

Proposal 3: When the on-board gNB performs feeder link switchover to the new NTN Gateway, gNB could notifies the AMF about the new DL NG-U TNL Information through Path Switch Request or PDU Session Resource Modify Indication message. Corresponding TP for 38.300 BL CR is provided in appendix 5.

Support of RRC_INACTIVE UE

Observation 2: For the option 1, the transfer of UE context may be very frequent and this solution defeats the purpose of RRC_INACTIVE state.

Observation 3: Both option 1and option 2 may have security risk, if agreed, SA5 needs to be confirmed.

Observation 4: Solution 3 requires an on board gNB to maintain the NG interfaces with the AMFs, even if it leaves the AMF service area.

Observation 5: Solution 3 requires an on board gNB to store the UE context for the RRC_INACTIVE UEs, which may bring a significant challenge to the satellite on the stoarage capability.

Proposal 4: We are open to the option3 and we think NG suspend/resume could be considered to provides a faster NG connection to support NG-based UE context retrieve procedure.

Reply LS to SA2 and SA5

Proposal 5: The detail design of OAM is pending to SA5 and it is proposed to discuss the above issues and agree the related draft reply LS in appendix 6.

Location based HO over XnAP/NGAP

Proposal 6: For location based CHO, it is proposed to transfer some information from the source NG-RAN node towards the target NG-RAN node, e.g. the coarse location of the UE, for better resource handling. Discuss and agree the TP for BLCR on TS 38.423 in appendix 8

- Reference

- RP-240775 Rel-19 Moderator WID NR NTN phase 3

- RAN3_126_agenda_20241122_EOM1

- TR 23.700-47 Study on integration of satellite components in the 5G architecture

- R3-250023 LS on reply to LS on OAM requirements to support regenerative payload

- R3-250030 LS reply on OAM requirements to support regenerative payload

- TP for BLCR on 38.300 Support of feeder link switch

START OF CHANGES

### 16.14.4	Switchover

#### 16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in transparent or regenerative NTN payloads.

#### 16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links.

#### 16.14.4.3	Procedures

The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switch over is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

In case of feederlink switch，on board gNB may provide the updated DL GTP-U tunnels for the served UEs to AMF via Path Switch Request or PDU Session Resource Modify Indication message.

END OF CHANGES

- Draft Reply LS to SA2 and SA5(S2-2413030/ R3-250023)

Title:	[DRAFT] Reply LS on OAM requirements to support regenerative payload transport links

Response to:	LS (S2-2413030/ R3-250023) on < LS on reply to LS on OAM requirements to support regenerative payload>

Release:	Rel-19

Work Item:	NR_NTN_enh

Source:	CATT [to become RAN3]

To:	SA2, SA5

Cc:

Contact Person:	Jiancheng Sun

sunjiancheng@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 thanks SA2 and SA5 for the reply LS on OAM requirements to support regenerative. The understandings of RAN3 are as follows:

- For regenerative payload, the movement of the NGSO satellites may cause the coverage of the on board gNB change over time. Accordingly, the supported TAIs list of the on board gNB also changes over time. Existing procedures .i.e. NGAP Setup/ RAN CONFIGURATION UPDATE can be reused to provide the supported TAI list of the on-board gNB to the AMF, but frequent interactions over NG interface would be needed, which may cause the signalling burden. Therefore, RAN3 decided to use OAM based solution.

- For the sequencing of OAM and legacy procedure, RAN3 understands that if OAM based solution is adopted, NG-RAN will not provide the supported TAI list over the RAN CONFIGURATION UPDATE. If Satellite Operator(s) would not like to use OAM based solution, NG-RAN could also be configured to report the supported TAI list as legacy. To be simple, if both OAM based solution and Signalling based solution are adopted, AMF could adopt the latest supported TAI list provided by OAM or legacy procedure.

- For the frequency of updates by OAM, this relies on the implementation of OAM, RAN3 believes that the design of OAM could avoid the frequent configuration, e.g. based on time window. The detail design of OAM is pending to SA5.

2. Actions:

To SA2 and SA5group:

ACTION: 	RAN3 kindly asks SA2 and SA5 to take the above information into account

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #127bis			07-11 Apr 2024				China , CN

TSG-RAN3 Meeting #128	19-23 May 2025			Malta , MT

- TP for BLCR on 38.413 Support of NG Suspend/Resume

-----------------Start of the Changes-------------------

### 8.7.2	RAN Configuration Update

#### 8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling.

#### 8.7.2.2	Successful Operation

Figure 8.7.2.2-1: RAN configuration update: successful operation

The NG-RAN node initiates the procedure by sending a RAN CONFIGURATION UPDATE message to the AMF including an appropriate set of updated configuration data that it has just taken into operational use. The AMF responds with a RAN CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the RAN CONFIGURATION UPDATE message, the AMF shall interpret that the corresponding configuration data is not changed and shall continue to operate the NG-C interface with the existing related configuration data.

If the Supported TA List IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite the whole list of supported TAs and the corresponding list of supported slices for each TA, and use them for subsequent registration area management of the UE.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the RAN CONFIGURATION UPDATE message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the Global RAN Node ID IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall associate the TNLA to the NG-C interface instance using the Global RAN Node ID.

If the RAN CONFIGURATION UPDATE message includes the NG-RAN TNL Association to Remove List IE, the AMF shall, if supported, initiate removal of the TNL association(s) indicated by NG-RAN TNL endpoint(s) and AMF TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is present, or the TNL association(s) indicated by NG-RAN TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is absent:

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the NG-RAN TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the NG-RAN TNL endpoints correspond to all NG-RAN TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

-    if the received TNL Association Transport Layer Address at AMF IE includes the Port Number IE, the AMF TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the AMF TNL endpoints correspond to all AMF TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If the RAN CONFIGURATION UPDATE message includes the RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node. If the RAN CONFIGURATION UPDATE message includes the Extended RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

If the NB-IoT Default Paging DRX IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite any previously stored NB-IoT default paging DRX value for the NG-RAN node.

If the RAT Information IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall handle this information as specified in TS 23.502 [10].

If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the NG TNLA Indicator IE is included in the RAN CONFIGURATION UPDATE message is set to "suspend", the AMF shall use this information for suspending TNL of NG interface.

If the NG TNLA Indicator IE is included in the RAN CONFIGURATION UPDATE message is set to "resume", the AMF shall use this information for resuming TNL of NG interface.

Editor’s Note: The procedure texts could be refined.

-----------------Next Change-------------------

### 8.7.x	NG Removal

#### 8.7.x.1	General

The purpose of the NG Removal procedure is to remove the interface instance between the NG-RAN node and the AMF in a controlled manner. If successful, this procedure erases any existing application level configuration data in the two nodes.

The procedure uses non-UE associated signaling.

#### 8.7.x.2	Successful Operation

Figure 8.7.x.2-1: NG Removal, successful operation

The NG-RAN node initiates the procedure by sending the NG REMOVAL REQUEST message to the AMF. Upon reception of the NG REMOVAL REQUEST message, the AMF shall reply with the NG REMOVAL RESPONSE message. After receiving the NG REMOVAL RESPONSE message, the NG-RAN node shall initiate removal of the TNL association towards the AMF and may remove all resources associated with that interface instance. The AMF may then remove all resources associated with the NG-RAN node.

#### 8.7.x.3	Unsuccessful Operation

Figure 8.7.x.3-1: NG Removal, unsuccessful operation

If the AMF cannot accept to remove the interface instance with NG-RAN node, it shall respond with an NG REMOVAL FAILURE message with an appropriate cause value.

#### 8.7.x.4	Abnormal Conditions

Void.

-----------------Next Change-------------------

8.17.1	Broadcast Session Setup

8.17.1.1	General

The purpose of the Broadcast Session Setup procedure is to request the NG-RAN node to setup MBS session resources for a broadcast MBS session. The procedure uses non-UE associated signalling.

8.17.1.2	Successful Operation

Figure 8.17.1.2-1: Broadcast Session Setup, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION SETUP REQUEST message to the NG-RAN node. If the NG-RAN node accepts all the MBS QoS flows in the MBS session at least in one of its cells, the NG-RAN node responds with the BROADCAST SESSION SETUP RESPONSE message.

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44], and TS 38.300 [8] for NTN operation.

If the MBS Session FSA ID List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account to determine cells/frequencies within the MBS service area to broadcast MBS session data as specified in TS 23.247 [44].

If the Supported UE Type List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the Associated Session ID IE is included in the BROADCAST SESSION SETUP REQUEST message the NG-RAN node shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.300 [8].

-----------------Next Change-------------------

8.17.2	Broadcast Session Modification

8.17.2.1	General

The purpose of the Broadcast Session Modification procedure is to request the NG-RAN node to update the MBS session resources or the area related to a previously established broadcast MBS session. The procedure uses non-UE associated signalling.

8.17.2.2	Successful Operation

Figure 8.17.2.2-1: Broadcast Session Modification, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUEST message to the NG-RAN node.

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area and send the BROADCAST SESSION MODIFICATION RESPONSE message. For NTN operation, the NG-RAN node shall, if supported, update the intended service area as specified in TS 38.300 [8].

If the MBS Session Modification Request Transfer IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall replace the previously provided information by the newly received one and update the MBS session resources and area as requested and send the BROADCAST SESSION MODIFICATION RESPONSE message.

If the Supported UE Type List IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the MBS NG-U Failure Indication IE is included in the BROADCAST SESSION MODIFICATION REQUEST message within the MBS Session Setup or Modification Request Transfer IE and set to "N3mb path failure", the NG-RAN node may provide new NG-U transport layer information to replace the failed one(s) or switch the data delivery to another 5GC as per the Broadcast MBS session restoration procedure for N3mb Path Failure specified in TS 23.527 [57].

-----------------Next Change-------------------

#### 9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| NG TNLA Indicator | O |  | ENUMERATED (suspend, resume, …) |  | YES | ignore |

Editor’s Note: The IE naming and content are FFS.

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |

-----------------Next Change-------------------

#### 9.2.6.x1	NG REMOVAL REQUEST

This message is sent by the NG-RAN node to initiate the removal of the interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |

#### 9.2.6.x2	NG REMOVAL RESPONSE

This message is sent by the AMF to acknowledge the initiation of removal of the interface instance.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.6.x3	NG REMOVAL FAILURE

This message is sent by the AMF to indicate that removing the interface instance cannot be accepted.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

-----------------Next Change-------------------

#### 9.3.1.209	MBS Service Area Information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |
| MBS Service Area Small Area List |  | 0..<maxnoofSmallAreasforMBS> |  |  |
| >Small Area | M |  | 9.3.1.x |  |

Editor’s Note: The IE name and encoding are pending RAN2 discussion.

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 8192. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 1024. |
| maxnoofSmallAreasIforMBS | Maximum no. of small areas allowed within one MBS Service Area. Value is 65536. |

-----------------Next Change-------------------

9.3.1.x	Small Area

This IE provides information on the Small Area for MBS Broadcast Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Small Area | M |  | OCTET STRING | Pending RAN2 discussion |

Editor’s Note: The IE name are pending RAN2 discussion.

-----------------Next Change-------------------

9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NGAP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM NGAP-CommonDataTypes

UNCHANGED PART OMITTED

NASNonDeliveryIndication,

NGReset,

NGResetAcknowledge,

NGRemovalFailure,

NGRemovalRequest,

NGRemovalResponse,

NGSetupFailure,

NGSetupRequest,

NGSetupResponse,

UNCHANGED PART OMITTED

id-NASNonDeliveryIndication,

id-NGReset,

id-NGRemoval,

id-NGSetup,

id-OverloadStart,

id-OverloadStop,

id-Paging,

id-PathSwitchRequest,

UNCHANGED PART OMITTED

NGAP-ELEMENTARY-PROCEDURES-CLASS-1 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFConfigurationUpdate			|

broadcastSessionModification	|

broadcastSessionRelease			|

broadcastSessionSetup			|

broadcastSessionTransport					|

distributionSetup				|

distributionRelease				|

handoverCancel					|

handoverPreparation				|

handoverResourceAllocation		|

initialContextSetup				|

mTCommunicationHandling			|

multicastSessionActivation					|

multicastSessionDeactivation				|

multicastSessionUpdate			|

nGReset							|

nGSetup							|

pathSwitchRequest				|

pDUSessionResourceModify		|

pDUSessionResourceModifyIndication			|

pDUSessionResourceRelease		|

pDUSessionResourceSetup			|

pWSCancel						|

rANConfigurationUpdate			|

timingSynchronisationStatus		|

uEContextModification			|

uEContextRelease				|

uEContextResume								|

uEContextSuspend							|

uERadioCapabilityCheck			|

uERadioCapabilityIDMapping		|

writeReplaceWarning							|

nGRemoval									,

...

}

UNCHANGED PART OMITTED

nGReset NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGReset

SUCCESSFUL OUTCOME		NGResetAcknowledge

PROCEDURE CODE			id-NGReset

CRITICALITY				reject

}

nGSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGSetupRequest

SUCCESSFUL OUTCOME		NGSetupResponse

UNSUCCESSFUL OUTCOME	NGSetupFailure

PROCEDURE CODE			id-NGSetup

CRITICALITY				reject

}

nGRemoval NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGRemovalRequest

SUCCESSFUL OUTCOME		NGRemovalResponse

UNSUCCESSFUL OUTCOME	NGRemovalFailure

PROCEDURE CODE			id-NGRemoval

CRITICALITY				reject

}

overloadStart NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		OverloadStart

PROCEDURE CODE			id-OverloadStart

CRITICALITY				ignore

}

UNCHANGED PART OMITTED

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

UNCHANGED PART OMITTED

NGRAN-CGI,

NGRAN-TNLAssociationToRemoveList,

NGRANTraceID,

NG-TNLA-Indicator,

NID,

NoPDUSessionIndication,

NotifySourceNGRANNode,

UNCHANGED PART OMITTED 	id-NewSecurityContextInd,

id-NGAP-Message,

id-NGRAN-CGI,

id-NGRAN-TNLAssociationToRemoveList,

id-NGRANTraceID,

id-NG-TNLA-Indicator,

id-NoPDUSessionIndication,

UNCHANGED PART OMITTED

-- **************************************************************

--

#### -- INTERFACE MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- NG Removal Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- NG REMOVAL REQUEST

--

-- **************************************************************

NGRemovalRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGRemovalRequestIEs} },

...

}

NGRemovalRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalRANNodeID				CRITICALITY reject	TYPE GlobalRANNodeID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- NG REMOVAL RESPONSE

--

-- **************************************************************

NGRemovalResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGRemovalResponseIEs} },

...

}

NGRemovalResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMFName 					CRITICALITY reject	TYPE AMFName					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- NG REMOVAL FAILURE

--

-- **************************************************************

NGRemovalFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGRemovalFailureIEs} },

...

}

NGRemovalFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

UNCHANGED PART OMITTED

-- **************************************************************

--

##### -- RAN Configuration Update Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- RAN CONFIGURATION UPDATE

--

-- **************************************************************

RANConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RANConfigurationUpdateIEs} },

...

}

RANConfigurationUpdateIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RANNodeName								CRITICALITY ignore	TYPE RANNodeName									PRESENCE optional	}|

{ ID id-SupportedTAList							CRITICALITY reject	TYPE SupportedTAList								PRESENCE optional	}|

{ ID id-DefaultPagingDRX						CRITICALITY ignore	TYPE PagingDRX										PRESENCE optional	}|

{ ID id-GlobalRANNodeID							CRITICALITY ignore	TYPE GlobalRANNodeID								PRESENCE optional	}|

{ ID id-NGRAN-TNLAssociationToRemoveList		CRITICALITY reject	TYPE NGRAN-TNLAssociationToRemoveList			PRESENCE optional	}|

{ ID id-NB-IoT-DefaultPagingDRX					CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX						PRESENCE optional	}|

{ ID id-Extended-RANNodeName					CRITICALITY ignore	TYPE Extended-RANNodeName							PRESENCE optional	}|

{ ID id-NG-TNLA-Indicator						CRITICALITY ignore	TYPE NG-TNLA-Indicator							PRESENCE optional	},

...

}

-----------------Next Change-------------------

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

UNCHANGED PART OMITTED

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-AssistanceInformation,

id-MBS-ServiceAreaSmallAreaList,

id-MBS-SessionTNLInfo5GC,

id-MBS-SupportIndicator,

id-MBSSessionFailedtoSetupList,

id-MBSSessionFailedtoSetuporModifyList,

id-MBSSessionSetupResponseList,

id-MBSSessionSetuporModifyResponseList,

id-MBSSessionToReleaseList,

id-MBSSessionSetupRequestList,

id-MBSSessionSetuporModifyRequestList,

UNCHANGED PART OMITTED

maxnoofCandidateRelayUEs,

maxnoofSuccessfulPSCellChangeReports,

maxnoofCellsTSS,

maxnoofPeriodicities,

maxnoofPartiallyAllowedS-NSSAIs,

maxnoofRSPPQoSFlows,

maxnoofSmallAreasforMBS

FROM NGAP-Constants

UNCHANGED PART OMITTED

MBS-ServiceArea ::= CHOICE {

locationindependent		MBS-ServiceAreaInformation,

locationdependent		MBS-ServiceAreaInformationList,

choice-Extensions		ProtocolIE-SingleContainer { {MBS-ServiceArea-ExtIEs} }

}

MBS-ServiceArea-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

MBS-ServiceAreaInformationList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-ServiceAreaInformationItem

MBS-ServiceAreaInformationItem ::= SEQUENCE {

mBS-AreaSessionID				MBS-AreaSessionID,

mBS-ServiceAreaInformation		MBS-ServiceAreaInformation,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformationItem-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCellList		MBS-ServiceAreaCellList																OPTIONAL,

mBS-ServiceAreaTAIList		MBS-ServiceAreaTAIList																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-ServiceAreaSmallAreaList		CRITICALITY ignore	EXTENSION MBS-ServiceAreaSmallAreaList	PRESENCE optional },

...

}

MBS-ServiceAreaCellList ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NR-CGI

MBS-ServiceAreaTAIList ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF TAI

MBS-ServiceAreaSmallAreaList ::= SEQUENCE (SIZE(1..maxnoofSmallAreasforMBS)) OF SmallArea

SmallArea ::= OCTET STRING

-- FFS IE name pending RAN2 discussion

MBS-SessionID ::= SEQUENCE {

tMGI								TMGI,

nID								NID																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-SessionID-ExtIEs} } 		OPTIONAL,

...

}

MBS-SessionID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UNCHANGED PART OMITTED

NG-TNLA-Indicator ::= ENUMERATED {

suspend,

resume,

...

}

UNCHANGED PART OMITTED

-----------------Next Change-------------------

9.4.7	Constant Definitions

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

UNCHANGED PART OMITTED

id-ConnectionEstablishmentIndication		ProcedureCode ::= 65

id-BroadcastSessionModification				ProcedureCode ::= 66

id-BroadcastSessionRelease					ProcedureCode ::= 67

id-BroadcastSessionSetup					ProcedureCode ::= 68

id-DistributionSetup 						ProcedureCode ::= 69

id-DistributionRelease 						ProcedureCode ::= 70

id-MulticastSessionActivation 				ProcedureCode ::= 71

id-MulticastSessionDeactivation 			ProcedureCode ::= 72

id-MulticastSessionUpdate 					ProcedureCode ::= 73

id-MulticastGroupPaging						ProcedureCode ::= 74

id-BroadcastSessionReleaseRequired			ProcedureCode ::= 75

id-TimingSynchronisationStatus				ProcedureCode ::= 76

id-TimingSynchronisationStatusReport		ProcedureCode ::= 77

id-MTCommunicationHandling					ProcedureCode ::= 78

id-RANPagingRequest							ProcedureCode ::= 79

id-BroadcastSessionTransport				ProcedureCode ::= 80

id-NGRemoval								ProcedureCode ::= x

UNCHANGED PART OMITTED

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

maxnoofESNPNs								INTEGER ::= 15

maxnoofCandidateRelayUEs					INTEGER ::= 32

maxnoofSuccessfulPSCellChangeReports       	INTEGER ::= 64

maxnoofCellsTSS								INTEGER ::= 16384

maxnoofPeriodicities						INTEGER ::= 8

maxnoofCAGforMDT							INTEGER ::= 256

maxnoofMDTSNPNs								INTEGER ::= 16

maxnoofPartiallyAllowedS-NSSAIs				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofSmallAreasforMBS						INTEGER ::= 65536

UNCHANGED PART OMITTED

id-PDUSessionListMTCommHReq								ProtocolIE-ID ::= 431

id-MaximumDataBurstVolume 								ProtocolIE-ID ::= 432

id-MN-only-MDT-collection								ProtocolIE-ID ::= 433

id-MBS-NGUFailureIndication								ProtocolIE-ID ::= 434

id-UserPlaneFailureIndication							ProtocolIE-ID ::= 435

id-UserPlaneFailureIndicationReport						ProtocolIE-ID ::= 436

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-MBS-ServiceAreaSmallAreaList							ProtocolIE-ID ::= x

id-NG-TNLA-Indicator									ProtocolIE-ID ::= x

-----------------End of the Change-------------------

- TP for BLCR on 38.423 Support of Location based HO over XnAP

-----------------Start of the Changes-------------------

## 9.1	Message Functional Definition and Content

### 9.1.1	Messages for Basic Mobility Procedures

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. NOTE: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| >NR A2X UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| >LTE A2X UE PC5 Aggregate Maximum Bit Rate | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.2.3.108 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| >Conditional Handover Time Based Information | O |  |  | This IE only applies to NTN. | YES | reject |
| >>Handover Window Start | M |  | INTEGER (0..549755813887) | Corresponds to information provided in the t1-Threshold contained in the ReportConfigNR IE as defined in TS 38.331 [10] | – |  |
| >>Handover Window Duration | M |  | INTEGER (1..6000) | Corresponds to information provided in the duration contained in the condEventT1 contained in the ReportConfigNR IE as defined in TS 38.331 [10] | – |  |
| >Maximum Number of Conditional Reconfigurations to Prepare | O |  | INTEGER (1..8 , …) | Indicates that the target NG-RAN node may prepare for CHO with candidate PSCell(s) configuration and indicates the maximum number of conditional reconfigurations that the target NG-RAN node may prepare. | YES | reject |
| >Coarse UE Location | O |  | OCTET STRING | Includes the coarseLocationInfo message as defined in TS 38.331 [18]. | YES | ignore |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| IAB Authorization Status | O |  | ENUMERATED(authorized, not authorized, ...) | This IE indicates the authorization status of the IAB node. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) | This IE indicates that information on DL LBT Failures occurring at the target gNB during handover execution that results in mobility failure is requested | YES | ignore |
| Aerial UE Subscription Information | O |  | 9.2.3.175 |  | YES | ignore |
| NR A2X Services Authorized | O |  | 9.2.3.176 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.2.3.177 |  | YES | ignore |
| A2X PC5 QoS Parameters | O |  | 9.2.3.178 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| Cell Based UE Trajectory Prediction | O |  | 9.2.3.180 | This IE contains information about cells that a UE is predicted to be connected to. | YES | ignore |
| Data Collection ID | O |  | 9.2.3.184 |  | YES | ignore |
| Candidate Relay UE Info List | O |  | 9.2.3.188 |  | YES | reject |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC configuration information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| Mobile IAB Authorization Status | O |  | 9.2.2.105 |  | YES | reject |
| Ranging and Sidelink Positioning Services Information | O |  | 9.2.3.208 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

-----------------Next Change-------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

UNCHANGED PART OMITTED

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-CoarseUELocation

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

UNCHANGED PART OMITTED

CHOinformation-Req ::= SEQUENCE {

cho-trigger						CHOtrigger,

targetNG-RANnodeUEXnAPID		NG-RANnodeUEXnAPID										OPTIONAL

-- This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace" --,

cHO-EstimatedArrivalProbability	CHO-Probability											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-Req-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-Req-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ID id-CHOTimeBasedInformation	CRITICALITY reject	EXTENSION CHOTimeBasedInformation		PRESENCE optional}|

{ ID id-CHO-Maxnoof-CondReconfig	CRITICALITY reject	EXTENSION CHO-Maxnoof-CondReconfig		PRESENCE optional }|

{ ID id-CoarseUELocation			CRITICALITY ignore	EXTENSION CoarseUELocation		PRESENCE optional },

...

}

CHOTimeBasedInformation ::= SEQUENCE {

cHO-HOWindowStart			CHO-HandoverWindowStart,

cHO-HOWindowDuration		CHO-HandoverWindowDuration,

iE-Extensions				ProtocolExtensionContainer { {CHOTimeBasedInformation-ExtIEs} }	OPTIONAL,

...

}

CHOTimeBasedInformation-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

UNCHANGED PART OMITTED

CoarseUELocation ::= OCTET STRING

UNCHANGED PART OMITTED

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

UNCHANGED PART OMITTED

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-CoarseUELocation																					ProtocolIE-ID ::= xxx

END

-- ASN1STOP

-----------------End of the Change-------------------