3GPP TSG-RAN WG3 Meeting #127                             	  R3-250108

Athens, GR, 17th – 21st Feb, 2025

Source:	CATT

Title:	(TP to TS38.300 BL CR) A-IoT protocol stack

Agenda Item:	16.2

Document for:	Discussion and Decision

- Introduction

On support of A-IoT, a study item has been completed in the last RAN WG meetings. Corresponding solutions on the architecture, signallings, locating, etc. are captured in the TR38.769 [1]. In RAN#106, a new work item for Ambient IoT has been approved(WID in [2]), aimed to standardize RAN aspects of Ambient IoT. In the WID, we will only focus on Topology 1, the major RAN3 scope are highlighted below:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this contribution, we will focus on the architecture aspects on support of Ambient IoT, especially the protocol stack on support of A-IoT. Base on the discussion, we will provide the observations and proposals accordingly.

- Discussion

For Topology 1, SA2 discussed and captured two architecture options for Topology 1 in their TR 23.700-13 [3], BS reader connects to AIoTF directly without AMF involvement or BS reader indirectly connects to AIoTF via AMF, as shown below:

Figure 8.1.2-2: Non-Roaming 5G System Architecture in reference point representation (Direct Path)

Figure 8.1.2-5: Non-Roaming 5G System Architecture in reference point representation (indirect Path via AMF)

As above, in Topology 1, AIoT RAN (BS reader) may connect to AIoTF directly, or connect to AIoTF indirectly via AMF.To support flexible deployments, both of the options as above should be supported.

Proposal 1: Both direct and indirect path between AIoT RAN and AIoTF should be supported.

In the RAN TR38.769 [1], the logical system architecture and protocol stack for the T1 are captured, as below:

6.4.1	Support of Topology 1

Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN.

Figure 6.4.1-1 Logical system architecture for topology 1

For Toplogy 1, architecture and protocol aspects of split RAN architecture are not studied.

In Topology 1, the XX interface is the NG-C interface.

Figure 6.4.1-2 shows the Protocol stack for Topology 1:

Figure 6.4.1-2. Protocol Stack for Topology 1

NOTE 1:	The protocol stack in Figure 6.4.1-2 does not illustrate how A-IoT upper layer information, if any, is transported over XXAP.

As agreed in the study phase, XX interface is the NG-C interface. Which means necessary information on support of Ambient IoT should be exchanged via NG-C interface.

Proposal 2: NG-C is used to convey A-IoT related information between AIoT RAN and AIoT CN.

However, how to support A-IoT related functions over the NG-C for both direct/indirect connection between AIoT RAN and AIOTF is not clear, which should be discussed and decided firstly in the normative phase before we go to the detail signalling design.

Two potential options to transfer A-IoT related functions over NGAP:

- Option 1: Define new NGAP procedures for A-IoT related services, i.e. Inventory, Command.

- Option 2: Define new protocol layer for A-IoT related functions and carry it over NGAP, similar to NRPPa.

For the option 1, it’s straightforward for the direct path case, but for the indirect path case, the new A-IoT procedures will be terminated at AMF, and AMF should convert and route them towards AIoTF, which means AMF is deeply involved to make support of indirect AIoTF deployment.

For the option 2, the new defined protocol layer is between AIoT CN and AIoTF, AMF does not need to interpret A-IoT functions transferred over the NGAP, it just need to route the A-IoT related messages between A-IoT RAN and AIoTF, which could simplify the impact to AMF for the indirect path case.

From RAN3 perspective, both of the two options are feasible to support A-IoT. To minimize the specification impact, a unified solution for both direct and indirect path between A-IoT RAN and AIoTF.

Proposal 3: A unified solution (either to enhance NGAP or define a new protocol layer on top of NGAP) should be adopted for both direct and indirect path between A-IoT RAN and AIoTF.

Different options may have different impact to AMF for indirect path case.  To minimize the impact to AMF, especially for indirect path case, it’s slightly preferred to define a new protocol layer, e.g. AIoT layer to for AIoT related functions. Then AMF just need to do the routing between AIoT RAN and AIoTF. The example of the protocol stacks for direct path and indirect path could be illustrated with the figures below:

Figure 1. A-IoT Protocol Stack for direct path

Figure 2. A-IoT Protocol Stack for indirect path

Proposal 4: To minimize the impact to AMF, it’s slightly preferred to define a new protocol layer, e.g. AIoT layer over the NGAP for AIoT related functions.

Due to the architecture options impact RAN and core network, we should send a liaison to SA2 to confirm our decision and check if any issue before we work on the NGAP details for AIoT.

Proposal 5: Send LS to SA2 to confirm the feasibility and potential impact to define an AIoT layer between AIoT RAN and AIoTF, drat LS is provided in the Annex.

Base on the discussion above, we provide the stage 2 TP for TS 38.300, in the section 5.

Proposal 6: Discuss and agree the TP to TS38.300 on AIoT system architecture aspects.

- Conclusion

Based on the discussion in section 2, the following proposals are provided:

Proposal 1: Both direct and indirect path between AIoT RAN and AIoTF should be supported.

Proposal 2: NG-C is used to convey A-IoT related information between AIoT RAN and AIoT CN.

Proposal 3: A unified solution (either to enhance NGAP or define a new protocol layer on top of NGAP) should be adopted for both direct and indirect path between A-IoT RAN and AIoTF.

Proposal 4: To minimize the impact to AMF, it’s slightly preferred to define a new protocol layer, e.g. AIoT layer over the NGAP for AIoT related functions.

Proposal 5: Send LS to SA2 to confirm the feasibility and potential impact to define an AIoT layer between AIoT RAN and AIoTF, drat LS is provided in the Annex.

Proposal 6: Discuss and agree the TP to TS38.300 on AIoT system architecture aspects.

- Reference

- TR 38.769 TR 38.769 Study on solutions for ambient IoT (Internet of Things)

- RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- TR 23.700-13-110 Study on Architecture support of Ambient power-enabled Internet of Things

- TP to TS 38.300 BL CR

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

# X	Support for Ambient IoT

### x.1	General

The figures below show the Protocol stack for Ambient IoT Topology 1, where:

- AIoTF performs the functions as specified in TS 23.501 [3].

- AIoT RAN may connect to AIoTF directly, or indirectly via AMF.

- NGAP and corresponding sublayers (terminated in gNB and AIoTF/AMF side) perform the functions listed in caluse 4.

- AIoT Layer protocol (terminated in gNB and AIoTF side) performs A-IoT related functions listed in TS xx.xxx.

Figure X.1-1 A-IoT Protocol Stack (direct path)

Figure X.1-2 A-IoT Protocol Stack (indirect path)

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>

- Draft LS to SA2 on support of direct and indirect connection between AIoT RAN and AIoTF

Title:	[Draft] LS on protocol stack to support direct and indirect connection between AIoT RAN and AIoTF

Response to:	-

Release:	Rel-19

Work Item:	Ambient_IoT_Solutions

Source:	CATT (to be RAN3)

To:	SA2, RAN2

Cc: 	SA4

Contact Person:

Name:	Jiancheng Sun

E-mail Address:	sunjiancheng@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

On Solutions for Ambient IoT (Internet of Things) in NR, RAN3 discussed the architecture options between AIoT RAN and AIoT CN considering both direct and indirect connection between AIoT RAN and AIoTF.

RAN3 discussed two potential options to carry AIoT related procedures between AIoT RAN and AIoTF, specify AIoT related procedures in the NGAP, or define a new protocol layer over NGAP. RAN3 slightly prefer to define AIoT layer for AIoT to minimize the impact to AMF, especially for indirect connection case.

RAN3 would like SA2 to further confirm the feasibility and potential impact to define an AIoT layer between AIoT RAN and AIoTF.

2. Actions:

To SA2:

ACTION: RAN3 kindly asks SA2 to take above into consideration and feedback if necessary.

3. Date of Next RAN3 Meetings:

RAN3#127bis	7th – 11th Apr 2025				Wuhan, CN

RAN3#128	19th – 23rd May 2025				Malta, MT