3GPP TSG-RAN WG3 #127                                             R3-250109

Athens, GR, 17th – 21st Feb, 2025

Source:	CATT

Title:	(TP to BL CR for XXAP) A-IoT Information exchange between A-IoT CN and A-IoT RAN

Agenda Item:	16.2

Document for:	Decision

- Introduction

On support of A-IoT, a study item has been completed in the last RAN WG meetings. Corresponding solutions on the architecture, signaling, locating, etc. are captured in the TR38.769 [1]. In RAN#106, a new work item for Ambient IoT has been approved (WID in [2]), aimed to standardize RAN aspects of Ambient IoT. In the WID, we will only focus on Topology 1, the major RAN3 scope are highlighted below:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this contribution, we provide our analysis for A-IoT procedures and information exchange between A-IoT CN and A-IoT RAN.

- Discussion

In TR38.769, the candidate procedure and some information exchanged between A-IoT CN and A-IoT RAN has been captured. As agreed in the study phase, XX interface is the NG-C interface. On how to transfer A-IoT related functions over the NG interface, there may be two potential options as discussed in [3]. Here we just discuss the detailed information in inventory and command procedure.

2.1 Signalling and candidate procedure

From RAN3 prospective, it may be not needed to further discussing the signaling and candidate procedure captured in clause 6.5.2 of TR38.769 unless triggered by other WG. Currently RAN3 could begin WI work based on these agreements.

Proposal 1: It is proposed for RAN3 to begin WI work based on current signalling and candidate procedure for Topology 1 captured in clause 6.5.2 of TR38.769 unless triggered by other WG.

2.2 Information in inventory procedure

For signalling message for A-IoT Inventory procedure, there are inventory request, inventory response and inventory report message captured in candidate procedures.

For AIoT Inventory request message, we’ve agreed to use AIoT Device Identification to support inventory for a single device, a group of device, or all devices. Some other information elements are also discussed in SI phase, as below:

- Session ID or Task ID;

- Cause/Purpose of inventory;

- Reader id;

- Estimated Number of devices;

- Periodicity;

For reader ID, as discussed in [4], it may be included in inventory request message.

For Inventory Session ID or Task ID, it could be used to triggering multiple inventory procedures to different areas, different devices or different readers within an AIoT RAN in parallel. Session ID or Task ID could also be used in inventory response message to correlate with inventory request message. But there may be an issue when multiple inventory procedures triggered to the same area, device or reader at the same time, further discussion may be needed for RAN2.

Proposal 2: Session ID or Task ID, if needed, could be used to triggering multiple inventory procedures to different areas, different devices or different readers within an AIoT RAN in parallel. It could also be used in inventory response message to correlate with inventory request message.

For the cause/purpose of inventory, it may indicate the cause of triggering inventory procedure, for example, inventory for Command, inventory for locating, inventory for requesting device ID. Whether need to distinguish the cause/purpose of the inventory is pending to the behaviours to be taken in AIoT RAN. Currently, we do not see any necessary to distinguish the purpose and take different actions in AIoT RAN. As above, no need to indicate AIoT RAN the cause/purpose of the Inventory procedure.

Proposal 3: No need to indicate AIoT RAN the cause/purpose of the Inventory procedure.

For Estimated Number of devices, it has been agreed in LS [5] as below:

Q1: Is it feasible for CN to provide the following information to the reader?

- the service type of A-IoT (e.g. inventory, command)

- whether the service is targeted for a single or multiple devices;

- the approximate number of target devices (if available).

SA2 answer: CN can provide the information listed above to the reader if available.

Proposal 4: Introducing estimated Number of devices in Inventory request message.

For Periodicity, it is used for AIoT RAN to trigger periodic inventory procedure by a single request from AIoT CN. For example, operator start a task to inventory warehouse once a day, it may be better for AIoT CN than for AIoT RAN to trigger inventory procedure every day because AIoT CN is more reliable. Thus, it seems not necessary to provide the periodicity information to AIoT RAN.

Proposal 5: Periodically inventory, if needed, could be triggered by the AIoT server or AIoT CN. Not necessary to introduce the Periodicity in XXAP message.

AIoT Device Identification was agreed to be used to trigger inventory request to a single device, a group of devices, or all devices.

Since there may be security issue when transferring Device info directly at AIoT radio interface, device info may be included or partially included in AIoT NAS PDU because AMF/AIoTF may apply security protection to NAS. But it is finally up to conclusion of SA2/SA3.

Proposal 6: How to include AIoT Device Identification in inventory request message is finally up to conclusion of SA2/SA3. The following way may be possible:

- including AIoT Device Identification explicitly in inventory request (as an XXAP IE);

- including AIoT Device Identification in the NAS PDU of the AIoT device;

- partially including AIoT Device Identification in AIoT NAS PDU;

- Both 1) and 2) at the same message, or both 1) and 3) at the same message.

According to the agreement in SI phase, a new class 1 procedure is introduced for inventory procedure. To indicate the result of inventory request, success or failure indication should be included in inventory response message, or introducing two messages (inventory response and inventory failure) to indicate the result. Either solution is feasible. We slightly prefer introducing a new inventory failure message with a failure cause due to simplicity.

Proposal 7: Introducing a new inventory failure messages with a failure cause to indicate the failure of inventory procedure.

For session ID, it could be included in inventory response and inventory failure message as discussed above. For reader ID, it should be included in report message, not response message.

Proposal 8: Session ID or Task ID, if needed, should be included in inventory response and inventory failure message to correlate with inventory request message.

For the content of Inventory report message, Reader ID may be needed for Topology 1 to locating purpose, i.e. indicate AIoT CN the coarse location of the device.

Proposal 9: reader ID should be included in inventory report message.

For Device Identification, as discussed in P6 above, it may be included in NAS PDU or explicitly in inventory report message which is up to the conclusion of SA2/SA3.

For Enrichment info (e.g., signal strength measurements by Reader), it needs further discussion.

Proposal 10: device Identification should be included in inventory report message. How to include AIoT Device Identification in inventory report message is finally up to conclusion of SA2/SA3.

If AIoT RAN or AIoT CN decide that inventory is complete, an inventory complete message is necessary to inform the other side. First it is reasonable for AIoT CN to complete the inventory procedure, for example, based on predefined time duration, or operator decides to complete the ongoing the inventory procedure. As for AIoT RAN, whether it could check the completion of inventory is up to RAN2. RAN3 may wait for the conclusion of RAN2.

Proposal 11: It is proposed to introduce an inventory complete message in XXAP interface. AIoT CN may indicate AIoT RAN the completion of inventory procedure. Whether AIoT RAN could check the completion of inventory is up to RAN2.

2.3 Information in Command procedure

For signalling message for A-IoT command procedure in Topology 1, there are command request and command response captured in candidate procedures.

We have agreed command service is performed to a single device. For the issue whether command service could be triggered towards a group of device, we think it is possible. For example, triggering read/write operation to a group of device. As for command operation to all devices, we are not sure whether there is the scenario to read/write all devices.

So, device info in command request message may include a device or a group of device info.

Proposal 12: The scenario of triggering command to a group of devices may be valid, so it is proposed to use AIoT Device Identification to indicate a device or a group of device in XXAP interface for command request.

In case of writing device, the action “write” and related “data” is encapsulated as NAS PDU and NG-RAN do not decode NAS. In case of reading device, the action “read” and the “data” as the result of read operation is also encapsulated as NAS PDU. So, NAS info is necessary in command request and command response message.

Proposal 13: It is proposed to include NAS information in command request and command response message.

For the other information in command response message, success or failure indication is always needed captured in LS [5] as below:

Q2: Can there be a case where a D2R response is not expected for a R2D message for command service type? If so, can the CN indicate to the reader whether a message is expected in D2R direction in response to an R2D message for command service type (e.g., either explicitly or implicitly by indicating the response message size if available)?

SA2 answer: No, a D2R response is always expected for a R2D message for command service type (e.g., the service data response for 'Read' service operation, and the operation result indication response for 'Write' or 'Disable' service operation).

Intruding a success or failure indication in command response message or a new command failure message is feasible to indicate the result of command operation.

Proposal 14: Intruding a success or failure indication in command response message or a new command failure message with failure cause is feasible.

For reader ID, it should be included in command response message to indicate the reader which could interact with the device. A-IoT CN could store the relationship between reader and device for subsequent operation. For example, when command procedure is triggered after then, reader ID could be included in command request message to indicate the suitable reader.

Proposal 15: It is proposed to include reader information in command request and command response message.

For Task ID/Session ID, for the same reason as inventory procedures, it is proposed to introduce Task ID/Session ID for AIoT command procedure.

Proposal 16: It is proposed to introduce Task ID/Session ID for AIoT command procedure.

- Conclusions

In this contribution, we mainly discussed A-IoT Procedures and Information exchange between A-IoT CN and A-IoT RAN. Based on the discussion above, the following proposals are provided:

signalling and candidate Procedure：

Proposal 1: It is proposed for RAN3 to begin WI work based on current signalling and candidate procedure for Topology 1 captured in clause 6.5.2 of TR38.769 unless triggered by other WGs.

Information in inventory procedure：

Proposal 2: Session ID or Task ID, if needed, could be used to triggering multiple inventory procedures to different areas, different devices or different readers within an AIoT RAN in parallel. It could also be used in inventory response message to correlate with inventory request message.

Proposal 3: No need to indicate AIoT RAN the cause/purpose of the Inventory procedure.

Proposal 4: Introducing estimated Number of devices in Inventory request message.

Proposal 5: Periodically inventory, if needed, could be triggered by the AIoT server or AIoT CN. Not necessary to introduce the Periodicity in XXAP message.

Proposal 6: How to include AIoT Device Identification in inventory request message is finally up to conclusion of SA2/SA3. The following way may be possible:

- including AIoT Device Identification explicitly in inventory request (as an XXAP IE);

- including AIoT Device Identification in the NAS PDU of the AIoT device;

- partially including AIoT Device Identification in AIoT NAS PDU;

- Both 1) and 2) at the same message, or both 1) and 3) at the same message.

Proposal 7: Introducing a new inventory failure messages with a failure cause to indicate the failure of inventory procedure.

Proposal 8: Session ID or Task ID, if needed, should be included in inventory response and inventory failure message to correlate with inventory request message.

Proposal 9: reader ID should be included in inventory report message.

Proposal 10: device Identification should be included in inventory report message. How to include AIoT Device Identification in inventory report message is finally up to conclusion of SA2/SA3.

Proposal 11: It is proposed to introduce an inventory complete message in XX interface. AIoT CN may indicate AIoT RAN the completion of inventory procedure. Whether AIoT RAN could check the completion of inventory is up to RAN2.

Information in Command procedure：

Proposal 12: The scenario of triggering command to a group of devices may be valid, so it is proposed to use AIoT Device Identification to indicate a device or a group of device in XX interface for command request.

Proposal 13: It is proposed to include NAS information in command request and command response message.

Proposal 14: Intruding a success or failure indication in command response message or a new command failure message with failure cause is feasible.

Proposal 15: It is proposed to include reader information in command request and command response message.

Proposal 16: It is proposed to introduce Task ID/Session ID for AIoT command procedure.

- Reference

[1] TR 38.769 Study on solutions for ambient IoT (Internet of Things)

[2] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[3] R3-250108 (TP to TS38.300 BL CR) A-IoT protocol stack, CATT

[4] R3-250110 A-IoT reader selection, CATT

[5] S2-2501241” Reply LS on Assistance Information from CN to the Reader” SA WG2 Meeting #166

- TP for XXAP interface message

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

x.x.1 INVENTORY REQUEST

This message is sent by the AMF/AIoTF and is used to request the A-IoT RAN to perform inventory operation.

Direction: AMF/AIoTF  A-IoT RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | Message Type definition |  | YES | reject |
| Task ID | M |  | INTEGER (0..FFS) |  | YES | reject |
| A-IoT Device Identification | M |  | FFS |  | YES | reject |
| Estimated Number of devices | O |  | INTEGER (0..FFS) |  | YES | ignore |
| NAS-PDU | O |  | OCTET STRING |  | YES | ignore |
| Reader Identification | O |  | FFS |  | YES | ignore |

x.x.2 INVENTORY RESPONSE

This message is sent by the A-IoT RAN as a response to the inventory request.

Direction: A-IoT RAN  AMF/AIoTF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | Message Type definition |  | YES | reject |
| Task ID | M |  | INTEGER (0..FFS) |  | YES | reject |

x.x.3 INVENTORY FAILURE

This message is sent by the A-IoT RAN to indicate that the inventory request was unsuccessful.

Direction: A-IoT RAN  AMF/AIoTF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | Message Type definition |  | YES | reject |
| Task ID | M |  | INTEGER (0..FFS) |  | YES | reject |
| Cause | M |  | FFS |  | YES | ignore |

x.x.4 INVENTORY REPORT

This message is sent by the A-IoT RAN to report inventory result.

Direction: A-IoT RAN  AMF/AIoTF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | Message Type definition |  | YES | reject |
| A-IoT Device Identification | M |  | FFS |  | YES | reject |
| Reader Identification | M |  | FFS |  | YES | ignore |

x.x.5 COMMAND	REQUEST

This message is sent by the AMF/AIoTF and is used to request the A-IoT RAN to perform command operation.

Direction: AMF/AIoTF  A-IoT RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | Message Type definition |  | YES | reject |
| Task ID | M |  | INTEGER (0..FFS) |  | YES | reject |
| A-IoT Device Identification | M |  | FFS |  | YES | reject |
| NAS-PDU | O |  | OCTET STRING |  | YES | ignore |
| Reader Identification | O |  | FFS |  | YES | ignore |

x.x.6 COMMAND RESPONSE

This message is sent by the A-IoT RAN as a response to the inventory request.

Direction: A-IoT RAN  AMF/AIoTF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | Message Type definition |  | YES | reject |
| Task ID | M |  | INTEGER (0..FFS) |  | YES | reject |
| NAS-PDU | O |  | OCTET STRING |  | YES | ignore |
| Reader Identification | M |  | FFS |  | YES | reject |

x.x.7 COMMAND FAILURE

This message is sent by the A-IoT RAN to indicate that the command request was unsuccessful..

Direction: A-IoT RAN  AMF/AIoTF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | Message Type definition |  | YES | reject |
| Task ID | M |  | INTEGER (0..FFS) |  | YES | reject |
| Cause | M |  | FFS |  | YES | ignore |

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>