3GPP TSG RAN WG3 Meeting #127	R3-250116

Athens, Greece, 17 – 21 Feb, 2025

Title: 	(TP to TS38.423) inter-CU LTM

Source: 	ZTE Corporation

Agenda item:	13.2

Document for:	Discussion and Decision

- Introduction

In the last RAN3 #126 meeting, RAN3 has made the following progress.

RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution. The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message. The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message. Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch. Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation. Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB. Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation? Continue to work on the solutions comparison and details…

Meanwhile, in this meeting, RAN3 has also received an LS (R3-250008) from RAN2, which includes the following progress from RAN2 on the inter-CU LTM.

Agreements on inter-CU LTM:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

- From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

7.	In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations.

8.	RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

9.	RAN2 to support intra-CU MCG LTM with SCG configuration.

10.	It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

11.	RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

This paper further discusses the inter-CU LTM based on the above progress.

- Discussion

  - Solution to support semi-persistent CSI-RS

RAN3 confirmed it is feasible to specify the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s), and RAN3 will continue to work on the details.

SP CSI-RS configuration

The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

- Periodic CSI-RS: CSI-RS is activated after configured, i.e., UE can measure the CSI-RS.

- Semi-persistent CSI-RS: after configured, Semi-persistent CSI-RS need an additional MAC CE for activation, and then UE can measure the semi-persistent CSI-RS.

From the perspective of configuration, there is no different between supporting Periodic CSI-RS and Semi-persistent CSI-RS. The source shall request candidate to provide CSI-RS information by UE association message, and candidate cells provide CSI-RS configuration to source gNB. After collected from all accepted candidate cells, source generates the common CSI-RESOURCE to each candidate cell or generate CSI-REPORT. After the interaction, source CU can configure the entire candidate configuration to the UE.

Proposal 1: From the perspective of configuration, supporting Periodic CSI-RS and Semi-persistent CSI-RS is no difference, and shall use the same procedure.

Because SP CSI-RS information shall be configured before LTM execution phase, source gNB shall request the candidate gNB(s) to provide SP CSI-RS for specified UE in the Xn: Handover request message, then the candidate gNB(s) provide the SP CSI-RS via Xn: Handover request acknowledge message.

Proposal 2: SP CSI-RS information is a UE-specific RS, it can be configured before LTM execution, by Xn: Handover procedure.

CSI-RS activation

Similar to SSB, CSI-RS is also used for early synchronisation. More, it shall be activated before LTM execution.

In detail, the source gNB shall coordinate with each of candidate gNBs to activate full/part of configured SP CSI-RS, then indicate UE to perform L1-RSRP measurement by MAC-CE.

Observation 1: source gNB shall coordinate with candidate gNB to active full/part of configured SP CSI-RS information, by Xn signalling/procedure.

The key issue to support Semi-persistent CSI-RS is that the source node and the target node shall inform each other when the candidate cell active the semi-persistent CSI-RS. There are two possible options from RAN3 perspective to supporting Semi-persistent CSI-RS:

- Option 1: Source node trigger:

After configuring the RS to UE, source node decides to trigger the Semi-persistent CSI-RS of candidate cell, and sending XnAP message to candidate node for request candidate node active Semi-persistent CSI-RS.

- Option 2: Candidate node trigger:

After configuring the RS to UE, candidate node decides to trigger the Semi-persistent CSI-RS, and sending XnAP message to source node for informing source node the Semi-persistent CSI-RS is active.

Proposal 3: RAN3 to discuss the coordination between serving cell and candidate cell(s) on support of semi-persistent CSI-RS(s), two options can take as baseline.

- Option 1: Source node decides to trigger activation of Semi-persistent CSI-RS, and request candidate node to active it. The message for such requesting can be FFS.

- Option 2: Candidate node decides to trigger activation of Semi-persistent CSI-RS, and inform source node of such activation it. The message for such informing can be FFS.

From our perspective, the option 1 is more reasonable, as source node has the fully knowledge of measurement results and UE location information. When the source node aware UE is near to the candidate cell, it will trigger the candidate cell to active its Semi-persistent CSI-RS. Accordingly, the source node also needs to indicate UE measurement for activated SP CSI-RS, which should be discussed in RAN2.

Proposal 3a: RAN3 agrees that Source node decides to trigger activation of Semi-persistent CSI-RS, and request candidate node to active it (i.e., option 1). The message for such requesting can be FFS.

CSI-RS deactivation

The activated CSI-RS will consume network/UE’s resource and power, so that, it is necessary to deactivate it if not used, so that, the activated CSI-RS shall be deactivated after LTM execution

Observation 2: The activated CSI-RS shall be deactivated after LTM execution.

Observation 3: After LTM execution, other candidate gNB(s) shall be informed of LTM execution and new source node, then it/they can deactivate CSI-RS if activated. How to inform other candidate gNB(s) of subsequent LTM execution is still discussing in RAN3.

Moreover, if proposal 3a is agreed, CSI-RS activation is triggered by source gNB, thus, for subsequence LTM, it is the new source gNB to (re)activate CSI-RS if needed.

Proposal 3b: RAN3 agrees that Source node decides to trigger deactivation of Semi-persistent CSI-RS, and request candidate node to deactivate it. The message for such requesting can be FFS.

  - Data forwarding for subsequent inter-CU LTM

In [R3-243395], it provides several options to support subsequent inter-CU LTM.

Take the UE moves from the source gNB to the candidate gNB1 and then to the candidate gNB2 for example. There are three options for the new source gNB to know the data forwarding information associated with other candidate gNBs. - Option 1: the source gNB collects the data forwarding information associated with all the candidate gNBs from each candidate gNB, and sends it to all the candidate gNBs during the LTM preparation phase. For example, the source gNB sends the data forwarding information to the gNB1 and gNB2 during the LTM preparation phase. This option may introduce new XnAP signalling between the source gNB and the candidate gNBs. - Option 2: the source gNB collects the data forwarding information associated with all the candidate gNBs from each candidate gNB, and sends it to the selected target gNB (i.e., new source gNB), e.g., via Cell Switch Notification message. For the subsequent inter-CU LTM, the new source gNB sends the data forwarding information associated with all the candidate gNBs to the new selected target gNB (i.e., next new source gNB). For example, the source gNB sends the data forwarding information to the gNB1 (as new source gNB) during the first-time cell switch, and then gNB1 (new source gNB) sends the data forwarding information to the gNB2 during the second-time cell switch. - Option 3: the source gNB collects the data forwarding information associated with all the candidate gNBs from each candidate gNB, and sends it to the selected target gNB, no matter which time the inter-CU LTM is performed. For example, the source gNB sends the data forwarding information to the gNB1 for the first-time cell switch, and the source gNB sends the data forwarding information to the gNB2 for the second-time cell switch.

From our view, all of above options are workable, but we think option 2 is straightforward. The reason is that, data forwarding is an optional feature, in legacy, it is the source node to request data forwarding if needed, and then the target node reply the data forwarding address to the source node if agreeable.

In legacy handover procedure, The source NG-RAN node may suggest downlink data forwarding per QoS flow established for a PDU session and may provide information how it maps QoS flows to DRBs. The target NG-RAN node decides data forwarding per QoS flow established for a PDU Session. In other words, which QoS flow will be suggested depends on the new source gNB, and which QoS flow will be accepted is determined by new candidate gNB.

Further, the DRB mapping in source gNB and target gNB also has impact on the need of data forwarding. In LTM, the QoS to DRB mapping in the new source gNB may be different to that in the old source gNB. Thus, per DRB data forwarding configuration need to be negotiated between the new source gNB and candidate gNB(s).

So, it is reasonable to negotiate new data forwarding TNL information after a LTM procedure is completed and before the subsequent LTM.

On the other hand, if the data forwarding configuration collected by the old source is still applicable for the new source gNB, i.e. per QoS data forwarding proposal and per DRB data forwarding proposal and DRB configuration are the same as old source gNB, the negotiation between new source gNB and all other candidate gNB is not needed.

Proposal 4: After LTM completion, for subsequent LTM, the new serving gNB can request other candidate node(s) to negotiate data forwarding per PDU session/DRB. New serving gNB and other candidate gNB negotiate data forwarding after LTM completion and before next subsequent LTM.

Another issue related to data forwarding for subsequent LTM is how to handle the data that is received in the early data forwarding during the precedent LTM procedure.

In legacy handover, the data forwarded to candidate gNB in early data forwarding will be released after handover procedure is completed, e.g. by reception of HANDCOVER CANCEL message.

But for subsequent LTM, the data forwarded to the candidate gNB except target gNB in early data forwarding will be useless after the precedent LTM procedure is completed. And in the subsequent LTM procedure, the new source gNB may perform early data forwarding to the candidate too. For the candidate gNB, it is unclear whether and when it should flush the data received in the precedent LTM procedure. If the data received in precedent LTM procedure is not flushed, they will bring additional buffer requirement to the candidate gNB.

Proposal 5: RAN3 to discuss whether and when a candidate gNB flush UE data that is received in early data forwarding after a LTM procedure.

  - Establish new Xn connection between new serving gNB and candidate gNBs and release old Xn connection between old serving gNB and candidate gNBs

After completion of the LTM, new UE associated signalling connection should be established between the new serving gNB and candidate gNB(s) in order to perform subsequent LTM. To establish the new signaling connections, the new serving gNB needs to know the information of other candidate gNB(s), e.g. the gNB ID and corresponding Xn AP ID allocated for the UE. With these information, the new serving gNB can initiate procedure to setup signaling connection with other candidate gNB(s).

The old serving gNB has such information of all candidate gNB(s).The old serving gNB can transfer such information to the new serving gNB in one of following procedures:

- LTM configuration update procedure after path switch.

- Cell switch notification

Proposal 6: Old serving gNB transfers the gNB ID of other candidate gNBs and Xn AP ID used by the candidate gNB to new serving gNB. FFS which procedure is used.

During path switch procedure, the AMF may update configuration for the UE, e.g. new per PDU session UL TNL address, released PDU session(s), and changed QoS parameters for QoS flows. The updated configuration needs to be updated to all candidate gNB(s) before next subsequent LTM. So after receiving PATH SWITCH REQUEST ACKNOWLEDGE message from AMF, the new serving gNB needs to initiate Xn procedure to update the configuration with all candidate gNBs.

In addition, the Xn procedure should also accomplish the setup of UE associated signaling connection between new serving gNB and candidate gNB(s), and update of data forwarding TNL information based on updated configuration.

If UE side configuration is involved, candidate gNB needs to update the prepared LTM configuration and include the update LTM configuration in the message sent to new serving gNB.

So the handover modification procedure can be used to accomplish at least following functions:

- setup Xn signaling for the UE between new serving gNB and candidate gNB(s)

- update LTM configuration

- update data forwarding TNL information (updated NG-U per PDU session UL TNL address)

Proposal 7: After path switch procedure, LTM configuration update procedure can be used to setup signaling connection between other candidate gNB(s), update LTM configuration, and update data forwarding TNL information.

Another issue is whether we support to release the UE associated Xn signaling connections between the old serving gNB and other candidate gNB(s). In our understanding, it is unnecessary always maintain the old Xn signaling connection. Release the old signaling connections should be allowed by specification. At least release it is needed in the case the old serving gNB is not a candidate gNB after LTM completion.

There are following options to release the old UE associated Xn signaling connections:

Option 1: During LTM configuration update (i.e., handover modification) procedure after path switch, the new serving gNB indicates candidate gNB(s) explicitly or implicitly to release the UE associated signalling connection between the old serving gNB.

Option 2: The new serving gNB indicates to old serving gNB that signaling connection between old serving gNB and candidate gNB can be released explicitly or implicitly after handover success or path switch completed. The old serving gNB initiates a class 1 Xn procedure to candidate gNBs to release UE associated Xn signaling connection between candidate gNB(s).

Note that to release UE associated signaling connection only release the resources related to the UE associated signalling connection between old serving gNB and candidate gNB(s). The prepared LTM configuration in the candidate gNB is not released in this procedure.

Proposal 8: RAN3 to discuss release of UE associated signaling connections between old serving gNB and candidate gNB.

Proposal 8a: If the release of UE associated signaling connection is necessary, RAN3 shall agree with option 2.

  - TA Information Transfer

For LTM, TA information Transfer is valuable, to avoid RACH procedure. In intra-CU LTM and inter-CU LTM, we have already agreed to introduce it. In order to support subsequent LTM, we shall transfer TA information to each of candidate DU.

Proposal 9: Introduce TA information Transfer during LTM preparation phase and LTM execution phase.

The TP to TS38.423 captures the corresponding the IEs to introduce TA information Transfer.

- Conclusion

After the above analysis, we provide the following proposals.

Observation 1: source gNB shall coordinate with candidate gNB to active full/part of configured SP CSI-RS information, by Xn signalling/procedure.

Observation 2: The activated CSI-RS shall be deactivated after LTM execution.

Observation 3: After LTM execution, other candidate gNB(s) shall be informed of LTM execution and new source node, then it/they can deactivate CSI-RS if activated. How to inform other candidate gNB(s) of subsequent LTM execution is still discussing in RAN3.

Proposal 1: From the perspective of configuration, supporting Periodic CSI-RS and Semi-persistent CSI-RS is no difference, and shall use the same procedure.

Proposal 2: SP CSI-RS information is a UE-specific RS, it can be configured before LTM execution, by Xn: Handover procedure.

Proposal 3: RAN3 to discuss the coordination between serving cell and candidate cell(s) on support of semi-persistent CSI-RS(s), two options can take as baseline.

  - Option 1: Source node decides to trigger activation of Semi-persistent CSI-RS, and request candidate node to active it. The message for such requesting can be FFS.

  - Option 2: Candidate node decides to trigger activation of Semi-persistent CSI-RS, and inform source node of such activation it. The message for such informing can be FFS.

Proposal 3a: RAN3 agrees that Source node decides to trigger activation of Semi-persistent CSI-RS, and request candidate node to active it (i.e., option 1). The message for such requesting can be FFS.

Proposal 3b: RAN3 agrees that Source node decides to trigger deactivation of Semi-persistent CSI-RS, and request candidate node to deactivate it. The message for such requesting can be FFS.

Proposal 4: After LTM completion, for subsequent LTM, the new serving gNB can request other candidate node(s) to negotiate data forwarding per PDU session/DRB. New serving gNB and other candidate gNB negotiate data forwarding after LTM completion and before next subsequent LTM.

Proposal 5: RAN3 to discuss whether and when a candidate gNB flush UE data that is received in early data forwarding after a LTM procedure.

Proposal 6: Old serving gNB transfers the gNB ID of other candidate gNBs and Xn AP ID used by the candidate gNB to new serving gNB. FFS which procedure is used.

Proposal 7: After path switch procedure, LTM configuration update procedure can be used to setup signaling connection between other candidate gNB(s), update LTM configuration, and update data forwarding TNL information. Proposal 7: RAN3 to discuss release of UE associated signaling connections between old serving gNB and candidate gNB.

Proposal 8: RAN3 to discuss release of UE associated signaling connections between old serving gNB and candidate gNB.

Proposal 8a: If the release of UE associated signaling connection is necessary, RAN3 shall agree with option 2.

Proposal 9: Introduce TA information Transfer during LTM preparation phase and LTM execution phase.

The TP to TS38.423 captures the corresponding the IEs to introduce TA information Transfer.

- Text Proposal to TS38.423

8.x	Procedures for L1/L2 Triggered Mobility

8.x.1	LTM Configuration Update

Editor’s note: Details need to be continued.

8.x.1.1	General

The purpose of the LTM Configuration Update procedure is to update LTM configuration data. The procedure uses UE associated signalling.

8.x.1.2	Successful Operation

Figure 8.x.1.2-1: LTM Configuration Update procedure. Successful operation

The NG-RAN node1 initiates the procedure by sending the LTM CONFIGURATION UPDATE message to NG-RAN node2.

The LTM CONFIGURATION UPDATE message may contain:

-	the Early Sync Information IE;

-	the Early Sync Information for SUL IE;

-	the LTM Information Request IE.

If the Early Sync Information IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider it as the consolidated early sync information to use for subsequent LTM. If the Early Sync Information for SUL IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider it as the consolidated early sync information for SUL to use for subsequent LTM.

If the LTM Information Request IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider that the NG-RAN node1 has requested the LTM information update for the UE.

If the CSI-RS Resource Configuration IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, include the CSI-RS Report Configuration IE in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message.

If the LTM Information Response IE is contained in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 shall, if supported, store it and use it as defined in TS 38.300 [9].

9.1.1.x1 CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

Direction: source NG-RAN node  target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Corresponds to the TCI-StateId IE as defined in TS 38.331 [10]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [10]. |  |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

9.1.1.x2		TA INFORMATION TRANSFER

This message is sent by the NG-RAN node1 to inform the NG-RAN node2 about the Timing Advance value and related information.

Direction: NG-RAN node1  NG-RAN node2

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| TA Information List |  | 1 |  |  | YES | ignore |
| > TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Preamble Index | M |  | INTEGER (0..63) |  | - |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [16]. | - |  |
| >>Source gNB-DU ID | M |  | gNB-DU ID 9.3.1.xxx |  | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |