3GPP TSG RAN WG3 Meeting #127	R3-250122

Athens, Greece, 17 – 21 Feb, 2025

Title: 	Discussion on SA2 Ambient IoT TR conclusion update

Source: 	ZTE Corporation

Agenda item:	8.1

Document for:	Discussion and Decision

- Introduction

In this meeting, RAN3 has received an incoming LS (R3-250075 “LS on SA2 Ambient IoT TR conclusion update”) from SA2, as below.

1. Overall Description: SA2 has discussed the open issues listed in TR 23.700-13(v1.2.1), clause 8 in SA2#166-AHe meeting, and reached additional conclusions, which may be useful for the future work of other working groups. Details of latest TR conclusion can be found in the attached file (link of SA2 TR). In addition, SA2 has discussed the following issues, and kindly request the other working groups to provide feedback. [Question to RAN3] SA2 has concluded for topology 1, the AIOTF selects the AIOT RAN node(s) and optionally provides a list of the BS readers to AIOT RAN. SA2 assumes that OAM can configure the AIOTF with needed information for this selection. SA2 have further discussed whether the AIOT RAN may provide the AIOT RAN information e.g., supported AIOT RAN serving area, supported BS reader ID list and corresponding BS reader serving area to the core network. SA2 kindly asks RAN3 to clarify whether RAN3 plan to support the above AIOT RAN information exchange with AIOTF. If so, SA2 kindly ask RAN3 to provide the definition of the appropriate parameters. 2. Actions: To RAN3 ACTION: SA2 asks RAN3 to provide feedback to SA2 question.

This paper addresses to answer the question from SA2.

- Discussion

SA2’s agreement

SA2 has concluded for topology 1, the AIOTF selects the AIOT RAN node(s) and optionally provides a list of the BS readers to AIOT RAN.

Based on the above agreement, for an AIoT handling procedure (e.g., Inventory procedure), the AIOTF shall select a target AIoT RAN node, i.e., the AIoT sends AIoT handling request message (e.g., Inventory request message) to the target AIoT RAN node.

It means that the AIOTF shall acquire the AIOT RAN serving area (i.e., AIoT handling area information) of each AIoT RAN node, the AIOT RAN serving area is to perform AIoT handling procedure, it can be e.g., a warehouse.

Figure 1: AIOTF selects target AIoT RAN node based on the pre-acquired {RAN node id, RAN serving area} pair.

Observation 1：AIOTF shall pre-acquire the AIOT RAN serving area of each AIoT RAN node.

Besides, SA2 agreed that, AIOTF may optionally provide a list of the reader identifiers within the AIoT handling request message, i.e., the AIOTF can provide the suggested reader(s) to the AIoT RAN for AIoT handling procedure.

It also means that the AIOTF shall acquire the reader’s serving area of each reader provided by AIoT RAN node.

Figure 2: AIOTF selects target AIoT RAN node and suggested reader based on the pre-acquired {RAN node id, RAN serving area} pair and {reader id, reader serving area} pair.

Observation 2: AIOTF shall pre-acquire the reader’s serving area of each reader.

SA2’s assumption

SA2 assumes that OAM can configure the AIOTF with needed information for this selection.

In R19 WI, the T1 reader is one kind of network node, e.g., RAN node, BS node, TRP, etc.

From RAN3’s point of view, when a node (either AIoT RAN or T1 reader) is deployed, its serving areas is not changed. So, for T1, both AIoT RAN and reader have the static configured AIoT serving areas.

As legacy, the configuration of network node is static and normally handled by OAM. On the other hand, NGAP signalling is normally used for dynamic or semi-static configuration.

In conclusion, RAN3 agree with SA2’s assumption, i.e., OAM can configure the AIOTF with needed information for this selection.

Proposal 1：RAN3 agrees to exchange the AIOT RAN information (used for reader selection) with AIOTF by OAM.

Definition of the appropriate parameters

SA2 has further discussed whether the AIOT RAN may provide the AIOT RAN information e.g., supported AIOT RAN serving area, supported BS reader ID list and corresponding BS reader serving area to the core network.

According to above analysis, in order for AIOTF to select target AIoT RAN node and to provide the suggested readers, the AIoT RAN shall provide the following AIoT RAN information to the AIOTF:

- a list of {AIoT RAN node identifier, its corresponding serving area} pair

- a list of {suggested reader identifier, its corresponding serving area} pair

Note: node identifier and its corresponding serving area are configured by OAM.

Note: reader is one kind of network node, e.g., BS, TRP.

Proposal 2：The detail of AIOT RAN information is as below.

- a list of {AIoT RAN node identifier, its corresponding serving area} pair

- a list of {suggested reader identifier, its corresponding serving area} pair

Note: node identifier and its corresponding serving area are configured by OAM.

Note: reader is one kind of network node, e.g., BS, TRP.

- Conclusion

Based on the above analysis, we provide the following observations and proposals.

Observation 1: AIOTF shall pre-acquire the AIOT RAN serving area of each AIoT RAN node.

Observation 2: AIOTF shall pre-acquire the reader’s serving area of each reader.

Proposal 1：RAN3 agrees to exchange the AIOT RAN information (used for reader selection) with AIOTF by OAM.

Proposal 2：The detail of AIOT RAN information is as below.

- a list of {AIoT RAN node identifier, its corresponding serving area} pair

- a list of {suggested reader identifier, its corresponding serving area} pair

Note: node identifier and its corresponding serving area are configured by OAM.

Note: reader is one kind of network node, e.g., BS, TRP.

We also provide the corresponding reply LS to SA2.