3GPP TSG-RAN WG3 Meeting #127	R3-250139

Athens, Greece, 17-21 Feb 2025

Agenda Item:	11.2

Source:	ZTE Corporation

Title:	Discussion on AI/ML assisted Network Slicing

Document for:	Discussions & Approval

- Introduction

This contribution aligns with the objectives [1] of the Rel-19 WI on 'Enhancements for AI/ML in NG-RAN.' Based on the conclusions and progress made in AI/ML-assisted Network Slicing, as captured in TR 38.743 [2], we present our analysis of the normative work for the network slicing use case. Additionally, the corresponding TP is included.

- Discussion

  - Slice Measurement Initiation Result

In the last meeting, we approved the introduction of a new request indication in the DATA COLLECTION REQUEST message: predicted slice available capacity and slice ID, to request slice-level AI/ML-related information. At the same time, we acknowledged that if the Report Characteristics indicates predicted resource status (First Bit) and slice ID is involved in the DATA COLLECTION REQUEST message, NG-RAN node 2 shall report both predicted resource status per cell and predicted resource status per slice.

Following the R18 partial reporting principle, we also need to include failure reporting information related to slice information in the response message. The DATA COLLECTION RESPONSE message is used to indicate which requested information failed to be initiated. Now, with the introduction of slice-level information, the response message also needs to include an indication of the requested slice-level information that cannot be reported.

Since the current design indicates a request for predicted resource status per cell or per slice using the First Bit (Predicted Resource Status), with the distinction being whether slice ID information is included, the same principle will be applied in the response message. A new Slice Measurement Initiation Result List IE will be introduced in the DATA COLLECTION RESPONSE message, under the existing Cell Measurement Initiation Result List. If the slice-level radio resource status cannot be reported, the Slice Measurement Initiation Result List should be included, and the First Bit should be indicated.

- Introduce Slice Measurement Initiation Result IE to indicate the predicted radio resource status per slice or predicted slice available capacity can not be initiated.

Possible Implementation:

| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| >> Slice Measurement Initiation Result List | O |  | 9.2.3.x2 | List of measurement objects that failed to be initiated per slice. |  |  |

The Slice Measurement Initiation Result List includes the S-NSSAI Measurement Failed Report Characteristics for each slice per PLMN. Two bitmaps are introduced in the S-NSSAI Measurement Failed Report Characteristics: Predicted Radio Resource Status and Predicted Slice Available Capacity.

- The newly introduced Slice Measurement Initiation Result List includes the S-NSSAI Measurement Failed Report Characteristics for each slice per PLMN.

  - Per Slice UE performance

In Rel-18, RAN3 agreed to introduce UE-level performance feedback to evaluate the performance of decisions made after handover. Currently, UE performance feedback includes metrics such as Average UE Throughput (UL/DL), Average Packet Delay(UL/DL), and Average Packet Loss (DL). During the Rel-19 SI phase, it was acknowledged that finer granularity UE performance feedback should be introduced for specific slice.

As described in TS 28.552 [3], average packet delay UL/DL, average UE throughput (UL/DL),and average packet loss (DL) can be calculated per PLMN ID and per QoS level and per supported S-NSSAI.

5.1.1.1	Packet Delay

5.1.1.1.1	Average delay DL air-interface

a)	This measurement provides the average (arithmetic mean) time it takes for packet transmission over the air-interface in the downlink direction. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

5.1.1.1.3	Average delay UL on over-the-air interface

a)	This measurement provides the average (arithmetic mean) over-the-air packet delay on the uplink. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

5.1.1.3	UE throughput

5.1.1.3.1	Average DL UE throughput in gNB

a)	This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI, and subcounters per PLMN ID, and subcounters per BWP. In the case of per BWP, the UE data volume refers to the total volume scheduled for each Active BWP with same bandwith except UEs with activated supplemental aggregated carrier(s).

5.1.1.3.3	Average UL UE throughput in gNB

a)	This measurement provides the average UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI, and subcounters per PLMN ID, and subcounters per BWP. In the case of per BWP, the UE data volume refers to the total volume scheduled for each Active BWP with same bandwith except UEs with activated supplemental aggregated carrier(s).

5.1.3.1.3	DL F1-U Packet Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at the gNB-DU). It is a measure of the DL packet loss on the F1-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3), and subcounters per supported S-NSSAI.

- Average packet delay UL/DL, average UE throughput (UL/DL),and average packet loss (DL) can be calculated per PLMN ID and per QoS level and per supported S-NSSAI in the SA5 specification.

When a UE initiates a PDU Session, the UE selects an appropriate S-NSSAI based on the application requirements to specify the needed network slice. The network uses the S-NSSAI to determine which slice is best suited to carry the PDU Session. For example, if a PDU Session is intended for eMBB services, it might select an S-NSSAI that identifies a network slice specifically configured for high-bandwidth, low-latency services. A PDU Session is mapped to a specific network slice.

Therefore, following option is proposed to reflect the slice-level UE performance:

-	Option 1: UE performance per S-NSSAI per PDU Session

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PDU Session List |  | 0..1 |  |  |
| >PDU Session Item |  | 1..<maxnoofPDUSessions> |  |  |
| >PDU Session ID | M |  | 9.2.3.18 |  |
| >S-NSSAI |  |  |  |  |
| >Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| >Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| >Average Packet Delay | O |  | 9.2.3.187 |  |
| >Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

-	Option 2: UE performance per QoS Flow per PDU Session

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PDU Session List |  | 0..1 |  |  |
| >PDU Session Item |  | 1..<maxnoofPDUSessions> |  |  |
| >PDU Session ID | M |  | 9.2.3.18 |  |
| >QoS Flow List |  | 1 |  |  |
| >>QoS Flow Item |  | 1..<maxnoofQoSFlows> | 9.2.3.10 |  |
| >>QoS Flow ID |  |  |  |  |
| >>Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| >>Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| >>Average Packet Delay | O |  | 9.2.3.187 |  |
| >>Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

-	Option 3: UE performance per S-NSSAI

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Slice UE performance List |  | 0..1 |  |  |
| > Slice UE performance Item |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 |  |
| >S-NSSAI UE performance List |  | 1 |  |  |
| >>S-NSSAI UE performance Item |  | 1..<maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| >>>Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| >>>Average Packet Delay | O |  | 9.2.3.187 |  |
| >>>Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

- UE performance per S-NSSAI can be considered to reflect the slice UE performance as a starting point.

3	Conclusion

Following is our proposals and observations:

Predicted information exchange:

- Introduce Slice Measurement Initiation Result IE to indicate the predicted radio resource status per slice or predicted slice available capacity can not be initiated.

- The newly introduced Slice Measurement Initiation Result List includes the S-NSSAI Measurement Failed Report Characteristics for each slice per PLMN.

Per Slice UE performance:

- Average packet delay UL/DL, average UE throughput (UL/DL),and average packet loss (DL) can be calculated per PLMN ID and per QoS level and per supported S-NSSAI in the SA5 specification.

- UE performance per S-NSSAI can be considered to reflect the slice UE performance as a starting point.

Reference

[1] RP-242385, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, ZTE Corporation, NEC

[2] TR 38.743

[3] TS 28.552