3GPP TSG-RAN WG3 Meeting #127	R3-250145

Athens, Greece, 17-21, Feb, 2025

Agenda Item:	20.2

Source:	ZTE Corporation

Title:	[Draft LS] Reply LS to on LMF-based AI/ML Positioning for case 3b

Document for:	Discussions & Approval

- Introduction

SA2 initiates the LS on LMF-based AI/ML positioning for case 3b to RAN WGs for the purpose to resolve the Editor’s Note in the TS 23.273 v19.1.0. There are three Editor’s Notes to be resolved based on RAN3’s feedback.

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

Editor's note:	Whether and how the LMF takes the RAN load into account in this procedure is FFS.

In this contribution, we provide our understanding on these three Editor’s Notes, and provide the reply LS to SA2.

2	Discussion

Following is the Editor’s Notes pointed out in the SA2’s LS to be resolved based RAN3’s feedback:

1st Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs. 2nd Editor's note: The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback. 3rd Editor's note: Whether and how the LMF takes the RAN load into account in this procedure is FFS.

Case 3b is NG-RAN node-assisted positioning with an LMF-side model, utilizing direct AI/ML positioning. In this case, the model is deployed on the LMF side, where both model training and model inference take place. The LMF needs to obtain measurements from the NG-RAN node as input. In the previous meeting, RAN3 has agreed to re-use the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1.

Therefore, in response to the Editor's Note: "What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation needs coordination with RAN WGs.", RAN3 can reply:

"The collection of input data for LMF-based AI/ML Positioning in Case 3b can be achieved by reusing the Measurement procedure defined in NRPPa, which supports requesting and reporting the relevant measurements to the LMF for Case 3b. And the details of the measurements for case 3b are still pending and subject to further discussion in RAN1, which is out of RAN3 scope. "

Proposal 1: In response to 1st Editor’s Note, "the collection of input data for LMF-based AI/ML Positioning in Case 3b can be achieved by reusing the Measurement procedure defined in NRPPa, which supports requesting and reporting the relevant measurements to the LMF for Case 3b. And the details of the measurements for case 3b are still pending and subject to further discussion in RAN1, which is out of RAN3 scope. "

The 2nd Editor Note exists in the clause 6.22.3 “Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements”. In Case 3b, when the model is deployed on the LMF side, the NG-RAN node only needs to provide Part A measurements to the LMF. From the perspective of RAN3, it is not distinguished over interface whether the requested data is used for model training or model inference. Therefore, the LMF only needs to reuse the legacy positioning request and reporting procedure (e.g., Measurement Information Transfer) to obtain the measurement data from the NG-RAN node.

Proposal 2: In response to 2nd Editor’s Note, "LMF reuses the legacy positioning request and reporting procedure (e.g., Measurement Information Transfer) to obtain the measurement data from the NG-RAN node. "

The third Editor's Note concerns the notification of RAN load for LMF's consideration, which was not discussed in RAN3. In the legacy positioning procedures, the LMF is responsible for initiating TRP request message, SRS configuration request message, or measurement request message to the NG-RAN node. However, it does not inquire about the NG-RAN node's load status.

If the NG-RAN node is under high load and unable to perform the positioning, it can respond with a failure message during one of the response processes and include a cause value, e.g., Request Item Temporarily not Available.

===============================TS38.455===============================

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Radio Network Layer related |
| Requested Item not Supported | The NG-RAN node does not support the requested measurement object, or cannot provide the requested information item. |
| Requested Item Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement object or information item. |
| Serving NG-RAN node changed | The UE has moved to another serving NG-RAN node. |
| Requested Item not Supported on Time | The NG-RAN node is unable to provide the measurement results on time. |

===============================TS38.455===============================

Similar to the already standardized AI/ML for NG-RAN over the Xn interface, when requesting AI/ML-related information from a neighboring node, there is no prior inquiry about whether the neighboring node has the capability to provide the information or the resources to perform the computation. If the neighboring node cannot provide the requested information, it simply responds with a failure message.

- There is no prior inquiry about whether the NG-RAN node has the capability to provide the information or the load resources to perform the measurement and calculation. The existing cause value "Requested Item Temporarily Not Available" can cover the situation where the NG-RAN node cannot perform the measurements due to high load.

Furthermore, for model training, the target data can be of various types and is not necessarily urgent. It is not mandatory to request data from a highly loaded NG-RAN node, as data can be obtained from other base stations. If a base station is under high load now, it is possible to first inquire with other nodes or wait until the current node has a lower load before requesting data from it.

Proposal 3: In response to the 3rd Editor’s Note, "from RAN3’s perspective, there is no need for the LMF to consider the RAN load when performing AI/ML direct positioning (Case 3b). "

The draft reply LS reflecting the proposals above is provided in the Section 5.

3	Conclusion

Following is our proposals and observation:

Proposal 1: In response to 1st Editor’s Note, "the collection of input data for LMF-based AI/ML Positioning in Case 3b can be achieved by reusing the Measurement procedure defined in NRPPa, which supports requesting and reporting the relevant measurements to the LMF for Case 3b. And the details of the measurements for case 3b are still pending and subject to further discussion in RAN1, which is out of RAN3 scope. "

Proposal 2: In response to 2nd Editor’s Note, "it is not distinguished over interface whether the requested data is used for model training or model inference from RAN3 perspective, and LMF reuse the legacy positioning request and reporting procedure (e.g., Measurement Information Transfer) to obtain the measurement data from the NG-RAN node. "

- There is no prior inquiry about whether the NG-RAN node has the capability to provide the information or the load resources to perform the measurement and calculation. The existing cause value "Requested Item Temporarily Not Available" can cover the situation where the NG-RAN node cannot perform the measurements due to high load.

Proposal 3: In response to the 3rd Editor’s Note, "from RAN3’s perspective, there is no need for the LMF to consider the RAN load when performing AI/ML direct positioning (Case 3b). "

Proposal 4: Agree the draft reply LS in the Section 5.

4	Reference

[1] R3-250074, LS on LMF-based AI/ML Positioning for case 3b, SA2

5	Draft Reply LS to SA2

3GPP TSG-RAN WG3 Meeting #127	R3-250145

Athens, Greece, 17 – 21 Feb 2025

Title:	[Draft] Reply LS on LMF-based AI/ML Positioning for case 3b

Response to:	R3-250074 (S2-2501341)

Release:	Release 19

Work Item:	AI/ML_CN, NR_AIML_Air, NR_AIML_air-Core

Source:	ZTE Corporation (to be RAN3)

To:	SA2

Cc:	RAN1, RAN2

Contact person:	Chen Jiajun

Chen.jiajun1@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN3 thanks SA2 for the reply LS on LMF-based AI/ML Positioning for case 3b in R3-250074/ S2-2501341.

The feedback from RAN3’s perspective is provided separately for each of the following Editor’s Notes.

- Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

In response to the 1st Editor’s Note, "the collection of input data for LMF-based AI/ML Positioning in Case 3b can be achieved by reusing the Measurement procedure defined in NRPPa, which supports requesting and reporting the relevant measurements to the LMF for Case 3b. And the details of the measurements for case 3b are still pending and subject to further discussion in RAN1, which is out of RAN3 scope. "

- Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

In response to 2nd Editor’s Note, "LMF reuses the legacy positioning request and reporting procedure (e.g., Measurement Information Transfer) to obtain the measurement data input data from the NG-RAN node. "

- Editor's note:	Whether and how the LMF takes the RAN load into account in this procedure is FFS.

In response to the 3rd Editor’s Note, " There is no prior inquiry about whether the NG-RAN node has the capability to provide the information or the load resources to perform the measurement and calculation. The existing cause value "Requested Item Temporarily Not Available" can cover the situation where the NG-RAN node cannot perform the measurements due to high load. From RAN3’s perspective, there is no need for the LMF to consider the RAN load when performing AI/ML direct positioning (Case 3b). "

# 2	Actions

To SA2

ACTION: 	RAN3 kindly ask SA2 to take information above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#127bis			2025-04-07 – 2025-04-11				China, CN

RAN3#128				2025-05-19 – 2025-05-23				Malta, MT