3GPP TSG-RAN WG3 Meeting #127	 R3-250166

Athens, Greece, Feb 17-21, 2025

Agenda Item:	18.2

Source: 	NEC

Title:  	UE Identity Index Value for LP-WUS subgroup

Document for:	Discussion and Decision

- 1 Introduction

In last RAN3#126 meeting, the following agreements are made:

WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring for RRC_IDLE and RRC_INACTIVE.

NG-RAN node can acquire the total number of CN assigned subgrouping via OAM.

In last RAN2#128 meeting, the following agreements are made:

FFS on the following options

- - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

- - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed.

- Option 3: The formula for UE_ID based PEI subgrouping is reused.

In this contribution, we further study whether need to extend UE Identity Index Value.

- 2 Discussion

- 2.1 UE Identity Index Value

The IE – (extended) UE Identity Index Value is used by the target NG-RAN node to calculate the Paging Frame and Paging Occasion for either normal DRX or eDRX, and calculate UE_ID based PEI subgroup ID. And it represents UE_ID in TS 38.304.

Before discussion, let’s take a look at the paging formula:

The PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

UE_ID:

If the UE operates in eDRX as specified in clause 7.4:

- 5G-S-TMSI mod 4096

else:

- 5G-S-TMSI mod 1024

It can be seen that for normal DRX scenarios, the UE_ID could be [0 ~ 1023], which means 10 bits are needed to represent UE_ID. For CN paging, the gNB can acquire 5G-S-TMSI from NGAP PAGING message and directly calculate UE_ID, while for RAN paging, the anchor gNB is not aware of UE’s 5G-S-TMSI (I-RNTI is used instead), in order to calculate paging occasion, the anchor gNB need to acquire UE_ID from Core Network Assistance Information for RRC INACTIVE sent by CN and this message includes 10 bit UE Identity Index Value. When it comes to RAN paging exchange between anchor gNB and other gNB, i.e., XnAP RAN PAGING message, the anchor gNB also need to inform other gNBs of UE_ID (i.e., UE Identity Index Value). Therefore this is the usage of UE Identity Index Value. It is also noted that the current UE_ID could be [0 ~ 8191] for PEI UE-ID based subgroup and could be [0 ~ 32767] for eDRX case, which means more bits are needed to represent UE_ID, that is reason why Extended UE Identity Index Value (16 bits) is introduced.

Back to LP-WUS discussion, RAN2 has agreed that the following options for LP-WUS subgroup will be down selected:

- Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

- Option 2: The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed.

- Option 3: The formula for UE_ID based PEI subgrouping is reused.

Option 1 and 2

the LP-WUS subgroup formula is shown as below :

SubgroupID = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID),

Where K = the subgrouping number for UE_ID based PEI subgrouping for option-1.

Where K = fixed or configured value (e.g., 8) for option-2.

Where UE-ID = 5G-S-TMSI mod X

In this formula, again, UE-ID should be 5G-S-TMSI mod X, we need to find out what value is X.

Based on RAN1 agreement, the maximum number of subgroups per PO supported in Rel-19 is 32, also, the maximum subgroup number for PEI is 8, which means the maximum K should be 8. In order to fully use UE-ID, for normal DRX case, the max UE-ID value should be N*Ns*K* subgroupsNumForUEID (i.e., 256*4*8*32), i.e., the UE-ID can be [0 ~ 262143], X = 262144. Therefore, 18 bits are needed in UE Identity Index Value. For eDRX case, N is extended and 20bits are needed.

Option 3

PEI subgroup formula is reused for LP-WUS subgroup formula.

SubgroupID = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID),

Where max subgroupsNumForUEID = 32

Where UE-ID = 5G-S-TMSI mod X

For normal DRX case, the max UE-ID value should be N*Ns* subgroupsNumForUEID (i.e., 256*4*32), i.e., the UE-ID can be [0 ~ 32767], X = 32768. Therefore, 15 bits are needed in UE Identity Index Value which means the legacy extended UE Identity Index Value is already enough. But for eDRX case, 17 bits are needed, which means the legacy extended UE Identity Index Value is not enough. It is noted that currently there is no agreement about whether to support eDRX with LP-WUS.

Based on the above analysis, we can see from that for option 1 and 2, the legacy extended UE Identity Index Value (i.e., 16bits) is not enough to calculate UE-ID for LP-WUS subgroup, new extended UE Identity Index Value, i.e., 18 bits without eDRX and 20 bits with eDRX should be introduced. For option 3, the legacy extended UE Identity Index Value is enough to calculate UE-ID for LP-WUS subgroup, but if eDRX is supported, new extended UE Identity Index Value is still needed.

Proposal-1: if RAN2 go for option 1 and 2 for LP-WUS subgroup, new extended UE Identity Index Value, i.e., 18 bits without eDRX and 20 bits with eDRX should be introduced.

Proposal-2: if RAN2 go for option 3 for LP-WUS subgroup, legacy extended UE Identity Index Value is enough if eDRX is not supported with LP-WUS. But if eDRX is supported, new extended UE Identity Index Value is still needed.

- 3 Conclusions

Proposal-1: if RAN2 go for option 1 and 2 for LP-WUS subgroup, new extended UE Identity Index Value, i.e., 18 bits without eDRX and 20 bits with eDRX should be introduced.

Proposal-2: if RAN2 go for option 3 for LP-WUS subgroup, legacy extended UE Identity Index Value is enough if eDRX is not supported with LP-WUS. But if eDRX is supported, new extended UE Identity Index Value is still needed.

- 4 References