3GPP TSG-RAN WG3 Meeting #127	R3-250169

Athens, Greece, 17th – 21st February 2025

Agenda Item:		11.2

Source:		NEC

Title:	UE performance feedback for AI/ML-based Slicing

Document for:		Discussion and decision

# Introduction

In RAN3#126 meeting, UE performance feedback for AI/ML-based slicing was discussed and the following agreement was achieved.

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

However, there is no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node. One LS to SA5 [1] was sent out to ask SA5 to provide feedback on the granularity of UE performance metrics specified in TS 28.558, whether per-UE or per-DRB.

In this contribution, we further discuss this issue and provide our considerations on the stage 3 impact.

# Discussion

As specified in TS38.423, UE performance measurements metrics for AI/ML for NG-RAN includes Average UE throughput DL, Average UE throughput UL Average packet delay and Average packet loss DL, and the UE performance feedback can be collected at node level or cell level in legacy.

In order to perform optimal resource management and mobility decisions for network slicing, we already agreed that slice level radio resource status and slice available capacity are exchanged between nodes. As for the UE performance feedback, slice level UE performance feedback should be the target metric and it has more advantages to evaluate and optimize the model performance.

In the agreed CR [2] in last RAN3 meeting, a Slice To Report List for Data Collection IE is added in the DATA COLLECTION REQUEST message to indicate the S-NSSAI list to which the request applies.

In order to reflect the above agreement “Source node can request per UE per S-NSSAI granularity for UE performance feedback” in specification, in TS38.423, a Reporting Granularity IE can be added in UE Performance Collection Configuration IE to indicate the granularity for which the UE performance is required. The receiving NG-RAN node shall take this into account for the configuration of UE performance collection.

The detailed IE change in TS38.423 is shown as below.

#### 9.2.3.186	UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond |
| Reporting Granularity | M |  | ENUMERATED(per UE, per S-NSSAI, …) | Granularity for which the UE performance is required. |

Proposal 1: To add a Reporting Granularity IE in UE Performance Collection Configuration IE in TS38.423.

According to TS28.558, all the existing UE level measurements can be calculated per QoS level and per supported S-NSSAI, which means slice level UE performance feedback already can be supported by legacy. If this can be confirmed by SA5, the receiving NG-RAN node shall collect the UE measurements per requested S-NSSAI and report the UE performance collection in DATA COLLECTION UPDATE message. Therefore, in DATA COLLECTION UPDATE message, a Slice Associated Info Result Item IE needs to be added under the UE Associated Info Result List IE. In this new IE, the reported S-NSSAI and its per S-NSSAI UE Performance and are included.

The detailed IE change in TS38.423 is shown as below.

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and optionally includes the Slice Radio Resource Status List IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| >>Predicted Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| >>> Slice Associated Info Result Item |  | 0 .. < maxnoofSliceItems > |  |  | – |  |
| >>>> S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>>>UE Performance | M |  | 9.2.3.179 |  | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

Proposal 2: To add a Slice Associated Info Result Item IE under the UE Associated Info Result List IE in DATA COLLECTION UPDATE message in order to report the S-NSSAI and its per S-NSSAI UE Performance.

Regarding QoS flow level UE performance feedback, the main concerns is that, after handover, the target node may assigning different QoS flows to a specific slice and so the performance for each QoS flow is not comparable. The only exception is that if all the QoS flows mapped to one slice have the same QoS parameters, then QoS flow level is applicable since it can be reflected by slice level. Therefore, QoS flow level UE performance feedback can be used as an optional choice for reference.

Proposal 3: To support QoS flow level UE performance feedback as an optional choice for reference for AI/ML-based network slicing.

# Proposals

In this contribution, we provide our analysis on stage 3 impact of UE performance feedback for AI/ML-based slicing and a corresponding TP to BLCR TS38.423.

Proposal 1: To add a Reporting Granularity IE in UE Performance Collection Configuration IE in TS38.423.

Proposal 2: To add a Slice Associated Info Result Item IE under the UE Associated Info Result List IE in DATA COLLECTION UPDATE message in order to report the S-NSSAI and its per S-NSSAI UE Performance.

Proposal 3: To support QoS flow level UE performance feedback as an optional choice for reference for AI/ML-based network slicing.

# Reference

[1] R3-247775, LS on per-UE UE performance metrics

[2] R3-247928, (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN