3GPP TSG-RAN WG3 Meeting #127	R3-250176

Athens, Greece, 17th – 21st February 2025

Agenda Item:		17.3

Source:		NEC

Title:			Support on-demand SIB1

Document for:		Discussion and decision

# Introduction

In RAN3#126 meeting, two BLCRs [1, 2] were endorsed to introduce one new class 1 procedure for providing UL WUS Configuration Information from NES Cell gNB to Cell A gNB and one new class 2 procedure for configuration update from Cell A gNB to NES Cell gNB over Xn interface, and to add UL WUS Configuration Information in F1 Setup message and gNB-DU Configuration Update message over F1 interface.

This contribution further discusses the RAN3 impact of OD SIB1 based on the agreed basic procedures.

# Discussion

## UL WUS configuration provision

Regarding the new defined class 1 procedure and the new class 2 procedure, one FFS is the names of the procedures. Since the purpose of the class 1 procedure is to provide UL WUS Configuration Information from NES Cell gNB to Cell A gNB, it is suitable to name it as “UL WUS Configuration Provision” procedure, and the three corresponding messages as “UL WUS Configuration Provision Request”, “UL WUS Configuration Provision Response” and ““UL WUS Configuration Provision Failure”.

Proposal 1: To name the class 1 procedure in TS 38.423 [3] as “UL WUS Configuration Provision” procedure, and the two corresponding messages as “UL WUS Configuration Provision Request” and “UL WUS Configuration Provision Response” (to remove FFS).

Regarding the new class 2 procedure, according to the agreement made in last meeting, “A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB”, therefore, we suggest to directly name the procedure as “UL WUS configuration Broadcast Stop Indication”.

Proposal 2: To rename the class 2 procedure and the corresponding message in TS 38.423 [3] as “UL WUS configuration Broadcast Stop Indication”.

As agreed in RAN#105 meeting, for Rel-19 NES, support Case-2 on OD SIB1:

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

There is one possible scenario for Case 2 that, for one NES Cell, multiple Cell A are applicable. In this case, the NES Cell can choose which Cell A for UL WUS configuration provision. It can choose only one or several or all Cell A from the applicable Cell A. The UL WUS Configuration Provision Request sent to different Cell A can be sent one by one or at the same time, this can be left to implementation. If only one response received from the requested Cell A, the NES Cell can activate OD SIB1.

Proposal 3: In case of multiple Cell A appliable for one NES Cell, the NES Cell can choose which Cell A for UL WUS configuration provision.

Proposal 4: If only one WUS configuration provision Response received from the requested Cell A, the NES Cell can activate OD SIB1.

## OD SIB1 activation/deactivation

The following three options for OD SIB1 activation was proposed and discussed:

- Option 1: After the UL WUS Configuration Provision request and Positive Response (confirm), NES Cell gNB goes to OD SIB1 Mode. Cell A gNB broadcasts UL WUS configuration via its SIB.

- Option 2: After the UL WUS Configuration Provision request and positive response (confirm), Cell A gNB broadcast received UL WUS configuration via its SIB. NES Cell goes to OD-SIB Mode upon reception of an explicit “UL config provided” message from Cell A gNB.

- Option 3: After the UL WUS Configuration Provision request and positive response (confirm), no change in NES Cell or Cell A gNBs. NES Cell goes to OD-SIB Mode and Cell A gNB broadcasts received UL WUS configuration via its SIB upon transmission/reception of an explicit “UL WUS configuration broadcast indication” message from NES Cell gNB.

Option 1 was agreed to adopt in last meeting, here we further discuss Option 2 and Option 3. Comparing to Option 1, the difference for Option 2 and Option 3 is, after UL WUS Configuration Provision procedure, whether Cell A or NES Cell needs another indication to start UL WUS configuration broadcast or to activate OD SIB1.

After the NES Cell provide its UL WUS configuration to cell A, the NES Cell can update Cell A if the current UL WUS configuration is changed or released. Meanwhile, the NES Cell can activate or deactivate (switch to legacy way) OD SIB1 transmission at any time. If the NES Cell is in SIB1 broadcast, the UE can receive SIB1 in legacy way; if the NES Cell is in OD SIB1, the UE will request cell A for the UL WUS configuration first and then trigger the NES Cell to send OD SIB1. Besides, RAN2 already agreed that “If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell”. Therefore, we think it is no need to synchronize between OD SIB1 in NES Cell and UL WUS configuration broadcast in Cell A, i.e., as stated in Option 1, after the UL WUS Configuration Provision request and Positive Response (confirm), NES Cell can go to OD SIB1 Mode at any time and Cell A can start to broadcast UL WUS configuration via its SIB at any time.

If NES Cell already goes to OD SIB1 mode but Cell A has not started to broadcast UL WUS configuration, the NES Cell is regarded as barred for UE. As normally NES Cell is just used for capacity boost and there is always another coverage cell for UE, there will have not much impact on UE even if the NES Cell is not applicable for a period of time.

If Cell A has started to broadcast UL WUS configuration but NES Cell is still in legacy SIB1 broadcast mode, as RAN2 agreed, UE will check the NES Cell first and use the legacy SIB1 broadcasted from NES Cell without requesting OD SIB1. Considering RAN2 agreed that “NES Cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration)”, the new SIB for UL WUS configuration can include UL WUS configuration of multiple NES Cells, so the signaling waste for broadcasting UL WUS configuration for a NES Cell which is not in OD SIB1 mode is negligible.

Therefore, we think no further activation indication for OD SIB1 after no matter from Cell A or NES Cell is needed.

Proposal 5: A further activation indication after UL WUS Configuration Provision procedure no matter from Cell A or NES Cell is not needed.

If NES Cell needs to inform Cell A about the UL WUS configuration update or release, the UL WUS Configuration Provision procedure can also be used to transmit an updated UL WUS configuration or a release indication.

Proposal 6: UL WUS Configuration Provision procedure can be used for UL WUS configuration update or release.

Proposal 7: To add a release indication in the UL WUS Configuration Provision Request message for UL WUS configuration release.

While considering the OD SIB1 activation/deactivation change can be dynamic and frequent, NES Cell may not need to inform cell A about the OD SIB1 activation or deactivation.

Proposal 8: NES Cell does not need to inform cell A about the OD SIB1 activation or deactivation.

As descripted above, there can be a scenario i.e. multiple NES Cells under a single Cell A, it is beneficial to perform some coordination by Cell A on e.g. which NES Cells under that Cell A can activate OD SIB1 transmission, taking both energy saving and UE performance into account.

Proposal 9: Cell A can provide guidance to assist NES Cell on OD SIB1 activation/deactivation.

In split architecture, considering gNB-CU is more knowledgeable on the SIB1 transmission status and load status of all NES Cells and neighbor cells, we prefer gNB-CU to decide OD SIB1 activation/deactivation of a NES Cell, or at least gNB-CU should provide guidance to assist gNB-DU on OD SIB1 activation/deactivation of a NES Cell.

Proposal 10: gNB-CU decides or assists gNB-DU on the OD SIB1 activation/deactivation of a NES Cell, and sends the request or assistance information to gNB-DU.

# Proposals

In this contribution, we analysed OD SIB1 transmission of a NES Cell based on the latest agreements and the following proposals were drawn from the analysis.

Proposal 1: To name the class 1 procedure in TS 38.423 [3] as “UL WUS Configuration Provision” procedure, and the two corresponding messages as “UL WUS Configuration Provision Request” and “UL WUS Configuration Provision Response” (to remove FFS).

Proposal 2: To rename the class 2 procedure and the corresponding message in TS 38.423 [3] as “UL WUS configuration Broadcast Stop Indication”.

Proposal 3: In case of multiple Cell A appliable for one NES Cell, the NES Cell can choose which Cell A for UL WUS configuration provision.

Proposal 4: If only one WUS configuration provision Response received from the requested Cell A, the NES Cell can activate OD SIB1.

Proposal 5: A further activation indication after UL WUS Configuration Provision procedure no matter from Cell A or NES Cell is not needed.

Proposal 6: UL WUS Configuration Provision procedure can be used for UL WUS configuration update or release.

Proposal 7: To add a release indication in the UL WUS Configuration Provision Request message for UL WUS configuration release.

Proposal 8: NES Cell does not need to inform cell A about the OD SIB1 activation or deactivation.

Proposal 9: Cell A can provide guidance to assist NES Cell on OD SIB1 activation/deactivation.

Proposal 10: gNB-CU decides or assists gNB-DU on the OD SIB1 activation/deactivation of a NES Cell, and sends the request or the assistance information to gNB-DU.

# Reference

[1] R3-247897, Introduction of Network Energy Saving Enhancement

[2] R3-247898, Introduction of Network Energy Saving Enhancement

[3] TS 38.423, NG-RAN; Xn application protocol (XnAP)