3GPP TSG-RAN WG3 #127						         　　　　　　　　　　　　　　　　　　　　　 R3-250177

17th – 21st Feb 2025 Athens, Greece

Agenda Item:	8.1. New Incoming LSs

Source:	NEC

Title:	Discussion on MMSID-aware admission control and QoS flow to DRB mapping

Document for:	Discussion, Decision

Introduction

SA sent an LS reply on multi-modality awareness in R3-250032/SP-241921 (copied below) to response to LS (R2-2409272/S2-2411300, R3-245017/S2-2411302).

1. Overall Description: SA has discussed the RAN2(R2-2409272), RAN3(R3-245682) and SA4(S4-242223) LSs on Multimodality awareness. Based on the understanding that in R2-2409272, RAN2 agreed that multi-modality information (e.g., MMSID), can be used for joint admission control and QoS flow to DRB mapping and that RAN3 agreed, as reported in R3-245682, that it is technically feasible to enhance NGAP to provide the MMSID to NG-RAN node, SA has reached consensus that the multimodality awareness can be achieved in the RAN by the CN providing multimodality information (i.e., the MMSID) to the NG-RAN by N2 SM information. SA expects SA2 to handle the above LSs and provide related specification updates based on the principle described above. 2. Actions: To RAN, RAN2, RAN3, SA2: ACTION: SA kindly asks RAN, RAN2, RAN3 and SA2 to take the above guidance into account in order to achieve a 3GPP-wide coordinated approach to the support of multimodality.

Discussion and proposal

With the provided MMSID, RAN can jointly setup/release/modify multi-modality QoS flows associated with the same MMSID. For example, the NG-RAN node may jointly establish, modify or release the DRB configuration and may change allocation of resources on NG or Uu accordingly.

NG-RAN can jointly setup/release/modify multi-modality QoS flows based on the provided MMSID (i.e., jointly setup/release/modify multi-modality QoS flows with the same MMSID).

- References

[1] R3-250032