3GPP TSG-RAN WG3 #127	R3-250183

Greece, Athens, 17th – 21st February 2025

Agenda item:		16.2

Title:		Discussion on AIoT architecture aspects

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The new working item for Ambient IoT in NR is agreed upon in the RAN#106 meeting with the following scope in [1]:

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. The following objectives are set, within the General Scope: - RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

This contribution will discuss the architecture issues of Ambient IoT.

# 2. Discussion

2.1	The discussion of XX interface

This is the logical system architecture for topology 1, defined in TR 38.769, and the XX interface between AIoT RAN and AIoT CN is the NG interface; it is not clear whether "XXAP" is realised by including AIoTF information in the NGAP or by carrying a new protocol layer over the NGAP.

Figure 6.4.1-1 Logical system architecture for topology 1

The SA2 agreed that the “AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response” in the TR 23.700-13. And here are the protocol stacks from SA2:

Figure 8.1.2.2-3: Example Protocol Stack between AIOTF and AIoT Device for Topology 1 (Direct Path)

Figure 8.1.2.3-3: Example Protocol Between AIOTF and AIoT Device for Topology 1 (indirect Path via AMF)

It is clear that the AIoTF information (inventory and command requests and responses) is sent through the AIoT NAS layer, which is a protocol layer on top of NGAP and invisible to the AIoT RAN. Therefore, there is no doubt that sending the AIoTF information between AIoTF and AIoT RAN via AIoT NAS data transmission over NGAP, and the AIoTF information carried over AIoT NAS is transparent to the NGAP.

The AIoT RAN should be aware of some necessary assistance information, such as the inventory or command indication, the size of D2R data, the estimated number of devices, etc. We will discuss the content of the NAS data transmission and the AIoT assistant information in our next contribution.

Observation 1: SA2 has agreed that the “AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response”.

Proposal 1: The AIoTF information (inventory and command request/response) between AIoTF and AIoT RAN reader is carried by the AIoT NAS layer over NGAP, and the AIoTF information carried over AIoT NAS is transparent to the AIoT RAN.

2.2	Direct/indirect connection between AIoT RAN and AIOTF

Also, as we can see from the above two figures, the AIoTF is a new CN function to support AIoT service and the AIoTF can connect to AIoT RAN directly or indirectly via AMF. It says in TR 38.769 that "NOTE 3: How the functions represented by the XX interfaces are defined in various solutions and A-IoT CN topologies (direct/indirect connection between AIoT RAN and AIOTF)".

The down selection should be decided by SA2, but the decision has an extreme impact on the RAN3 workload, so we should discuss the down selection and provide our RAN3 opinion. In the case of an indirect connection, the NGAP described in TS 38.413 could be used and improved to help send AIoT messages, and the AMF would only act as a relay to send AIoT data. So, the working load is acceptable. For the direct connection case, RAN3 defines the XX interface between AIoT RAN and AIoT function, which is expected to be a new interface, and RAN3 has to design it from the beginning. Although some companies think the new interface should be very similar to the NG interface, it still takes a lot of time to define a new technical specification for it.

Considering we only have 4 meetings to finish this new AIoT WI for RAN3, we need to be careful about its workload. Therefore, we prefer to select the indirect connection between AIoTF and AIoT RAN via AMF in Rel-19. The indirect connection can be considered in further releases.

Observation 2: The direct connection between AIoTF and AIoT RAN requires RAN3 to define a new interface, whereas the indirect connection between AIoTF and AIoT RAN requires some NGAP enhancement.

Observation 3: RAN3 only has 4 meetings to finish this new AIoT WI in Rel-19.

Proposal: 2: We prefer to select the indirect connection between AIoTF and AIoT RAN via AMF in Rel-19. And the indirect connection can be considered in further releases.

2.3	Reader selection

Reader selection is studied in previous meetings, and the following text is captured in TR 38.769:

Reader selection may need coordination between A-IoT RAN node and A-IoT CN. NOTE 5: How to perform reader selection needs further discussions. NOTE 6: In Topology 1, whether the AIoTF needs to be aware of the readers served by the A-IoT RAN node and their location and how this information is provided to the AIoTF needs further discussion.

The SA2 also has some conclusions about the reader selection captured in TR 23.700-13:

3. For the requested operation the AIOTF: 1). Performs initial reader selection by either: - The AF providing information to identify readers to include in the initial reader selection network. The information can identify multiple or an individual Reader to use for the request. NOTE 1: Which readers are identified by the information from the AF is up to the network deployment, configuration or implementation. - If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader. - If reader selection information or UE Reader ID is not provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g. preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc. If no readers can be selected then the request is rejected.

The SA2 conclusion makes it very clear that the AF could decide the reader ID(s) and send them to the AIoTF; otherwise, the AIoTF selects readers on its own.

From the point of view of AIoTF, there are two implementations for select readers:

- Case 1: The AIoTF knows there is/are reader(s) that could be used (e.g., the AF indicates which reader to use, or the AF preconfigures the AIoTF which reader to use, or the AIoT RAN provides the AIoT service before as mentioned in the SA2 conclusion).

- Case 2: The AIoTF do not know which readers to use. (Maybe some assistance information is provided, e.g., AIoT service area).

For case 1, the AIoTF sending the inventory/command messages to corresponding reader(s) would trigger the AIoT reader providing AIoT service.

For case 2, the AIoTF needs to know if the interested RAN node (the RAN node in the AIoT service area) could be used as reader(s). Therefore, the AIoTF needs to send a message to ask if the interested RAN node has the capability to perform as a reader. If the interested RAN node feedback that it has the capability to be used as reader(s), the AIoTF sends the inventory/command messages to this RAN node. Otherwise, the AIoTF could check other nodes in the AIoT service area.

Proposal 3: If the AIoTF does not know which reader(s) could be used, the AIoTF needs to send a checking message to the interested RAN node to know whether it has the capability to be used as reader(s).

# 3. Conclusion

Observation 1: SA2 has agreed that the “AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response”.

Observation 2: The direct connection between AIoTF and AIoT RAN requires RAN3 to define a new interface, whereas the indirect connection between AIoTF and AIoT RAN requires some NGAP enhancement.

Observation 3: RAN3 only has 4 meetings to finish this new AIoT WI in Rel-19.

Proposal 1: The AIoTF information (inventory and command request/response) between AIoTF and AIoT RAN reader is carried by the AIoT NAS layer over NGAP, and the AIoTF information carried over AIoT NAS is transparent to the AIoT RAN.

Proposal: 2: We prefer to select the indirect connection between AIoTF and AIoT RAN via AMF in Rel-19. And the indirect connection can be considered in further releases.

Proposal 3: If the AIoTF does not know which reader(s) could be used, the AIoTF needs to send a checking message to the interested RAN node to know whether it has the capability to be used as reader(s).

# References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Huawei