3GPP TSG-RAN WG3 #127	R3-250184

Greece, Athens, 17th – 21st February 2025

Agenda item:		16.2

Title:		Discussion on AIoT procedure aspects

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The new working item for Ambient IoT in NR is agreed upon in the RAN#106 meeting with the following scope in [1]:

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. The following objectives are set, within the General Scope: - RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

This contribution will discuss the procedure issues of Ambient IoT.

# 2. Discussion

2.1	The discussion on the AIoT NAS protocol

In TR 38.769, the following procedures are agreed upon for topology 1:

Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1

Figure 6.5.2.2-1: Message flow for A-IoT Command in Topology 1

The procedures related to RAN3 are as follows: Inventory Request, Inventory Response, Inventory Report, Command Request and Command Response.

As we discussed in another contribution, the SA2 agreed that the “AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response” in the TR 23.700-13. And the AIoT NAS protocol is a protocol layer over NGAP, and the data carried over the AIoT NAS protocol is transparent to the NGAP.

Based on what we know, the NAS layer data should include more than just the Inventory Response and Command Request and Response from TR 23.700-13. It should also carry the Inventory Request from the AIoTF to the AIoT RAN node and the Inventory Report from the AIoT RAN node to the AIoTF.

In the current specification, we already have the NAS protocol between UE and the AMF, while the AIoT NAS layer is a protocol layer between AIoT devices and AIoTF. The transmitter and receiver of the AIoT NAS layer data is different from that of the legacy NAS layer data. And considering the AIoT devices have much lower complexity than the legacy UEs, the AIoT NAS layer protocol should be much simpler than the legacy NAS layer protocol. Therefore, the AIoT NAS protocol should be different from the legacy NAS protocol, and the legacy NAS layer data transportation messages defined in NGAP cannot be reused to transport AIoT NAS layer data. Therefore, RAN3 needs to define the AIoT NAS data transport procedure to transmit AIoT NAS layer data over NGAP where the AIoT NAS layer data is included in a transparent container. A downlink AIoT NAS data transport message to transmit AIoT NAS layer data from AIoTF to AIoT RAN node and an uplink AIoT NAS data transport message to transmit AIoT NAS layer data from AIoT RAN node to AIoTF should be defined.

Observation 1: The AIoT NAS protocol between the AIoT device and the AIoTF is introduced to carry the Inventory Request, Inventory Response, Inventory Report, Command Request and Command Response. And the AIoT NAS protocol is a protocol layer over the NGAP, and the data carried by the AIoT NAS protocol is transparent to the NGAP.

Proposal 1: RAN3 needs to define a new AIoT NAS data transport procedure to transmit AIoT NAS layer data over the NGAP where the AIoT NAS layer data is included in a transparent container over NGAP. A downlink AIoT NAS data transport message to transmit AIoT NAS layer data from the AIoTF to the AIoT RAN node and an uplink AIoT NAS data transport message to transmit AIoT NAS layer data from the AIoT RAN node to the AIoTF.

2.2	The discussion on assistance information

RAN3 has agreed to the following information exchange between the AIoT RAN node and the AIoT CN in TR 38.769:

#### 6.5.1.1 Inventory Inventory can be sent by the A-IoT CN for a single device, or a group of devices, or all devices. The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following: (1) A-IoT Device Identification (to find a single device, a group of devices, or all devices) NOTE 1: The definition of this identification is out of RAN3 scope. For topology 1, A-IoT RAN needs to store the A-IoT Device Identification received from the inventory request. NOTE 2: Whether to interpret and process AIoT Device Identification received from the inventory request needs further discussion. (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered) Multiple individual A-IoT Device IDs (one ID per device) can be provided to the A-IoT CN via a single Inventory Report. NOTE 3: For topology 1, whether device ID received from the device is sent transparently to the A-IoT CN by the A-IoT RAN depends on RAN2 and SA2. NOTE 4: Whether the description in this clause apply to A-IoT enabled gNB in topology 2 needs further discussion and is dependent on SA2 and RAN2. #### 6.5.1.2 Command Command can be sent by the A-IoT CN for a single device. For topology 1, A-IoT RAN node should be able to differentiate between command and inventory. NOTE 1: Whether command can be performed for a group of devices depends on RAN2, SA2 and SA3. NOTE 2: Whether the A-IoT-enabled-gNB should be able to differentiate between command and inventory needs further discussion, which depends on SA2 and RAN2. NOTE 3: Whether the description in this clause apply to A-IoT enabled gNB in topology 2 needs further discussion and is dependent on SA2 and RAN2.

And SA2 has agreed to the following information provided from the AF to AIoTF in TR 38.769:

### 8.3.5 Information provided by the AF The AF provides one or some of the following information: - Information about AIoT services: - Service type indicating e.g. inventory or command (e.g. read or write). - Information to be used for AIoT reader selection: - At least one of the UE reader ID or the target area information. - Optionally, Information about the target AIoT device(s): - At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices. - Optionally, Information to be used for resource allocation that is detailed in TR 38.769 [8]: - Approximate number of AIoT devices. - Approximate D2R message size. NOTE: Information to be used for resource allocation will be aligned with RAN WGs during the normative phase.

Since the content of Inventory Request/Response, Inventory Report, and Command Request/Response are transparent to the AIoT RAN node, some AIoT RAN visible information should be included in the AIoT NAS data transport messages.

Inventory Request:

If the AIoT NAS transport container includes Inventory Request message from the AIoTF to the AIoT RAN node, the content of the AIoT NAS data transport message may include the container with some AIoT RAN visible information like that:

- Container including Inventory Request.

- Service type indication or inventory indication. Indicating the AIoT NAS transport container includes the Inventory Request, so that the AIoT RAN reader could trigger the paging procedure.

- Service scope of the inventory. For instance, the inventory should trigger in a specific area.

- Approximate number of AIoT devices. It is helpful for resource allocation and paging. Include the approximate number of AIoT devices, and the reader can generate Qurey-like parameters based on the estimated number of devices. For example, if the estimated number is 100, the reader may generate a Qurey-like parameter value of 128 or 256. If the inventory request does not include the estimated number of devices, the reader has to generate the Qurey-like value randomly. Device access conflicts may frequently occur if the query-like value is smaller than the actual number of devices.

- Approximate D2R message size. It is helpful for resource allocation for PDRCH.

- AIoT Session ID. It should be supported to uniquely identify the inventory session since there may be multiple inventory sessions in the NGAP interface.

- Reader ID(s). One AIoT RAN node may hold multiple readers, and the AF or AIoTF may decide to use one or more.

- AIoTF ID. It indicates where the inventory request comes from, and the AIoTF ID also indicates the target of the inventory response and inventory report(s) in the following inventory procedure.

Proposal 2: If the AIoT NAS data transport message contains Inventory Request from the AIoTF to the AIoT RAN node, the RAN-visible information may be service type indication or inventory indication, service scope of the inventory, approximate number of AIoT devices, approximate D2R message size, reader ID(s), and AIoTF ID.

Inventory Response:

If the AIoT NAS transport container includes Inventory Response message from the AIoT RAN node to the AIoTF, the content of the AIoT NAS data transport message may include the container with some assistance information like that:

- Container including Inventory Response message.

- AIoTF ID. It is used to identify the destination of the Inventory Response container if the indirect connection between the AIoT RAN node and AIoTF is applied. The AIoT RAN needs to explicitly send it to the AMF so that the AMF can forward the Inventory Response to the target AIoTF entity.

From our perspective, the AMF functions as a relay, transmitting the Inventory Response from the AIoT RAN node to the AIoTF in the case of an indirect connection. AIoTF ID is enough for the AMF to transmit the inventory response to the target AIoTF; there is no other necessary assistance information that needs to be included outside of the AIoT NAS transport container.

Proposal 3: If the AIoT NAS data transport message contains Inventory Response from the AIoT RAN node to the AIoTF, the necessary assistance information is the AIoTF ID.

Inventory Report:

If the AIoT NAS transport container includes Inventory Report message from the AIoT RAN node to the AIoTF, the content of the AIoT NAS data transport message may include the Inventory Report container, AIoTF ID, and Reader ID.

- Container including Inventory Report message.

- AIoTF ID. The same reason as mentioned in Inventory Response part.

- The Reader ID. The purpose of the Reader ID is to report the reader's location, as specified in TR 38.769.

Proposal 4: If the AIoT NAS data transport message contains Inventory Report from the AIoT RAN node to the AIoTF, the necessary assistance information is the AIoTF ID and Reader ID.

Command Request:

If the AIoT NAS transport container includes Command Request message from the AIoTF to the AIoT RAN node, the content of the AIoT NAS data transport message may include the container with some AIoT RAN visible information like that:

- Container including Command Request.

- Service type indication or Command indication. Indicating the AIoT NAS transport container includes the Command Request, so that the AIoT RAN reader schedules the PRDCH resource to transmit the Command Request to the AIoT device.

- Reader ID(s). One AIoT RAN node may hold multiple readers; it is necessary to indicate the target reader of this Command Request.

The descriptions above are applicable in the “A-IoT Inventory and Command" case. For the “Command-only” case, the RAN-visible information is the same as proposed in the inventory request part because the Command Request will trigger the AIoT paging procedure and resource allocation as the inventory request does.

Proposal 5: For the “A-IoT Inventory and Command" case, if the AIoT NAS data transport message contains Command Request from the AIoT RAN node to AIoTF, the RAN-visible information may be service type indication or Inventory indication and reader ID.

Proposal 6: For the “Command-only” case, if the AIoT NAS data transport message contains Inventory Request from AIoTF to the AIoT RAN node, the RAN-visible information may be service type indication or Inventory indication, service scope of the inventory, approximate number of AIoT devices, approximate D2R message size, reader ID(s), and AIoTF ID.

Command Response:

If the AIoT NAS transport container includes Command Response message from the AIoT RAN node to the AIoTF, the content of the AIoT NAS data transport message may include the Command Response container, AIoTF ID, and Reader ID for the same reason as mentioned in the Inventory Report part.

Proposal 7: If the AIoT NAS data transport message contains Command Response from the AIoT RAN node to the AIoTF, the necessary assistance information is the AIoTF ID and Reader ID.

# 3. Conclusion

Observation 1: The AIoT NAS protocol between the AIoT device and the AIoTF is introduced to carry the Inventory Request, Inventory Response, Inventory Report, Command Request and Command Response. And the AIoT NAS protocol is a protocol layer over the NGAP, and the data carried by the AIoT NAS protocol is transparent to the NGAP.

Proposal 1: RAN3 needs to define a new AIoT NAS data transport procedure to transmit AIoT NAS layer data over the NGAP where the AIoT NAS layer data is included in a transparent container over NGAP. A downlink AIoT NAS data transport message to transmit AIoT NAS layer data from the AIoTF to the AIoT RAN node and an uplink AIoT NAS data transport message to transmit AIoT NAS layer data from the AIoT RAN node to the AIoTF.

Proposal 2: If the AIoT NAS data transport message contains Inventory Request from the AIoTF to the AIoT RAN node, the RAN-visible information may be service type indication or inventory indication, service scope of the inventory, approximate number of AIoT devices, approximate D2R message size, reader ID(s), and AIoTF ID.

Proposal 3: If the AIoT NAS data transport message contains Inventory Response from the AIoT RAN node to the AIoTF, the necessary assistance information is the AIoTF ID.

Proposal 4: If the AIoT NAS data transport message contains Inventory Report from the AIoT RAN node to the AIoTF, the necessary assistance information is the AIoTF ID and Reader ID.

Proposal 5: For the “A-IoT Inventory and Command" case, if the AIoT NAS data transport message contains Command Request from the AIoT RAN node to AIoTF, the RAN-visible information may be service type indication or Inventory indication and reader ID.

Proposal 6: For the “Command-only” case, if the AIoT NAS data transport message contains Inventory Request from AIoTF to the AIoT RAN node, the RAN-visible information may be service type indication or Inventory indication, service scope of the inventory, approximate number of AIoT devices, approximate D2R message size, reader ID(s), and AIoTF ID.

Proposal 7: If the AIoT NAS data transport message contains Command Response from the AIoT RAN node to the AIoTF, the necessary assistance information is the AIoTF ID and Reader ID.

# References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Huawei