3GPP TSG-RAN WG3 #127	R3-250185

Greece, Athens, 17th – 21st February 2025

Agenda item:		16.3

Title:		Discussion on AIoT location reporting aspects

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The new working item for Ambient IoT in NR is agreed upon in the RAN#106 meeting with the following scope in [1]:

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. The following objectives are set, within the General Scope: - RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

This contribution will discuss the location reporting issues of Ambient IoT.

# 2. Discussion

TR 38.769 includes the following descriptions for AIoT location reporting:

### 6.9.2 Topology 1 For support of A-IoT device location, the A-IoT RAN node may report the A-IoT RAN node ID to the A-IoT CN in the Inventory Report or Command Response messages. The A-IoT RAN node may additionally report a Reader ID to the A-IoT CN in the Inventory Report and/or the Command Response messages, which corresponds to the reader to which the A-IoT device responded. NOTE 1: Whether multiple Reader ID(s) can be included by the A-IoT RAN node in the Inventory Report/Command Response message needs further discussion. NOTE 2: Definition and format of the A-IoT RAN node ID and Reader ID needs further discussion. NOTE 3: Whether and how to have awareness of Reader location in A-IoT CN needs further discussion.

There are three issues left for topology 1 that need further discussion.

2.1	Whether multiple Reader ID(s) can be included in location report

In topology 1, an A-IoT RAN node may serve one or more readers. Some companies proposed that multiple readers could be used to get a higher-accuracy device location. One AIoT device may respond to more than one reader in an AIoT RAN node. For example, the AIoT device may be covered by more than one reader. This means that the network could figure out where the device is by using multiple reader IDs, which is a more accurate location than the "reader ID" granularity location. As RAN3 discussed in previous meetings, it greatly increases the complexity of the AIoT location mechanism, and the benefit is very limited.

However, we still think the inventory report/command response should be able to include multiple reader IDs, but not for AIoT device locations. It is possible that the AIoT RAN may merge the AIoT device responses from the multiple readers into a single Inventory Report/Command Response. And there is another case in which the different readers under an AIoT RAN node may receive the same Inventory Request/Command Request from the AIoTF; a single AIoT device may respond to the same Inventory Request/Command Request to the different readers. In that case, the AIoT RAN node may include multiple reader IDs in the Inventory Report.

Observation 1: There are two scenarios in which the Inventory Report may contain multiple reader IDs. The first is that AIoT RAN may merge the AIoT device responses from the multiple readers into a single Inventory Report. The second is that the different readers under an AIoT RAN node may receive the same inventory request from the AIoTF; a single AIoT device may respond to the same inventory request to the different readers.

Proposal 1: The Inventory Report/Command Request should be able to include multiple Reader IDs, but not for AIoT device location reasons.

2.2	Whether and how to have awareness of Reader location in A-IoT CN

In previous meetings, it is common understanding that the AIoT RAN node can have a reader for each TRP for topology 1, so the reader location can be considered as the TRP location. And we believe the AIoTF needs to know the location of the reader because it is useful for AIoTF selecting readers when the AIoT service area is configured from AF.

The AIoT RAN node and the OAM know the geographical coordinates of the TRPs and can transfer the information to the AIoTF as the reader's location. The geographical coordinates information of the TRPs could be included in the interface setup between the AIoT RAN node and the AIoT CN, or the information can be transmitted with the reader ID in the Inventory Report/Command Request as reader location.

Proposal 2: The AIoTF should be aware of the location of readers.

Proposal 3: The AIoT location can be transmitted during the interface setup procedure between AIoT RAN node and the AIoT CN or with the reader ID in the Inventory Report/Command Request as reader location.

2.3	Discussion on the format of the A-IoT RAN node ID and Reader ID

We think there are three options for the format of the reader ID:

Option 1: If the readers are set up based on the TRPs of the AIoT RAN node, it is simple that the TRP ID defined in TS38.305 could be represented as the reader ID.

Option 2: If the global reader ID is defined like the Global Cell Identifier, the AIoT RAN node ID is not needed or the AIoT RAN node ID is just a part of the global reader ID.

Option 3: The reader also could be identified by an AIoT RAN node ID and a simple index of the reader served by the AIoT RAN node. For example, an AIoT RAN node serves 3 readers, and the 3 readers can be simply identified by reader #1, reader #2, and reader #3 with the AIoT RAN node ID.

Furthermore, a Global gNB ID defined in TS 38.413 could perfectly identify an AIoT RAN node.

Proposal 4: the Reader ID could have 3 options:

- Option 1: TRP ID could be used as the reader ID.

- Option 2: Design a global reader ID like Global Cell Identifier.

- Option 3: Reader is identified by an AIoT RAN node ID and a simple index of the reader.

Proposal 5: The format of the AIoT RAN node ID should be aligned with the Global gNB ID defined in TS 38.413.

# 3. Conclusion

Observation 1: There are two scenarios in which the Inventory Report may contain multiple reader IDs. The first is that AIoT RAN may merge the AIoT device responses from the multiple readers into a single Inventory Report. The second is that the different readers under an AIoT RAN node may receive the same inventory request from the AIoTF; a single AIoT device may respond to the same inventory request to the different readers.

Proposal 1: The Inventory Report/Command Request should be able to include multiple Reader IDs, but not for AIoT device location reasons.

Proposal 2: The AIoTF should be aware of the location of readers.

Proposal 3: The AIoT location can be transmitted during the interface setup procedure between AIoT RAN node and the AIoT CN or with the reader ID in the Inventory Report/Command Request as reader location.

Proposal 4: the Reader ID could have 3 options:

-	Option 1: TRP ID could be used as the reader ID.

-	Option 2: Design a global reader ID like Global Cell Identifier.

-	Option 3: Reader is identified by an AIoT RAN node ID and a simple index of the reader.

Proposal 5: The format of the AIoT RAN node ID should be aligned with the Global gNB ID defined in TS 38.413.

# References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Huawei