3GPP TSG-RAN WG3 Meeting #127	R3-250190

Athens, Greece, 17th – 21st February 2025

Agenda Item:	31.1

Source:	Ericsson, Vodafone, Huawei

Title:	Discussion on X2/Xn exchange of signalling TNL addresses for signalling transport establishment

Document for:	Discussion and agreement

# 1	Introduction

We have been made aware of an inter-operability issue concerning the establishment of X2 signalling transport in an MOCN sharing deployment, where the signalling TNL addresses exchanged cause the establishment to fail.

The underlying transport deployment consists of one (separate) X2 signalling transport network per operator participating in MOCN, with both sides, the (initiating) eNB and the candidate eNB obviously exchanging information in such a way that successful establishment of the X2 signalling transport is not (always) possible.

It is assumed that the same situation may most likely appear in 5G MOCN networks as well, so the discussion is equally valid for X2 and Xn.

# 2	Discussion

Automated setup of X2 interfaces has been introduced in the very beginning of 4G, with the (initiating) eNB, once becoming aware of neighbouring cells by means of UE reports to which no X2 connection has been yet established, attempting to exchange signalling transport addresses to enable X2 interface setup, by first establishing an SCTP association and then perform setup of X2 at application layer.

One potential MOCN deployment scenario described in the introduction may look as follows:

Figure 1: Possible deployment of separate (X2) TNL networks for MOCN.

As shown in Figure 1, the chosen TNL setup consists of two separate IP Networks, and one of the tasks in deployment is obviously to ensure that both ends, the (initiating) eNB and the candidate eNB utilise the information to be exchanged information within the ENB/MME Configuration Transfer procedures in a way that guarantees that signalling transport uses either of the shown transport networks, i.e. IP addresses of either IP-Network-A or IP-Network B are exchanged - a situation where the (initiating) eNB indicates an “A-address” and the candidate eNB indicates a “B-address” will not lead to a successful establishment of the SCTP association.

Next we try to examine the content of the S1AP procedures involved in the exchange of X2 (signalling) TNL addresses – and it reveals an answer to the question “What could go wrong?”:

Figure 2: Examining Potential Failure(s) at exchange of X2 TNL addresses

Potential Failure scenario:

-	The (initiating) eNB uses the Target/Source eNB ID IEs as indication for the MME to route the configuration transfer messages accordingly.

-	The PLMN IDs indicated in the Global eNB IDs are according to the rules stated for global eNB IDs in case several PLMN IDs are indicated in SIB1.

-	In 4G, the selected TAI is (also) used for the potential involvement of HeNB GWs, in order to route towards the proper GW.

-	The IP network to which the indicated address at the (initiating) eNB is associated with is (meant to be) indicated in the “source eNB-ID”, both by means of the PLMN-ID in the Global eNB ID and the selected TAI.

-	The candidate eNB however interprets the information in the Target eNB ID IE only and selects and IP address that is associated with the PLMN ID indicated in the Target eNB ID IE.

Another, similar tailure scenario:

In principle the same as scenario 1, only that the usage (at the sending side) and the interpretation (at the receiving side) of IEs is inter-changed between the (initiating) eNB and the candidate eNB, i.e.

-	the (initiating) eNB associates the TNL address with the PLMN ID indicated in the target eNB ID

-	while the candidate eNB associates the TNL address it with the PLMN ID indicated in the Source eNB ID.

We observe the following from these potential failure scenarios:

Observation 1:	Network functions associated with shared radio networks are obviously still a source different interpretation of the standard, and it appears that 3GPP is not sufficiently explicit to fully avoid above mentioned failure scenarios, neither for 4G nor for 5G – despite the fact that this feature was introduced more than 15 years ago.

Observation 2:	It can be assumed that correcting the standard in order to avoid such different interpretations will cause non-backwards compatible changes in existing implementations, hence coming to an agreement in 3GPP will not be possible without (the unlikely reaching of a) consensus.

Observation 3:	In our opinion, one of the reasons why this potential failure scenario was not discussed sufficiently seems to be the fact that 3GPP (RAN3) has always avoided to specify assumption on the Transport Network Layer when defining Radio Network Layer functions. And we would not like to see this principle being changed (so late).

Observation 4:	As far as we can see, there is no error handling or recovery mechanism specified for the Configuration Transfer function, i.e. the fact that the addresses exchanged did not lead to a successful signalling TNL establishment can only be “guessed” by the involved peer nodes, but there is no explicit indication defined to trigger re-trying the exchange of TNL addresses. One can easily imagine solutions how to overcome such lack of specification content, but this will affect all implementations of deployed networks (at least of those networks applying MOCN).

Proposal 1:	We propose to acknowledge the existence of different interpretation of the current standard, both for 4G and 5G networks.

Proposal 2:	To resolve the situation, we propose to add a NOTE to both 36.300 and 38.300 stating that “It is assumed that the candidate eNB/NG-RAN node is able to determine which TNL addresses to provide in order to establish successful SCTP connectivity, based on information received from the eNB/NG-RAN node (e.g. PLMN ID) that has triggered TNL address discovery.”

As one can easily deduce, such a note aims to increase awareness that if an X2/Xn is set up between nodes where either one or both are eNBs/gNBs operating using MOCN, the PLMN ID should be considered by the target node when providing a TNL address to the requesting node. But it does not yet provide an explicit guidance on which instance of the PLMN ID IE in the SON Configuration Transfer IE (i.e. from the target or source-node ID or from the source or target selected TAI) should be considered by the target node when providing a TNL address.

Proposal 3	Therefore we also propose discussing a possible introduction of new Information to the SON Configuration Transfer IE to both 36.413 and 38.413. E.g., a new IE could explicitly indicate to the candidate node the PLMN the TNL Configuration Information is associated with.

Proposal 4:	We propose to discuss the candidate solution indicated in Annex A and how to further proceed based on stage 2 and stage 3 CRs submitted in R3-250228(36.300), R3-250230 (36.413), R3-250229 (38.300) and R3-250231 (36.413). Note, that the procedure text and semantics are FFS and represent one possible solution.

# Annex A – proposed NOTE to stage 2 and new IE in stage 3

<<<<<<<<<<<<<<<<<<<< Start of change for 36.300 >>>>>>>>>>>>>>>>>>>>

NOTE:	It is assumed that the candidate eNB is able to determine which TNL addresses to provide in order to establish successful SCTP connectivity, based on information received from the eNB (e.g. PLMN ID) that has triggered TNL address discovery.

<<<<<<<<<<<<<<<<<<<< Start of change for 38.300 >>>>>>>>>>>>>>>>>>>>

NOTE:	It is assumed that the candidate NG-RAN node is able to determine which TNL addresses to provide in order to establish successful SCTP connectivity, based on information received from the NG-RAN node (e.g. PLMN ID) that has triggered TNL address discovery.

<<<<<<<<<<<<<<<<<<<< Start of change for 36.413 >>>>>>>>>>>>>>>>>>>>

### 8.16.2	Successful Operation

#### 8.16.2.1	MME Configuration Transfer

Figure 8.16.2.1-1: MME Configuration Transfer procedure. Successful operation.

The procedure is initiated with an MME CONFIGURATION TRANSFER message sent from the MME to the eNB.

<<<<<<<<<<<<<<<<<<<< unmodified text omitted >>>>>>>>>>>>>>>>>>>>

If the eNB receives the SON Information IE containing the SON Information Request IE set to “Deactivate Muting”, the eNB may consider deactivating muting for over-the-air synchronisation that was activated by a former muting request from the corresponding eNB.

Interaction with the eNB Configuration Transfer procedure:

If the eNB receives the SON Configuration Transfer IE containing the Requested TNL Information IE and the SON Information Request IE set to "X2 TNL Configuration Info" the eNB shall, if supported, take the PLMN ID contained in the Requested TNL Information IE into account and include X2 TNL configuration information within the eNB CONFIGURATION TRANSFER message associated with this PLMN.

<<<<<<<<<<<<<<<<<<<< Next change for 36.413 >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.26	SON Configuration Transfer

This IE contains the configuration information, used by e.g., SON functionality, and additionally includes the eNB identifier of the destination of this configuration information and the eNB identifier of the source of this information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SON Configuration Transfer |  |  |  |  |  |  |
| >Target eNB-ID | M |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.1.37 |  |  |  |
| >>Selected TAI | M |  | TAI 9.2.3.16 |  |  |  |
| >Source eNB-ID | M |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.1.37 |  |  |  |
| >>Selected TAI | M |  | TAI 9.2.3.16 |  |  |  |
| >SON Information | M |  | 9.2.3.27 |  |  |  |
| >X2 TNL Configuration Info | C-ifSONInformationRequest |  | 9.2.3.29 | Source eNB X2 TNL Configuration Info. | YES | ignore |
| >Synchronisation Information | C-if Activate Muting |  | 9.2.3.42 | Information on cell selected as source of synchronisation and aggressor cells. | YES | ignore |
| >Requested TNL Information | O |  | PLMN Identity 9.2.3.8 | Indicates to the candidate eNB the PLMN ID with which the X2 TNL Configuration Info IE is associated. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifSONInformationRequest | This IE shall be present if the SON Information IE contains the SON Information Request IE set to the value “X2TNL Configuration Info” |
| ifActivateMuting | This IE shall be present if the SON Information IE contains the SON Information Request IE set to the value “Activate Muting” |

<<<<<<<<<<<<<<<<<<<< Start of change for 38.413 >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.8.2	Downlink RAN Configuration Transfer

#### 8.8.2.1	General

The purpose of the Downlink RAN Configuration Transfer procedure is to transfer RAN configuration information from the AMF to the NG-RAN node. This procedure uses non-UE associated signalling.

#### 8.8.2.2	Successful Operation

Figure 8.8.2.2-1: Downlink RAN configuration transfer

The procedure is initiated with an DOWNLINK RAN CONFIGURATION TRANSFER message sent from the AMF to the NG-RAN node.

<<<<<<<<<<<<<<<<<<<< unmodified text omitted >>>>>>>>>>>>>>>>>>>>

If the NG-RAN node is configured to use one IPsec tunnel for all NG and Xn traffic (IPsec star topology) then the traffic to the peer NG-RAN node shall be routed through this IPsec tunnel and the IP-Sec Transport Layer Address IE shall be ignored.

Interaction with the Uplink RAN Configuration Transfer procedure:

If the NG-RAN node receives the SON Configuration Transfer IE containing the Requested TNL Information IE and the SON Information Request IE set to "Xn TNL Configuration Info", the NG-RAN node shall, if supported, take the PLMN ID contained in the Requested TNL Information IE into account and include Xn TNL configuration information within the UPLINK RAN CONFIGURATION TRANSFER message associated with this PLMN.

<<<<<<<<<<<<<<<<<<<< Next change for 38.413 >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.6	SON Configuration Transfer

This IE contains the configuration information, used by e.g., SON functionality, and additionally includes the NG-RAN node identifier of the destination of this configuration information and the NG-RAN node identifier of the source of this information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Target RAN Node ID SON |  | 1 |  |  | - |  |
| >Global RAN Node ID | M |  | 9.3.1.5 |  | - |  |
| >Selected TAI | M |  | TAI 9.3.3.11 |  | - |  |
| >NR CGI | O |  | 9.3.1.7 | This IE is ignored if the SON Information IE contains the SON Information Reply IE. | YES | ignore |
| Source RAN Node ID |  | 1 |  |  | - |  |
| >Global RAN Node ID | M |  | 9.3.1.5 |  | - |  |
| >Selected TAI | M |  | TAI 9.3.3.11 |  | - |  |
| SON Information | M |  | 9.3.3.7 |  | - |  |
| Xn TNL Configuration Info | C-ifSONInformationRequest |  | 9.3.3.9 | Source NG-RAN node Xn TNL Configuration Info. | - |  |
| Requested TNL Information | O |  | PLMN Identity 9.3.3.5 | Indicates to the candidate NG-RAN node the PLMN ID with which the Xn TNL Configuration Info IE is associated. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifSONInformationRequest | This IE shall be present if the SON Information IE contains the SON Information Request IE set to "Xn TNL Configuration Info" |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>