3GPP TSG-RAN WG3 Meeting #127	R3-250192

Athens, Greece, Feb 17th - Feb 21st, 2025

Agenda item:	13.2

Source:	Qualcomm Incorporated

Title:	Signalling enhancements for inter-CU LTM HO

WID/SID:	NR_Mob_Ph4-Core – Release 19

Document for:	Discussion and decision

# Introduction

In the last RAN3 meeting (RAN3 #126 [1]), there was continued progress in specifying inter-CU LTM, and agreements were made on CSI-RS configuration for L1 measurements and reporting, LTM candidate cell configuration, establishing UE associations upon inter-gNB LTM cell switch, LTM cancellation, and release of UE context at source gNB.

RAN3 #126 agreements:

RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution.

The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB.

Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation?

In this contribution, we continue the discussion on signalling enhancements for inter-CU LTM handover, focusing on the following procedures and scenarios:

- CSI configuration for SSB and CSI-RS-based L1 measurements and reporting,

- Activation and deactivation of Semi-persistent CSI-RS resources for L1 measurements and reporting,

- Early TA RACH configuration,

- TA Information Transfer,

- LTM cancellation,

- LTM reference configuration,

- Inter-CU LTM for Dual Connectivity (DC) scenarios.

# 2 CSI configuration for SSB and CSI-RS-based L1 measurements and reporting

Many agreements on CSI configuration for SSB and CSI-RS-based L1 measurements and reporting were made in the previous RAN3 meetings. The signalling for CSI configuration for CSI-RS-based L1 measurements and reporting was completed in the last RAN3 meeting (RAN3 #126 [1]). The signalling for CSI configuration for SSB-based L1 measurements and reporting was partially completed in the previous RAN3 meeting (RAN3 #125-bis [2]).

We address the remaining signalling aspects of CSI configuration for SSB-based L1 measurements and reporting (see Figure 1).

For CSI configuration for SSB-based L1 measurements and reporting, it was agreed that the source gNB-CU transmits the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the corresponding CSI report configuration to the source gNB-CU via Handover Request ACK message.

Based on the response from its own underlying candidate gNB-DUs and the candidate gNB-CUs regarding the prepared cells in the initial round of LTM preparation, the source gNB-CU may generate an updated common LTM CSI configuration consisting of the list of transmitting SSBs in all the prepared cells, to enable subsequent inter-gNB LTM.

Figure 1: Example signalling flow of the SSB-based CSI configuration procedure

Proposal 1. For SSB-based CSI configuration, based on the response from its own underlying candidate gNB-DUs and the candidate gNB-CUs regarding the prepared cells in the initial round of LTM preparation, the source gNB-CU may generate an updated common LTM CSI configuration consisting of:

- List of LTM candidate cells,

- For each such cell, the list of transmitting SSBs of the cell.

Proposal 2. The source gNB-CU may include in LTM Configuration Update message to a candidate gNB-CU the updated common LTM CSI resource configuration.  This information is forwarded by the candidate gNB-CU to its underlying candidate gNB-DUs using F1 UE Context Modification Request.

Proposal 3. A candidate gNB-DU provides to the candidate gNB-CU the following information in F1 UE Context Modification Response, which is then forwarded by the candidate gNB-CU to the source gNB-CU in Xn LTM Configuration Update Ack:

- For each prepared candidate cell, the associated lower layer configuration, which includes the updated LTM CSI report configuration.

# 3 Activation and deactivation of Semi-persistent CSI-RS resources for L1 measurements and reporting

The signalling for CSI configuration for CSI-RS-based L1 measurements and reporting was completed in the last RAN3 meeting (RAN3 #126 [1]). The configuration can be for periodic CSI-RS as well as for semi-persistent CSI-RS (SP-CSI-RS) resources on LTM candidate cells.

In this section, we discuss the activation and deactivation aspects of configured SP-CSI-RS resources on LTM candidate cells.

During the CSI configuration procedure in LTM preparation, the source gNB-DU is provided with information regarding the configured SP-CSI-RS resources on all the LTM candidate cells. We think that the source gNB-DU should make the decision regarding the activation and deactivation of configured SP-CSI-RS resources based on, e.g., wider SSB L1 measurement reports of the LTM candidate cells, from the UE. The SSB L1 measurement reports indicate the wide beams on the candidate cells which are likely to provide good coverage to the UE when the UE performs the LTM cell switch, and so the corresponding CSI-RS beams should be the beams the UE should measure and provide reports to the network. Following figure depicts typical SSB beams and associated CSI-RS beams.

CSI-RS

SSB

CSI-RS

CSI-RS

SSB

SSB

The overall signalling flow is shown in Figure 2. After the LTM preparation procedure is completed, the network provides the UE with configuration regarding SP-CSI-RS resources as part of the LTM configuration.

Figure 2: Activation and deactivation of SP-CSI-RS resources for L1 measurements and reporting

Proposal 4. Based on configured SSB and/or CSI-RS beam measurements on LTM candidate cells, reported in L1 measurement reports, the source gNB-DU decides to activate/deactivate SP-CSI-RS resources for a list of LTM candidate cells and transmits to a candidate gNB-DU the SP-CSI-RS Activation/Deactivation indications using signalling messages over F1 and Xn.

The candidate gNB-DU can be an underlying DU of the source gNB-CU or a candidate gNB-CU.

Proposal 5. The SP-CSI-RS Activation/Deactivation signalling message for a candidate gNB-DU includes:

For each cell from a list of indicated LTM candidate cells in the message:

- List of SP-CSI-RS resources to be activated,

- List of SP-CSI-RS resources to be deactivated.

Proposal 6. Upon receiving the SP-CSI-RS Activation/Deactivation signalling message, a candidate gNB-DU begins transmission or stops transmission of the indicated SP-CSI-RS resources, depending on the indications in the message to activate or deactivate each SP-CSI-RS resource.

Note that, though we discuss above of activating or deactivating a SP-CSI-RS resource, strictly speaking, the activation/deactivation is for a SP-CSI-RS resource set.

Observation 1. After the source gNB-CU sends the SP-CSI-RS Activation/Deactivation signalling message to its underlying gNB-DUs and other candidate gNB-DUs, the source gNB-CU signals the corresponding activation/deactivation indication to the UE, i.e., for each cell in an indicated list of LTM candidate cells in the signalling message, the following information is provided:

- List of SP-CSI-RS resources to be activated,

- List of SP-CSI-RS resources to be deactivated.

# 4 Early TA RACH configuration

In a previous RAN2 meeting (RAN2 #125-bis [3]), RAN2 agreed on the following:

- Early UL sync is supported for inter-CU LTM.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline.

The above agreements indicate that similar RACH configuration and TA estimation mechanism as in Rel-18 intra-CU LTM can be used for Rel-19 inter-CU LTM. In this and the following section, we propose extensions to the Rel-18 backhaul signalling mechanism for RACH configuration for Early TA acquisition and the transfer of TA information from a candidate DU to the source DU. Please see Figure 3 for the overall signalling flow.

Proposal 7. During LTM preparation, in Xn Handover Request, the source gNB-CU may transmit a request to a candidate gNB-CU for obtaining RACH configuration and for allocation of CFRA resources, e.g., RAP IDs, for Early TA acquisition. The following information is included:

- LTM candidate cell ID,

- List of source gNB-DU ID and candidate gNB-DU IDs of source gNB-CU.

A set of CFRA resources should be allocated for an underlying DU of the source gNB-CU, and these are to be used by the UEs served by the DU to initiate RACH for Early TA acquisition.

Proposal 8. Upon receiving the request from the source gNB-CU, the candidate gNB-CU forwards the request to its underlying candidate gNB-DU in F1 UE Context Setup Request, including:

- LTM candidate cell ID,

- List of source gNB-DU ID and candidate gNB-DU IDs of source gNB-CU,

- The source gNB ID.

As per principles established in Rel-18 intra-CU LTM, upon receiving the request, the candidate gNB-DU determines the split of the available CFRA resources among the indicated gNB-DUs of the source gNB, and stores this information along with the source gNB ID.

Proposal 9. If the suggested LTM candidate cell can be prepared, in the F1 UE Context Setup Response message, the candidate DU provides to the candidate CU:

- LTM candidate cell ID,

- List of pairs of the form (gNB-DU ID of the source CU, CFRA resource set, i.e., RAP IDs),

- CFRA configuration for Early TA acquisition,

- The source gNB ID.

Proposal 10. The candidate gNB-CU forwards the provided information from its underlying candidate DU to the source gNB-CU in Xn Handover Request Ack:

- LTM candidate cell ID,

- List of pairs of the form (gNB-DU ID of the source CU, CFRA resource set, i.e., RAP IDs),

- CFRA configuration for Early TA acquisition.

Figure 3: Example signalling flow of Early TA RACH configuration procedure

Proposal 11. Upon receiving the Xn Handover Request Ack from the candidate gNB-CU, the source gNB-CU forwards the received information to the corresponding underlying DU in F1 UE Context Modification Request:

- LTM candidate cell ID,

- CFRA resource set, i.e., RAP IDs,

- CFRA configuration for Early TA acquisition.

After the initial round of preparation of the candidate cells in a candidate gNB-CU, the source gNB-CU has information regarding the candidate gNB-DU IDs in a candidate gNB-CU.

In the “second round” of LTM preparation, the source gNB-CU uses the new UE-associated procedure, LTM Configuration Update, to obtain the RACH configuration and for allocation of CFRA resources for Early TA acquisition for the other candidate gNB-CUs.

Proposal 12. In Xn LTM Configuration Update message to a candidate gNB-CU, the source gNB-CU includes the following information:

- For each other candidate gNB-CU:

  - List of candidate gNB-DU IDs.

Proposal 13. Upon receiving the message from the source gNB-CU, the candidate gNB-CU forwards the request to its underlying candidate gNB-DUs in F1 UE Context Modification Request, including:

- For each other candidate gNB-CU:

  - List of candidate gNB-DU IDs,

  - The candidate gNB ID.

Upon receiving the request, the candidate gNB-DU determines the split of the available CFRA resources among the indicated candidate gNB-DUs of other candidate gNB-CUs and stores this information along with the candidate gNB IDs.

Proposal 14. In the F1 UE Context Modification Response message, the candidate DU provides to the candidate CU:

- For each prepared LTM candidate cell:

  - For each other candidate gNB-CU:

    - List of pairs of the form (candidate gNB-DU ID, CFRA resource set, i.e., RAP IDs),

    - CFRA configuration for Early TA acquisition,

    - The candidate gNB ID.

Proposal 15. In the Xn LTM Configuration Update Ack to the source gNB-CU, the candidate gNB-CU relays the information received from its underlying gNB-DU:

- For each prepared LTM candidate cell:

  - For each other candidate gNB-CU:

    - List of pairs of the form (candidate gNB-DU ID, CFRA resource set, i.e., RAP IDs),

    - CFRA configuration for Early TA acquisition,

    - The candidate gNB ID.

Proposal 16. The source gNB-CU reuses the Xn LTM Configuration Update procedure to forward the received Early TA configuration information from each candidate gNB-CU to the other candidate gNB-CUs.

# 5 TA Information Transfer

A candidate DU determines the TA value based on the received RACH preamble from the UE. However, the candidate DU cannot determine the UE ID from the received RACH preamble since the RAP ID is from the pool of assigned RAP IDs to the source DU, for the candidate cell.

Proposal 17. Upon successfully receiving the RACH preamble from the UE on a candidate cell, a candidate gNB-DU determines the TA value. The candidate gNB-DU transmits to the candidate gNB-CU the following TA information using F1 non-UE associated DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Source gNB ID,

- Source gNB-DU ID,

- LTM candidate cell ID.

Proposal 18. The candidate gNB-CU forwards the TA information to the source gNB-CU using TA Information Transfer message over Xn.

Proposal 19. The source gNB-CU forwards the TA information to the source gNB-DU using F1 non-UE associated CU-DU TA Information Transfer.

The source DU then identifies the UE based on the TA information associated with RAP ID and provides to the UE the TA value in the LTM Cell Switch MAC CE command during LTM execution.

Figure 4: TA information transfer from a candidate DU to source DU with signalling over  Xn

During subsequent LTM operation, a similar set of proposals as the above hold.

Proposal 20. Upon successfully receiving the RACH preamble from the UE on a candidate cell during subsequent LTM, a candidate gNB-DU transmits to the candidate gNB-CU the following TA information using F1 non-UE associated DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Serving gNB ID,

- Serving gNB-DU ID,

- LTM candidate cell ID.

Proposal 21. The candidate gNB-CU forwards the TA information along with RAP ID to the serving gNB-CU using TA Information Transfer over Xn.

Proposal 22. The serving gNB-CU then forwards the TA information to the serving gNB-DU using the CU-DU TA Information Transfer message.

Proposal 23. As in Rel-18 Intra-CU LTM, the source gNB-DU or the serving gNB-DU maintains a check on the validity of the TA values, and it is up to DU implementation how to do so.

# 6 LTM cancellation

A candidate gNB-DU may initiate the cancellation of a subset of its prepared LTM candidate cells for a UE if it runs out of radio resources, e.g., due to congestion. The candidate gNB-DU then transmits a message to the serving gNB-CU via its candidate gNB-CU providing a cancellation indication. It is assumed that UE association is established over the Xn interface between the serving gNB-CU and the candidate gNB-CU upon inter-gNB LTM cell switch.

Proposal 24. A candidate gNB-DU can initiate the cancellation of a subset of its prepared LTM candidate cells for a UE, and it transmits a signalling message, e.g., UE Context Modification Required, to its candidate gNB-CU, which then forwards the cancellation information to the serving gNB-CU using the LTM Cancel message. In the UE Context Modification Required message and the LTM Cancel message, the list of cancelled LTM candidate cells is provided.

Observation 2. Upon receiving the LTM Cancel message, the serving gNB-CU transmits an RRC reconfiguration message to the UE indicating the release of the corresponding LTM candidate cell configurations.

# 7 LTM reference and delta configurations

It was agreed by RAN2 (RAN2 #127 meeting [4]) that a single reference configuration will be used for inter-CU LTM. This implies that the source gNB-CU generates the single reference configuration.

Proposal 25. The source gNB-CU transmits the lower layer part of the reference configuration for inter-CU LTM to its underlying DUs in a signalling message, which is FFS. An underlying candidate gNB-DU responds to the source gNB-CU with a delta lower layer configuration of the LTM candidate cell based on the reference configuration.

Proposal 26. The source gNB-CU transmits the reference configuration for inter-CU LTM to a candidate gNB-CU in an Xn signalling message, which is FFS. The candidate gNB-CU forwards the lower part of the reference configuration to its underlying DUs in a signalling message, which is FFS. An underlying candidate gNB-DU responds to the candidate gNB-CU with a delta lower layer configuration of the LTM candidate cell based on the reference configuration.

# 8 Inter-CU LTM for DC scenarios

The following are some of the DC scenarios to consider for study:

- Inter-MN LTM without SN change.

- Inter-SN LTM without MN change.

  - There are separate procedures for the MN-initiated and the SN-initiated cases similar as legacy Layer 3 SN change.

- Coexistence of Intra-CU MCG LTM and Inter-CU SCG LTM.

- Inter-MN LTM with SN release.

The scenarios b) and c) can be prioritized given the fact that inter-MN LTM has security issues that need to be resolved first.

Proposal 27. The following DC scenarios can be prioritized for study:

- SN initiated inter-SN LTM without MN change.

- Coexistence of intra-CU MCG LTM and inter-CU SCG LTM.

# 9 Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations and proposals.

CSI configuration for SSB and CSI-RS-based L1 measurements and reporting

Proposal 1. For SSB-based CSI configuration, based on the response from its own underlying candidate gNB-DUs and the candidate gNB-CUs regarding the prepared cells in the initial round of LTM preparation, the source gNB-CU may generate an updated common LTM CSI configuration consisting of:

- List of LTM candidate cells,

- For each such cell, the list of transmitting SSBs of the cell.

Proposal 2. The source gNB-CU may include in LTM Configuration Update message to a candidate gNB-CU the updated common LTM CSI resource configuration.  This information is forwarded by the candidate gNB-CU to its underlying candidate gNB-DUs using F1 UE Context Modification Request.

Proposal 3. A candidate gNB-DU provides to the candidate gNB-CU the following information in F1 UE Context Modification Response, which is then forwarded by the candidate gNB-CU to the source gNB-CU in Xn LTM Configuration Update Ack:

- For each prepared candidate cell, the associated lower layer configuration, which includes the updated LTM CSI report configuration.

Activation and deactivation of Semi-persistent CSI-RS resources for L1 measurements and reporting

Proposal 4. Based on configured SSB and/or CSI-RS beam measurements on LTM candidate cells, reported in L1 measurement reports, the source gNB-DU decides to activate/deactivate SP-CSI-RS resources for a list of LTM candidate cells and transmits to a candidate gNB-DU the SP-CSI-RS Activation/Deactivation indications using signalling messages over F1 and Xn.

The candidate gNB-DU can be an underlying DU of the source gNB-CU or a candidate gNB-CU.

Proposal 5. The SP-CSI-RS Activation/Deactivation signalling message for a candidate gNB-DU includes:

For each cell from a list of indicated LTM candidate cells in the message:

- List of SP-CSI-RS resources to be activated,

- List of SP-CSI-RS resources to be deactivated.

Proposal 6. Upon receiving the SP-CSI-RS Activation/Deactivation signalling message, a candidate gNB-DU begins transmission or stops transmission of the indicated SP-CSI-RS resources, depending on the indications in the message to activate or deactivate each SP-CSI-RS resource.

Observation 1. After the source gNB-CU sends the SP-CSI-RS Activation/Deactivation signalling message to its underlying gNB-DUs and other candidate gNB-DUs, the source gNB-CU signals the corresponding activation/deactivation indication to the UE, i.e., for each cell in an indicated list of LTM candidate cells in the signalling message, the following information is provided:

- List of SP-CSI-RS resources to be activated,

- List of SP-CSI-RS resources to be deactivated.

Early TA RACH configuration

Proposal 7. During LTM preparation, in Xn Handover Request, the source gNB-CU may transmit a request to a candidate gNB-CU for obtaining RACH configuration and for allocation of CFRA resources, e.g., RAP IDs, for Early TA acquisition. The following information is included:

- LTM candidate cell ID,

- List of source gNB-DU ID and candidate gNB-DU IDs of source gNB-CU.

Proposal 8. Upon receiving the request from the source gNB-CU, the candidate gNB-CU forwards the request to its underlying candidate gNB-DU in F1 UE Context Setup Request, including:

- LTM candidate cell ID,

- List of source gNB-DU ID and candidate gNB-DU IDs of source gNB-CU,

- The source gNB ID.

Proposal 9. If the suggested LTM candidate cell can be prepared, in the F1 UE Context Setup Response message, the candidate DU provides to the candidate CU:

- LTM candidate cell ID,

- List of pairs of the form (gNB-DU ID of the source CU, CFRA resource set, i.e., RAP IDs),

- CFRA configuration for Early TA acquisition,

- The source gNB ID.

Proposal 10. The candidate gNB-CU forwards the provided information from its underlying candidate DU to the source gNB-CU in Xn Handover Request Ack:

- LTM candidate cell ID,

- List of pairs of the form (gNB-DU ID of the source CU, CFRA resource set, i.e., RAP IDs),

- CFRA configuration for Early TA acquisition.

Proposal 11. Upon receiving the Xn Handover Request Ack from the candidate gNB-CU, the source gNB-CU forwards the received information to the corresponding underlying DU in F1 UE Context Modification Request:

- LTM candidate cell ID,

- CFRA resource set, i.e., RAP IDs,

- CFRA configuration for Early TA acquisition.

Proposal 12. In Xn LTM Configuration Update message to a candidate gNB-CU, the source gNB-CU includes the following information:

- For each other candidate gNB-CU:

  - List of candidate gNB-DU IDs.

Proposal 13. Upon receiving the message from the source gNB-CU, the candidate gNB-CU forwards the request to its underlying candidate gNB-DUs in F1 UE Context Modification Request, including:

- For each other candidate gNB-CU:

  - List of candidate gNB-DU IDs,

  - The candidate gNB ID.

Proposal 14. In the F1 UE Context Modification Response message, the candidate DU provides to the candidate CU:

- For each prepared LTM candidate cell:

  - For each other candidate gNB-CU:

    - List of pairs of the form (candidate gNB-DU ID, CFRA resource set, i.e., RAP IDs),

    - CFRA configuration for Early TA acquisition,

    - The candidate gNB ID.

Proposal 15. In the Xn LTM Configuration Update Ack to the source gNB-CU, the candidate gNB-CU relays the information received from its underlying gNB-DU:

- For each prepared LTM candidate cell:

  - For each other candidate gNB-CU:

    - List of pairs of the form (candidate gNB-DU ID, CFRA resource set, i.e., RAP IDs),

    - CFRA configuration for Early TA acquisition,

    - The candidate gNB ID.

Proposal 16. The source gNB-CU reuses the Xn LTM Configuration Update procedure to forward the received Early TA configuration information from each candidate gNB-CU to the other candidate gNB-CUs.

TA Information Transfer

Proposal 17. Upon successfully receiving the RACH preamble from the UE on a candidate cell, a candidate gNB-DU determines the TA value. The candidate gNB-DU transmits to the candidate gNB-CU the following TA information using F1 non-UE associated DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Source gNB ID,

- Source gNB-DU ID,

- LTM candidate cell ID.

Proposal 18. The candidate gNB-CU forwards the TA information to the source gNB-CU using TA Information Transfer message over Xn.

Proposal 19. The source gNB-CU forwards the TA information to the source gNB-DU using F1 non-UE associated CU-DU TA Information Transfer.

Proposal 20. Upon successfully receiving the RACH preamble from the UE on a candidate cell during subsequent LTM, a candidate gNB-DU transmits to the candidate gNB-CU the following TA information using F1 non-UE associated DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Serving gNB ID,

- Serving gNB-DU ID,

- LTM candidate cell ID.

Proposal 21. The candidate gNB-CU forwards the TA information along with RAP ID to the serving gNB-CU using TA Information Transfer over Xn.

Proposal 22. The serving gNB-CU then forwards the TA information to the serving gNB-DU using the CU-DU TA Information Transfer message.

Proposal 23. As in Rel-18 Intra-CU LTM, the source gNB-DU or the serving gNB-DU maintains a check on the validity of the TA values, and it is up to DU implementation how to do so.

LTM cancellation

Proposal 24. A candidate gNB-DU can initiate the cancellation of a subset of its prepared LTM candidate cells for a UE, and it transmits a signalling message, e.g., UE Context Modification Required, to its candidate gNB-CU, which then forwards the cancellation information to the serving gNB-CU using the LTM Cancel message. In the UE Context Modification Required message and the LTM Cancel message, the list of cancelled LTM candidate cells is provided.

Observation 2. Upon receiving the LTM Cancel message, the serving gNB-CU transmits an RRC reconfiguration message to the UE indicating the release of the corresponding LTM candidate cell configurations.

LTM reference and delta configurations

Proposal 25. The source gNB-CU transmits the lower layer part of the reference configuration for inter-CU LTM to its underlying DUs in a signalling message, which is FFS. An underlying candidate gNB-DU responds to the source gNB-CU with a delta lower layer configuration of the LTM candidate cell based on the reference configuration.

Proposal 26. The source gNB-CU transmits the reference configuration for inter-CU LTM to a candidate gNB-CU in an Xn signalling message, which is FFS. The candidate gNB-CU forwards the lower part of the reference configuration to its underlying DUs in a signalling message, which is FFS. An underlying candidate gNB-DU responds to the candidate gNB-CU with a delta lower layer configuration of the LTM candidate cell based on the reference configuration.

Inter-CU LTM for DC scenarios

Proposal 27. The following DC scenarios can be prioritized for study:

- SN initiated inter-SN LTM without MN change.

- Coexistence of intra-CU MCG LTM and inter-CU SCG LTM.

# References

[1] Chair notes, RAN3 #126 meeting.

[2] Chair notes, RAN3 #125-bis meeting.

[3] Chair notes, RAN2 #125-bis meeting.

[4] Chair notes, RAN2 #127 meeting.