3GPP TSG-RAN WG3 Meeting #127	R3-250197

Athens, Greece, 17-21 Feb, 2025

Title:	(TPs for TS 38.300, TS 38.410 BL CRs) Ambient IoT RAN Architecture

Agenda Item:	16.2

Source:	Huawei, China Telecom, Lenovo, China Unicom

Document for:	other

# Introduction

In RAN#106 meeting, the New Work Item: Solutions for Ambient IoT (Internet of Things) in NR [1] was agreed, and RAN3 needs to specify the related aspects in the following RAN3 meetings.

In this contribution, we discuss the RAN Architecture aspects and provide the corresponding TP to BL CRs.

# Discussion

## 2.1	Use cases, topologies and deployments

In the WID [1], the following can be found:

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. … - RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In SA2 TR [3], the following can be found:

- The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1: - Send requests that trigger BS Reader(s) (via AIoT RAN) to perform AIoT operations, either directly or via an AMF.

From the above and the RAN TR [2], the following Observations can be made:

Observation 1: Support deployment scenario 1 with Topology 1.

Observation 2: Suport DO-DTT, DT for indoor inventory and indoor command.

Observation 3: Support direct communication (gNB connects with AIOTF directly via NG interface) and indirect communication (gNB connects with AIOTF indirectly via AMF).

As we need to support both direct communication and indirect communication, in RAN3, we either design the procedures/messages for these two communications separately, or regardless of the peer CN node, introduce same procedure/messages between the gNB and CN. It is obvious that the latter one can simplify RAN3 discussion and avoid unnecessary duplications/complexity.

Proposal 1: Unified handling by the gNB is preferred, i.e., unless explicated stated, same procedures/messages to be used between gNB and CN regardless of the peer CN node in direct communication and indirect communication.

## 2.2	“Including AIOTF information in NGAP” vs “Carrying a new protocol layer over NGAP”

In the SI discussion, it was concluded that whether "XXAP" is realised by including AIoTF information in NGAP or by carrying a new protocol layer over NGAP, needs further discussion and decision. In this section, we name these two alternatives as Option 1 and Option 2.

- Option 1: Including AIoTF information in NGAP

Option 1 in direct communication									Option 1 in indirect communication

Figure 1: Option 1 AIoTF information in NGAP

In option 1, in direct communication, the AIOTF will encode/decode both the AIOTF Information Transfer IEs and the NGAP messages, and in indirect communication, the AMF will encode/decode the NGAP messages, but the AIOTF Information Transfer IEs will be forwarded by the AMF transparently.

- Option 2: Carrying a new protocol layer over NGAP

Option 2 in direct communication									Option 2 in indirect communication

Figure 2: Option 2 AIoTF information in NGAP

In option 2, in direct communication, the AIOTF will encode/decode both the XXAP messages and the NGAP messages, and in indirect communication, the AMF will encode/decode the NGAP messages, but the XXAP messages will be forwarded by the AMF transparently.

The following table lists the differences between these two options:

Table 1 Differences of Option 1 and Option 2

| Option 1 AIoTF information in NGAP | Option 2 AIoTF information in NGAP |
| --- | --- |
| No need for new RAN3 specification | Need one new RAN3 specification |
| Similar as SMF/MB-SMF Related IEs designed in NGAP. | Similar as NRPPa protocol, introduce NGAP: A-IoT Transport procedures to carry XXAP messages. |
| AIOT operations over new NGAP procedures, detailed information carried via AIOTF related IEs which are transparent to the AMF. | AIoT operation procedures over XXAP, these XXAP messages are transparent to AMF. |
| NG Interface management procedure enhancement needed. | NG Interface management procedure enhancement needed. |

As NGAP impacts are not avoidable in both options, capturing everything in the NGAP spec is easiler than changing NGAP spec and in the meanwhile creating another new spec, we propose to adopt option 1 to support the RAN CN interface.

Proposal 2: Introduce AIOTF related Transfer IEs in NGAP.

# Conclusion

In this paper, how to support RAN CN interface is discussed, the following observations and proposals are made:

Observation 1: Support deployment scenario 1 with Topology 1.

Observation 2: Suport DO-DTT, DT for indoor inventory and indoor command.

Observation 3: Support direct communication (gNB connects with AIOTF directly via NG interface) and indirect communication (gNB connects with AIOTF indirectly via AMF).

Proposal 1: Unified handling by the gNB is preferred, i.e., same procedures/messages to be used between gNB and CN regardless of the peer CN node in direct communication and indirect communication.

Proposal 2: Introduce AIOTF related Transfer IEs in NGAP.

The corresponding TP to BL CR of TS 38.300 is proposed in section 5.

# Reference

[1] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR [Ambient_IoT_Solutions]

[2] 3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things) V2.0.1 (2024-12)

[3]	3GPP TR 23.700-13 Study on Architecture support of Ambient power-enabled Internet of Things (Release 19) V1.2.1 (2024-11)

# Text Proposal to TS 38.300 BL CR

---------------Start of the First Change---------------

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A2X	Aircraft-to-Everything

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AI	Artificial Intelligence

A-IoT	Ambient IoT

AIOTF	Ambient IoT	Function

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

//skip unchanged part

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

IFRI	Intra Frequency Reselection Indication

IOT	Internet of Things

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

L2	Layer-2

L3	Layer-3

---------------Start of the Next Change---------------

# xx	Ambient IoT

## xx.x	Network Architecture

To enable Ambient IoT operations, the gNB serves one or more readers, connects by means of the NG-C interface (refer to 4.3.1.2) to the 5GC, more specifically:

-	direct communication: gNB communicates with AIOTF directly, or;

-	indirect communication: gNB communicates with AIOTF indirectly via AMF.

NOTE 1:	It is not expected a deployment will use both direct communication and indirect communication.

In case of direct communication, the gNB communicates with AIOTF directly via NG-C interface, as illustrated in Figure xx.x-1 as below:

Figure xx.x-1 NG-C Protocol Stack in direct communication

In case of the indirect communication, the AIOTF Information is exchanged between the gNB and the AIOTF transparently via the AMF, as illustrated in Figure xx.x-2 as below:

Figure xx.x-2 NG-C Protocol Stack in indirect communication

NOTE 2:	For the interface and protocol stack between the AMF and the AIOTF is defined in TS 23.xxx [xx].

---------------End of the Changes---------------

# Text Proposal to TS 38.410 BL CR

---------------Start of the First Change---------------

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AIOTF: as defined in TS 23.501 [8].

gNB: as defined in TS 38.300 [7].

NB-IoT: as defined in TS36.300 [11].

ng-eNB: as defined in TS 38.300 [7].

NG-RAN node: as defined in TS 38.300 [7].

UPF: as defined in TS 23.501 [8].

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

A-IoT	Ambient IoT

AIOTF	Ambient IoT Function

AMF	Access and Mobility Management Function

CIoT	Cellular IoT

CN	Core Network

DRX	Discontinuous Reception

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

NB-IoT	Narrow Band Internet of Things

NG-U	NG User plane interface

PTP	Point to Point

PTM	Point to Multipoint

QMC	QoE Measurement Collection

QoE	Quality of Experience

RIM	Remote Interference Management

SMF	Session Management Function

UP	User Plane

UPF	User Plane Function

# 4	General aspects

## 4.1	NG Architecture

The NG-RAN is layered into a Radio Network Layer (RNL) and a Transport Network Layer (TNL). The NG-RAN architecture, i.e. the NG-RAN logical nodes and interfaces between them, are defined as part of the RNL.

The NG-RAN architecture consists of a set of gNBs and ng-eNBs which are connected to the 5GC through the NG interface and is specified in TS 38.300 [7].

The NG-RAN may have several NG access points towards the 5GC. As a minimum, each NG access point 
(in NG-RAN or 5GC) shall independently fulfil the requirements of the relevant NG specifications (3GPP 38.41x series - see clause 7).

NG is a logical interface.

There may be multiple NG-C logical interfaces towards the 5GC from any one NG-RAN node. The selection of the NG-C interface is then determined by the NAS Node Selection function as described in clause 5.

There may be multiple NG-U logical interfaces towards the 5GC from any one NG-RAN node. The selection of the NG-U interface is done within the 5GC and signalled to the NG-RAN node by the AMF.

## 4.2	NG interface general principles

The general principles for the specification of the NG interface are as follows:

-	the NG interface is open;

-	the NG interface supports the exchange of signalling information between the NG-RAN and 5GC;

-	from a logical standpoint, the NG is a point-to-point interface between an NG-RAN node and a 5GC node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the NG-RAN and 5GC;

-	the NG interface supports control plane and user plane separation;

-	the NG interface separates Radio Network Layer and Transport Network Layer;

-	the NG interface is future proof to fulfil different new requirements and support of new services and new functions;

-	the NG interface is decoupled with the possible NG-RAN deployment variants;

-	the NG Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding /decoding efficiency.

## 4.3	NG interface specification objectives

The NG interface specification facilitates the following:

-	inter-connection of NG-RAN nodes with AMFs supplied by different manufacturers;

-	inter-connection of NG-RAN nodes with AIOTFs supplied by different manufacturers;

-	separation of NG interface Radio Network functionality and Transport Network functionality to facilitate introduction of future technology.

## 4.4	NG interface capabilities

The NG interface supports:

-	procedures to establish, maintain and release NG-RAN part of PDU sessions;

-	procedures to perform intra-RAT handover and inter-RAT handover;

-	the separation of each UE on the protocol level for user specific signalling management;

-	the transfer of NAS signalling messages between UE and AMF;

-	mechanisms for resource reservation for packet data streams;

-	procedures to establish, maintain and release NG-RAN part of MBS sessions;

-	the transfer of A-IoT NAS signalling messages between device and AIOTF;

-	procedures to establish, maintain and release NG-RAN part of A-IoT services.

---------------End of the Change---------------