3GPP TSG-RAN WG3 Meeting #127	R3-250198

Athens, Greece, 17-21 Feb, 2025

Title:	(TP for TS 38.413 BL CR) NG Interface Management for Ambient IoT

Agenda Item:	16.2

Source:	Huawei, China Telecom, China Unicom

Document for:	other

# Introduction

In RAN#106 meeting, the New Work Item: Solutions for Ambient IoT (Internet of Things) in NR [1] was agreed, and RAN3 needs to specify the related aspects in the following RAN3 meetings.

In both direct path communication (gNB communicates with AIOTF directly via NG interface) and indirect path communication (gNB communicates with AIOTF indirectly via AMF), NG interface is used, in this contribution, we discuss the management of NG interface.

# Discussion

# Interface Management Procedures

Currently, the NG-RAN node initiates the NG Setup procedure, and in case of indirect path communication, the gNB providing A-IoT radio connects with AMF via NG interface, therefore, existing NG interface management procedures should be reused/enhanced to support indirect path communication.

Proposal 1: Existing NG interface management procedures are used in indirect path communication.

For direct path communication, the gNB needs to establish NG interface with the AIOTF, in order to have unified handling at gNB side for both direct communication and indirect path communication, we slightly preferred to also use existing NG interface management procedures between the gNB and the AIOTF.

In current NGAP specification, there are following interface management procedures:

- NG Setup

- RAN Configuration Update

- NG Reset

- Error Indication

- AMF Configuration Update

- AMF Status Indication

- Overload Start

- Overload Stop

The procedure 5) ~ 8) are AMF specific procedures and not applicable to AIOTF, for direct path communication, we can reuse/enhance the procedure 1) ~ 4) between gNB and AIOTF.

Proposal 2: Existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used to support direct path communication.

# A-IoT Information provided by the gNB to 5GC

In case a gNB only support A-IoT radio, some of the existing IEs in NG SETUP REQUEST message and RAN CONFIGURATION UPDATE message may not applicable, e.g., Supported TAs, Default Paging DRX, etc. We can introduce an optional Only Support A-IoT IE to indicate that these not applicable IEs should be ignored.

Similarly, an optinal IE is also needed to be introduced in NG SETUP RESPONSE message to indicate that some IEs in that message are not applicable.

Proposal 3: Introduce an optional Only Support A-IoT IE in NG SETUP REQUEST message and RAN CONFIGURATION UPDATE message to indicate that the IEs not applicable to A-IoT should be ignored.

Proposal 3a: In indirect communication, introduce an option IE in NG SETUP RESPONSE message to indicate that some IEs in that message are not applicable.

Currently, in the above mentioned two messages, the gNB provides to the 5GC about its Global RAN Node ID, Name, Supported TA List, etc, in case the gNB serves A-IoT readers, the gNB can also provide A-IoT Information to the core network in these two messages, i.e., introduce A-IoT RAN Information Transfer IE in these two messages as optional IE, detailed information (e.g., the supported Inventory Area IDs, the supported readers) within the A-IoT RAN Information Transfer IE can be further discussed.

Proposal 4: Introduce an optional A-IoT RAN Information Transfer IE in NG SETUP REQUEST message and RAN CONFIGURATION UPDATE message to carry A-IoT related information.

For the detailed content of the above A-IoT RAN Information Transfer IE, it is related to the following statements of Topology 1 in the TR:

Reader selection may need coordination between A-IoT RAN node and A-IoT CN. NOTE 5: How to perform reader selection needs further discussions. NOTE 6: In Topology 1, whether the AIoTF needs to be aware of the readers served by the A-IoT RAN node and their location and how this information is provided to the AIoTF needs further discussion.

Based on the discussion in the study, within the Inventory Request, the CN will provide the scope of inventory request (e.g., a certain area in which the inventory is to be triggered), but from our understanding, before that, in NG Setup Request, the gNB needs to provide the supported Inventory Area ID List and or the Served Reader List to the core network.

About the AIOTF awareness of readers, some companies consider that in legacy, the eNB and the gNB do not report its cell list to the CN in S1/NG Setup therefore not prefer to provide reader list to the CN, and some other companies consider that in positioning, the gNB reports its TRP list and location of the TRPs to the CN.

Considering that reader id granularity device locating report is to be supported by the network, the AIOTF has to know the location of the readers, via either Reader list and location info in NG Setup Request, or reader id and location in Inventory Report. And report reader location in each inventory report repeatly is not as efficient as report once in the NG Setup Request.

Proposal 5: AIOTF needs to be aware of readers and their location.

Based on the latest version of SA2 TR [3], the following can be found:

b. For topology 1, the AIOTF selects AIOT RAN nodes and optionally a list of the BS readers. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN. NOTE 1: Providing the UE Reader list to the RAN only applies to the RRC-based option. For topology 1, NGAP is terminated at an AIOT RAN node. An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g., BS reader IDs and their corresponding service areas. The OAM may configure the AIOTF with BS Reader information including e.g., their corresponding service area when the BS Readers are integrated in the network. NOTE 2: Whether RAN3 will specify BS reader information (e.g. BS reader ID, service area) reporting to AIOTF over NGAP depends on RAN3 decision and if so SA2 may include this feature in SA2 specifications. NOTE 3: If the request from the AIOTF to the AIOT RAN does not include a BS reader list, then AIOT RAN can use all available BS readers. NOTE 4: The BS reader’s service area needs to coordinate with RAN WG(s).

Therefore, the AIOTF will be able to get the reader ids and corresponding service areas, from our understanding, to allow different communications and implementations, it is beneficial for the gNB to report the Supported Inventory Area IDs and the Reader ID List together with the reader location in the A-IoT RAN Information Transfer IE in NG SETUP REQUEST message and RAN CONFIGURATION UPDATE message, or the OAM configures these RAN related information to the AIOTF.

Propoal 6: gNB reports both the Supported Inventory Area IDs and the Reader ID List together with the reader location in the A-IoT RAN Information Transfer IE to the AIOTF, and/or the OAM configures these RAN related information to the AIOTF.

This also related to the LS from SA2 in [4], and the draft reply LS is provided in [8], the content of the draft reply LS can further consider the progress in this discussion.

# 3	Conclusion

In this contribution, we discuss the management of NG interface, get the following proposals:

Proposal 1: Existing NG interface management procedures are used in indirect path communication.

Proposal 2: Existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used to support direct path communication.

Proposal 3: Introduce an optional Only Support A-IoT IE in NG SETUP REQUEST message and RAN CONFIGURATION UPDATE message to indicate that the IEs not applicable to A-IoT should be ignored if received.

Proposal 3a: In indirect communication, introduce an option IE in NG SETUP RESPONSE message to indicate that some IEs in that message are not applicable.

Proposal 4: Introduce an optional A-IoT RAN Information Transfer IE in NG SETUP REQUEST message and RAN CONFIGURATION UPDATE message to carry A-IoT related information.

Proposal 5: AIOTF needs to be aware of readers and their location.

Propoal 6: gNB to report both the Supported Inventory Area IDs and the Reader ID List together with the reader location in the A-IoT RAN Information Transfer IE, and/or the OAM configures these RAN related information, to the AIOTF.

The corresponding TP to BL CR of TS 38.413 is proposed in section 5.

# 4.	Reference

[1] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR [Ambient_IoT_Solutions]

[2] 3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things) V2.0.1 (2024-12)

[3]	3GPP TR 23.700-13 Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)

[4] S2-2501343 LS on SA2 Ambient IoT TR conclusion update, Source: SA2, To: RAN3

[5] R3-250196 [Draft] Reply LS on SA2 Ambient IoT TR conclusion update, Source: Huawei [to be RAN3], To: SA2

# 5.	Text Proposal to TS 38.413 BL CR

---------------Start of the First Change---------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

//Skip unchanged part

[57]	3GPP TS 23.527: "5G System; Restoration procedures".

[58]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[xx]	3GPP TS23.xxx: “Architecture support for Ambient power-enabled Internet of Things; Stage 2”

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ACL functionality: as defined in TS 36.413 [16].

CAG cell: as defined in TS 38.300 [8].

DAPS Handover: as defined in TS 38.300 [8].

Elementary Procedure: NGAP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between the NG-RAN node and the AMF, or between the NG-RAN node and the AIOTF. These Elementary Procedures are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as standalone procedures, which can be active in parallel. The usage of several NGAP EPs together or together with EPs from other interfaces is specified in stage 2 specifications (e.g., TS 38.401 [2], TS 38.410 [3] and TS 38.300 [8]).

---------------Start of the First Change---------------

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

ACL		Access Control List

A-IoT	Ambient IoT

AIOTF	Ambient IoT	 Function

AMF	Access and Mobility Management Function

AUN3	Authenticable Non-3GPP

CAG	Closed Access Group

CGI	Cell Global Identifier

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DC	Dual Connectivity

DL	Downlink

EPC	Evolved Packet Core

FN-RG	Fixed Network Residential Gateway

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber-Coax

IAB	Integrated Access and Backhaul

IOT	Internet of Things

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

N3IWF	Non 3GPP InterWorking Function

---------------Start of the Next Change---------------

# 5	NGAP Services

NGAP provides the signalling service between the NG-RAN node and the AMF, or between the NG-RAN node and the AIOTF that is required to fulfil the NGAP functions described in TS 38.410 [3]. NGAP services are divided into two groups:

Non UE-associated services:	They are related to the whole NG interface instance between the NG-RAN node and AMF, or between the NG-RAN node and the AIOTF, utilising a non UE-associated signalling connection.

UE-associated services:	They are related to one UE. NGAP functions that provide these services are associated with a UE-associated signalling connection that is maintained for the UE in question.

---------------Start of the Next Change---------------

## 8.7	Interface Management Procedures

### 8.7.1	NG Setup

#### 8.7.1.1	General

The purpose of the NG Setup procedure is to exchange application level data needed for the NG-RAN node and the AMF, or between the NG-RAN node and the AIOTF, to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes, replaces it by the one received and clears AMF overload state information at the NG-RAN node. If the NG-RAN node and AMF do not agree on retaining the UE contexts this procedure also re-initialises the NGAP UE-related contexts (if any) and erases all related signalling connections in the two nodes like an NG Reset procedure would do.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation with AMF

Figure 8.7.1.2-2: NG setup: successful operation with AIOTF

The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF or the AIOTF. The AMF or the AIOTF responds with an NG SETUP RESPONSE message including the appropriate data.

//Skip unchanged part

If the AMF supports mobile IAB, the AMF shall include the Mobile IAB Supported IE in the NG SETUP RESPONSE message. If the Mobile IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and further use it for AMF selection for the mobile IAB-MT.

In case the NG-RAN node provides A-IoT radio, the NG-RAN node shall, if supported, inlcude the A-IoT RAN Information Transfer IE in the NG SETUP REQUEST message. Upon receiving the A-IoT RAN Information Transfer IE, the core network shall, if supported, use this information as specified in TS 23.xxx [xx].

Upon receiving the Only Support A-IoT IE in the NG SETUP REQUEST message, the core network shall, if supported, consider that the NG-RAN node only provides A-IoT radio, and ignore the not applicable IEs as specified in clause 9.2.6.1.

In case of NG Setup with AIOTF, upon receiving AIOTF Identifier IE in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, ignore the IEs not applicable for AIOTF as specified in clause 9.2.6.2.

#### 8.7.1.3	Unsuccessful Operation

Figure 8.7.1.3-1: NG setup: unsuccessful operation with AMF

Figure 8.7.1.3-2: NG setup: unsuccessful operation with AIOTF

If the AMF or the AIOTF cannot accept the setup, it should respond with an NG SETUP FAILURE message and appropriate cause value.

If the NG SETUP FAILURE message includes the Time to Wait IE, the NG-RAN node shall wait at least for the indicated time before reinitiating the NG Setup procedure towards the same AMF.

#### 8.7.1.4	Abnormal Conditions

If the AMF does not identify any of the PLMNs/SNPNs indicated in the NG SETUP REQUEST message, it shall reject the NG Setup procedure with an appropriate cause value.

If none of the RATs indicated by the NG-RAN node in the NG SETUP REQUEST message is supported by the AMF or the AIOTF, then the AMF or the AIOTF shall fail the NG Setup procedure with an appropriate cause value.

### 8.7.2	RAN Configuration Update

#### 8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF, or the NG-RAN node and the AIOTF, to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling.

#### 8.7.2.2	Successful Operation

Figure 8.7.2.2-1: RAN configuration update: successful operation with AMF

Figure 8.7.2.2-2: RAN configuration update: successful operation with AIOTF

The NG-RAN node initiates the procedure by sending a RAN CONFIGURATION UPDATE message to the AMF or the AIOTF including an appropriate set of updated configuration data that it has just taken into operational use. The AMF or the AIOTF responds with a RAN CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the RAN CONFIGURATION UPDATE message, the AMF or the AIOTF shall interpret that the corresponding configuration data is not changed and shall continue to operate the NG-C interface with the existing related configuration data.

//Skip unchanged part

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the A-IoT RAN Information Transfer IE in the RAN CONFIGURATION UPDATE message, the core network shall overwrite all the previously stored A-IoT RAN information, if any.

Upon receiving the Only Support A-IoT IE in the RAN CONFIGURATION UPDATE message, the core network shall, if supported, consider that the NG-RAN node only provides A-IoT radio, and ignore the the not applicable IEs as specified in clause 9.2.6.4.

#### 8.7.2.3	Unsuccessful Operation

Figure 8.7.2.3-1: RAN configuration update: unsuccessful operation with AMF

Figure 8.7.2.3-1: RAN configuration update: unsuccessful operation with AIOTF

If the AMF or the AIOTF cannot accept the update, it shall respond with a RAN CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the Time to Wait IE is included in the RAN CONFIGURATION UPDATE FAILURE message, the NG-RAN node shall wait at least for the indicated time before reinitiating the RAN Configuration Update procedure towards the same AMF.

#### 8.7.2.4	Abnormal Conditions

If the NG-RAN node, after initiating the RAN Configuration Update procedure, receives neither a RAN CONFIGURATION UPDATE ACKOWLEDGE nor a RAN CONFIGURATION UPDATE FAILURE message, the NG-RAN node may reinitiate a further RAN Configuration Update procedure towards the same AMF or the AIOTF, provided that the content of the new RAN CONFIGURATION UPDATE message is identical to the content of the previously unacknowledged RAN CONFIGURATION UPDATE message.

---------------Start of the Next Change---------------

### 8.7.4	NG Reset

#### 8.7.4.1	General

The purpose of the NG Reset procedure is to initialise or re-initialise the RAN, or part of RAN NGAP UE-related contexts, in the event of a failure in the 5GC or vice versa. This procedure does not affect the application level configuration data exchanged during, e.g., the NG Setup procedure. The procedure uses non-UE associated signalling.

#### 8.7.4.2	Successful Operation

##### 8.7.4.2.1	NG Reset initiated by the AMF or the AIOTF

Figure 8.7.4.2.1-1: NG reset initiated by the AMF: successful operation

Figure 8.7.4.2.1-2: NG reset initiated by the AIOTF: successful operation

In the event of a failure at the AMF or the AIOTF which has resulted in the loss of some or all transaction reference information, an NG RESET message shall be sent to the NG-RAN node.

At reception of the NG RESET message the NG-RAN node shall release all allocated resources on NG and Uu related to the UE association(s) indicated explicitly or implicitly in the NG RESET message and remove the indicated UE contexts including NGAP ID.

After the NG-RAN node has released all assigned NG resources and the UE NGAP IDs for all indicated UE associations which can be used for new UE-associated logical NG-connections over the NG interface, the NG-RAN node shall respond with the NG RESET ACKNOWLEDGE message. The NG-RAN node does not need to wait for the release of radio resources to be completed before returning the NG RESET ACKNOWLEDGE message.

If the NG RESET message contains the UE-associated Logical NG-connection List IE, then:

-	The NG-RAN node shall use the AMF UE NGAP ID IE and/or the RAN UE NGAP ID IE to explicitly identify the UE association(s) to be reset.

-	The NG-RAN node shall include in the NG RESET ACKNOWLEDGE message, for each UE association to be reset, the UE-associated Logical NG-connection Item IE in the UE-associated Logical NG-connection List IE. The UE-associated Logical NG-connection Item IEs shall be in the same order as received in the NG RESET message and shall include also unknown UE-associated logical NG-connections. Empty UE-associated Logical NG-connection Item IEs, received in the NG RESET message, may be omitted in the NG RESET ACKNOWLEDGE message.

-	If the AMF UE NGAP ID IE is included in the UE-associated Logical NG-connection Item IE for a UE association, the NG-RAN node shall include the AMF UE NGAP ID IE in the corresponding UE-associated Logical NG-connection Item IE in the NG RESET ACKNOWLEDGE message.

-	If the RAN UE NGAP ID IE is included in the UE-associated Logical NG-connection Item IE for a UE association, the NG-RAN node shall include the RAN UE NGAP ID IE in the corresponding UE-associated Logical NG-connection Item IE in the NG RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the NG RESET message is received, any other ongoing procedure (except for another NG Reset procedure) on the same NG interface related to a UE association, indicated explicitly or implicitly in the NG RESET message, shall be aborted.

##### 8.7.4.2.2	NG Reset initiated by the NG-RAN node

Figure 8.7.4.2.2-1: NG reset initiated by the NG-RAN node: successful operation with AMF

Figure 8.7.4.2.2-2: NG reset initiated by the NG-RAN node: successful operation with AIOTF

In the event of a failure at the NG-RAN node which has resulted in the loss of some or all transaction reference information, an NG RESET message shall be sent to the AMF and the AIOTF.

At reception of the NG RESET message the AMF shall release all allocated resources on NG related to the UE association(s) indicated explicitly or implicitly in the NG RESET message and remove the NGAP ID for the indicated UE associations.

At reception of the NG RESET message the AIOTF shall release all allocated NG resources.

After the AMF has released all assigned NG resources and the UE NGAP IDs for all indicated UE associations which can be used for new UE-associated logical NG-connections over the NG interface, the AMF shall respond with the NG RESET ACKNOWLEDGE message.

If the NG RESET message contains the UE-associated Logical NG-connection List IE, then:

-	The AMF shall use the AMF UE NGAP ID IE and/or the RAN UE NGAP ID IE to explicitly identify the UE association(s) to be reset.

-	The AMF shall include in the NG RESET ACKNOWLEDGE message, for each UE association to be reset, the UE-associated Logical NG-connection Item IE in the UE-associated Logical NG-connection List IE. The UE-associated Logical NG-connection Item IEs shall be in the same order as received in the NG RESET message and shall include also unknown UE-associated logical NG-connections. Empty UE-associated Logical NG-connection Item IEs, received in the NG RESET message, may be omitted in the NG RESET ACKNOWLEDGE message.

-	If the AMF UE NGAP ID IE is included in the UE-associated Logical NG-connection Item IE for a UE association, the AMF shall include the AMF UE NGAP ID IE in the corresponding UE-associated Logical NG-connection Item IE in the NG RESET ACKNOWLEDGE message.

-	If the RAN UE NGAP ID IE is included in a UE-associated Logical NG-connection Item IE for a UE association, the AMF shall include the RAN UE NGAP ID IE in the corresponding UE-associated Logical NG-connection Item IE in the NG RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the NG RESET message is received, any other ongoing procedure (except for another NG Reset procedure) on the same NG interface related to a UE association, indicated explicitly or implicitly in the NG RESET message, shall be aborted.

#### 8.7.4.3	Unsuccessful Operation

Not applicable.

#### 8.7.4.4	Abnormal Conditions

##### 8.7.4.4.1	Abnormal Condition at the 5GC

If the NG RESET message includes the UE-associated Logical NG-connection List IE, but neither the AMF UE NGAP ID IE nor the RAN UE NGAP ID IE is present for a UE-associated Logical NG-connection Item IE, then the AMF shall ignore the UE-associated Logical NG-connection Item IE. The AMF may return the empty UE-associated Logical NG-connection Item IE in the UE-associated Logical NG-connection List IE in the NG RESET ACKNOWLEDGE message.

##### 8.7.4.4.2	Abnormal Condition at the NG-RAN

If the NG RESET message includes the UE-associated Logical NG-connection List IE, but neither the AMF UE NGAP ID IE nor the RAN UE NGAP ID IE is present for a UE-associated Logical NG-connection Item IE, then the NG-RAN node shall ignore the UE-associated Logical NG-connection Item IE. The NG-RAN node may return the empty UE-associated Logical NG-connection Item IE in the UE-associated Logical NG-connection List IE in the NG RESET ACKNOWLEDGE message.

##### 8.7.4.4.3	Crossing of NG RESET Messages

If an NG Reset procedure is ongoing in the NG-RAN node and the NG-RAN node receives an NG RESET message from the peer entity on the same NG interface related to one or several UE associations previously requested to be reset, indicated explicitly or implicitly in the received NG RESET message, the NG-RAN node shall respond with the NG RESET ACKNOWLEDGE message as described in 8.7.4.2.1.

If an NG Reset procedure is ongoing in the AMF and the AMF receives an NG RESET message from the peer entity on the same NG interface related to one or several UE associations previously requested to be reset, indicated explicitly or implicitly in the received NG RESET message, the AMF shall respond with the NG RESET ACKNOWLEDGE message as described in 8.7.4.2.2.

If an NG Reset procedure is ongoing in the AIOTF and the AIOTF receives an NG RESET message from the peer entity on the same NG interface, the AIOTF shall respond with the NG RESET ACKNOWLEDGE message as described in 8.7.4.2.2.

### 8.7.5	Error Indication

#### 8.7.5.1	General

The Error Indication procedure is initiated by a node in order to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

If the error situation arises due to reception of a message utilising UE-associated signalling, then the Error Indication procedure uses UE-associated signalling. Otherwise the procedure uses non-UE associated signalling.

#### 8.7.5.2	Successful Operation

Figure 8.7.5.2-1: Error indication initiated by the AMF

Figure 8.7.5.2-2: Error indication initiated by the NG-RAN node to the AMF

Figure 8.7.5.2-3: Error indication initiated by the AIOTF

Figure 8.7.5.2-4: Error indication initiated by the NG-RAN node to the AIOTF

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the receiving node.

The ERROR INDICATION message shall contain at least either the Cause IE or the Criticality Diagnostics IE.

In case the Error Indication procedure is triggered by utilising UE-associated signalling the AMF UE NGAP ID IE and the RAN UE NGAP ID IE shall be included in the ERROR INDICATION message. If one or both of the AMF UE NGAP ID IE and the RAN UE NGAP ID IE are not correct, the cause shall be set to an appropriate value, e.g., "Unknown local UE NGAP ID" or "Inconsistent remote UE NGAP ID".

#### 8.7.5.3	Abnormal Conditions

Void.

---------------Start of the Next Change---------------

### 9.2.6	Interface Management Messages

#### 9.2.6.1	NG SETUP REQUEST

This message is sent by the NG-RAN node to transfer application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF, NG-RAN node  AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 1 |  | Supported TAs in the NG-RAN node. This IE is ignored if the Only Support A-IoT IE is included. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | M |  | Paging DRX 9.3.1.90 | This IE is ignored if the Only Support A-IoT IE is included. | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| A-IoT RAN Information Transfer | O |  | OCTET STRING | Containing the A-IoT RAN Information Transfer IE specified in subclause 9.3.x.1. | YES | reject |
| Only Support A-IoT | O |  | ENUMERATED (true, …) | This IE indicates that the NG-RAN node only provides A-IoT radio. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |

#### 9.2.6.2	NG SETUP RESPONSE

This message is sent by the AMF or AIOTF to transfer application layer information for an NG-C interface instance.

Direction: AMF  NG-RAN node, AIOTF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 | This IE is ignored if the AIOTF Name IE is included. | YES | reject |
| Served GUAMI List |  | 1 |  | This IE is ignored if the AIOTF Name IE is included. | YES | reject |
| >Served GUAMI Item |  | 1..<maxnoofServedGUAMIs> |  |  | - |  |
| >>GUAMI | M |  | 9.3.3.3 |  | - |  |
| >>Backup AMF Name | O |  | AMF Name 9.3.3.21 |  | - |  |
| >>GUAMI Type | O |  | ENUMERATED (native, mapped, …) |  | YES | ignore |
| Relative AMF Capacity | M |  | 9.3.1.32 |  | YES | ignore |
| PLMN Support List |  | 1 |  |  | YES | reject |
| >PLMN Support Item |  | 1..<maxnoofPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>Slice Support List | M |  | 9.3.1.17 | Supported S-NSSAIs per PLMN or per SNPN. | - |  |
| >>NPN Support | O |  | 9.3.3.44 | If NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>Extended Slice Support List | O |  | 9.3.1.191 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >>Onboarding Support | O |  | ENUMERATED (true, ...) | Indication of onboarding support. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for IAB. | YES | ignore |
| Extended AMF Name | O |  | 9.3.3.51 |  | YES | ignore |
| Mobile IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for mobile IAB. | YES | ignore |
| AIOTF Identifier | O |  | FFS |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedGUAMIs | Maximum no. of GUAMIs served by an AMF. Value is 256. |
| maxnoofPLMNs | Maximum no. of PLMNs per message. Value is 12. |

#### 9.2.6.3	NG SETUP FAILURE

This message is sent by the AMF or the AIOTF to indicate NG setup failure.

Direction: AMF  NG-RAN node, AIOTF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to Wait | O |  | 9.3.1.56 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF, NG-RAN node  AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| A-IoT RAN Information Transfer | O |  | OCTET STRING | Containing the A-IoT RAN Information Transfer IE specified in subclause 9.3.x.1. | YES | reject |
| Only Support A-IoT | O |  | ENUMERATED (true, …) | This IE indicates that the NG-RAN node only provides A-IoT radio. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |

#### 9.2.6.5	RAN CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the AMF or the AIOTF to acknowledge the NG-RAN node transfer of updated information for an NG-C interface instance.

Direction: AMF  NG-RAN node, AIOTF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.6.6	RAN CONFIGURATION UPDATE FAILURE

This message is sent by the AMF or the AIOTF to indicate RAN configuration update failure.

Direction: AMF  NG-RAN node, AIOTF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to Wait | O |  | 9.3.1.56 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

---------------Start of the Next Change---------------

#### 9.2.6.11	NG RESET

This message is sent by both the NG-RAN node and the AMF to request that the NG interface, or parts of the NG interface, be reset.

In case the gNB communicates with the AIOTF directly, this message is also sent by both the NG-RAN node and the AIOTF to request that the NG interface be reset.

Direction: NG-RAN node  AMF and AMF  NG-RAN node, NG-RAN node  AIOTF and AIOTF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| CHOICE Reset Type | M |  |  |  | YES | reject |
| >NG interface |  |  |  |  |  |  |
| >>Reset All | M |  | ENUMERATED (Reset all, …) |  | - |  |
| >Part of NG interface |  |  |  |  |  |  |
| >>UE-associated Logical NG-connection List | M |  | 9.3.3.25 |  | - |  |

#### 9.2.6.12	NG RESET ACKNOWLEDGE

This message is sent by both the NG-RAN node and the AMF, or both the NG-RAN node and the AIOTF, as a response to an NG RESET message.

Direction: NG-RAN node  AMF and AMF  NG-RAN node, NG-RAN node  AIOTF and AIOTF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| UE-associated Logical NG-connection List | O |  | 9.3.3.25 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.6.13	ERROR INDICATION

This message is sent by both the NG-RAN node and the AMF, or both the NG-RAN node and the AIOTF, to indicate that some error has been detected in the node.

Direction: NG-RAN node  AMF and AMF  NG-RAN node, NG-RAN node  AIOTF and AIOTF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | O |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | O |  | 9.3.3.2 |  | YES | ignore |
| Cause | O |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| 5G-S-TMSI | O |  | 9.3.3.20 |  | YES | ignore |

---------------Start of the Next Change---------------

### 9.3.x	AIOTF Related IEs

#### 9.3.x.1	A-IoT RAN Information Transfer

This IE is transparent to the AMF in case of indirect path communication.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Supported Inventory Area List |  | 1 | Containing the full list of supported Inventory Areas. |  |
| Supported Inventory Area Item |  | 1..<maxnoofInventoryAreas> |  |  |
| >Inventory Area Code | M |  | FFS |  |
| >>Supported Reader List |  | 1 |  |  |
| >>>Supported Reader Item |  | 1..<maxnoofReaders> |  |  |
| >>>>Reader Identity | M |  | FFS |  |
| >>>>Reader Location Information | O |  | FFS |  |
| >PLMN Support List |  | 1 |  |  |
| >>PLMN Support Item |  | 1..<maxnoofPLMNs> |  |  |
| >>>PLMN Identity | M |  | 9.3.3.5 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofInventoryAreas | Maximum no. of Inventory Areas supported by a gNB. Value is 256. |
| maxnoofReaders | Maximum no. of Readers served by a gNB. Value is 256. |
| maxnoofPLMNs | Maximum no. of PLMNs per message. Value is 12. |