3GPP TSG-RAN WG3 Meeting #127	R3-250199

Athens, Greece, 17-21 Feb, 2025

Title:	(TPs to TS 38.413 TS 38.410 BL CRs) A-IoT Inventory procedures

Agenda Item:	16.2

Source:	Huawei, China Telecom, China Unicom

Document for:	other

# Introduction

In RAN WID [2], RAN3 scope is concluded:

R19 RAN WID: - RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

Based on the study item progress and R19 RAN WID, this contribution futher discuss the signaling and procedures over CN-RAN interface to support A-IoT Inventory, and provide the corresponding TPs.

# Discussion

## 2.1	Inventory related procedures

As captured in current RAN TR 38.769 [1], the A-IoT Inventory procedures include one Class 1 Inventory Request procedure and one or multiple Class 2 Inventory Report procedure(s) over NG interface.

Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1

Therefore, we need to introduce a Class 1 Inventory Request procedure, including INVENTORY REQUEST, INVENTORY RESPONSE and INVENTORY FAILURE messages, and a Class 2 Inventory Report procedure, including INVENTORY REPORT messages.

- Proposal 1: Introduce a Class 1 Inventory Request procedure, including INVENTORY REQUEST, INVENTORY RESPONSE and INVENTORY FAILURE messages

- Proposal 2: Introduce a Class 2 Inventory Report procedure, including INVENTORY REPORT message.

## 2.2	IEs in Inventory related messages

- 2.2.1		Correlation Identifier IE

In the up-to-date SA2 TR 23.700-13 [3], there are the following conclusions about A-IoT Inventory/Command Procedures:

SA2 TR 13.700-13: All the procedures follow have the following steps: 1. The AF makes a service request to the NEF, including parameters as defined in clause 8.3.5. 2. The NEF determines an AIOTF for the requested operation, and invokes a new service operation on the AIOTF. 3. For the requested operation the AIOTF: … 4). Constructs a request for an Inventory operation using the determined A-IoT Device Identification information page the AIoT Devices, and a correlation identifier for the AIOTF to correlate the inventory responses to the request. The Inventory request is routed to the Readers determined by the initial reader selection. 5). The Reader executes the inventory request, reporting AIoT specific NAS message responses from the AIoT Device to the AIOTF, including its Reader ID and correlation identifier from the AIOTF. The Reader may aggregate results from multiple AIoT Devices in the responding messages. The AIOTF can determine which request the results are for using the correlation identifier. …

Considering that one AIOTF may connect with multiple application servers, different application servers may trigger Inventory procedures within a short period, and one gNB may connect with multiple A-IoT CN, different A-IoT CN may trigger Inventory procedures within a short period as well. To be more robust, we follow SA2 TR conclusion to include same correlation identifier provided by the AIOTF in all inventory messages for the same service, to allow parallel A-IoT transactions over NG interface.

- Proposal 3: Include Correlation Identifier IE in all NGAP Inventory related messages.

- 2.2.2		AIOTF Identifier IE

- There are following SA2 TR conclusions about indirect communication via AMF:

- SA2 TR 13.700-13: - 2. The AIoTF performs the following functionality. … f. When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF. … - The AMF shall be enhanced to support: - On the Nz reference point, the AMF supports Services which are used by an AIoTF for Ambient IoT Operations e.g. to send AIoT requests towards AIOT RAN and to receive AIoT responses from the AIOT RAN. - On the N2 reference point, the AMF supports sending AIOT information to the AIOT RAN (e.g. operation requests) and receiving responses from the AIOT RAN. - The AMF routes the AIoT messages between the AIOT RAN (over N2 reference point) and the AIOTF (over Nz reference point).

From SA2 TR conclusions, we can see that in indirect communication, the AMF needs to additionally provide an AIOTF identifier with the request from the AIOTF, such that the AMF can be routed back the requesting AIOTF. As one AMF may connects to multiple AIOTFs, the AIOTF allocated correlation identifier is not unified over the NG interface between the gNB and AMF. Based on SA2 TR conclusion, we need to include both correlation identifier and AIOTF identifier in the NGAP Inventory related messages, to allow parallel A-IoT transactions between gNB and AMF, and routing by the AMF. Note that this also applies to Command related message..

- Proposal 4: In indirect communication, besides the Correlation Identifier IE, the AIOTF Identifier IE should be included in the all NGAP Inventory/Command related messages.

- 2.2.3		Inventory Request Transfer IE in INVENTORY REQUEST message

In the INVENTORY REQUEST message, besides the Correlation Identifier IE, the AIOTF Identifier IE, the detailed information of the request should be included in the Inventory Request Transfer IE which is transparent to the AMF in case of indirect communication. From our understanding, this Inventory Request Transfer IE should at least include Device Identification for Paging IE, the Inventory Target information IE, and the Inventory Assistacne Information IE.

Proposal 5: The INVENTORY REQUEST message also includes an Inventory Request Transfer IE, which is transparent to the AMF in case of indirect communication.

Proposal 6: The Inventory Request Transfer IE contains at least the Device Identification for Paging IE, the Inventory Target information IE, and the Inventory Assistacne Information IE.

- Device Identification for Paging IE in the Inventory Request Transfer IE

- As captured in current RAN TR 38.769, A-IoT Device Identification can be included in INVENTORY REQUEST message, and the Device Identification is used to find a single device, a group of devices, or all devices. In R19 RAN WID, there is also the following description about Device Identification.

- R19 RAN WID: - RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. …

- It is understood that in this release, there is no need to include a list of identifiers in a single paging message. And in RAN TR and RAN2 LS [4], RAN2 agreed that the reader needs to know whether the inventory is for a single device or more than one device, when we take this into account with the WID, for the “more than one device” inventory, there should be a single group id, or no identifier.

- Observation: The Device Identification for Paging IE should be able to include a single device ID for single device inventory, to include a single group id for a group of devices inventory, this can be achieved by including two branches in a CHOICE type IE.

- For all devices inventory, there will be no identifier in paging message over radio, for NGAP signalling, there are two options:

- Option 1：Not include Device Identification for Paging IE

- Option 2:  Always include the Device Identification for Paging IE, and design the Device Identification for Paging IE with a NULL branch in case of all device Inventory

- From our view, as we need to have two branches to support single device inventory and group device inventory, it is easy to have a third NULL branch.

- Proposal 7: Define the Device Identification for Paging IE as CHOICE type, with three branches, first branch for single device inventory, second branch for a group device inventory, and third branch as NULL for all devices inventory.

- Inventory Target information IE in the Inventory Request Transfer IE

Based on RAN TR, the scope of inventory request is included in INVENTORY REQUEST message.

- RAN TR 38.769: The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following: (1) A-IoT Device Identification (to find a single device, a group of devices, or all devices) (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

- From our understanding, the AIOTF can provide inventory target information (e.g., inventory area identity list and/or reader list) to the gNB, so that the gNB selects proper RAN reader(s) according to the indicated inventory area identity list, or use the reader(s) according to the reader list if provided by CN.

- Proposal 8: The Inventory Target information IE includes a list of Inventory Area Identy and/or a list of Reader IDs.

- Inventory Assistacne Information IE in the Inventory Request Transfer IE

- In RAN2 part of RAN TR 38.769, the following information are listed as useful to be visible to the reader from CN:

- RAN TR 38.769: The following information are considered useful to be visible to the reader from CN: - The A-IoT service type (e.g., inventory, command) NOTE 3: It needs to be further discussed if more information on command type (e.g. read/write/disable) is useful. The reader needs to know whether a D2R response is expected in D2R direction and, if so, the expected D2R message size. It needs to be further discussed on the details on how the reader gets that information (related to the "expected D2R message size" discussion in clause 6.3.5). It needs to be further discussed on whether the command type, if needed, is explicit or inferred from the "expected D2R message size" if available. - Whether the service is targeted for one or more than one A-IoT devices - The approximate number of target A-IoT devices of this service

And in the up-to-date SA2 TR 23.700-13 [3], there are the following related conclusions:

- SA2 TR 23.700-13: The AF provides one or some of the following information: - Information about AIoT services - Service type indicating e.g., inventory or command (e.g. read or write) - Information to be used for AIoT reader selection - At least one of the UE reader ID or the target area information - Optionally, Information about the target AIoT device(s) - At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices. - Optionally, Information to be used for resource allocation that is detailed in TR 38.769 [8] - Approximate number of AIoT devices - Approximate D2R message size

RAN2 has sent LS to SA2 in R2-2409412, to ask whether CN can provide D2R message size to the reader. As described in SA2 LS reply S2-2413035 and conclusions captured in the up-to-date SA2 TR 23.700-13, there is no final conclusion on whether AIOTF can provide D2R message size if AF does not provide it. Therefore, from RAN3 perspective, the AIOTF can provide such information, the leftover issue is whether provide this information optionally or mandatorily.

About “whether a response is expected”, based on S2-2501241, there is no need to have such indication, as there will always be a response from the device in response to a command.

SA2 LS reply S2-2413035: Q1: Can the CN provide, to the reader, an estimate of the expected size of the following D2R message(s) in response to the service request? SA2 answer: If provided by AF, AIOTF provides the reader with the approximate D2R message size based on AF request. Whether the CN can determine the approximate D2R message size if not provided by AF is under discussion in SA2.

SA2 TR 13.700-13: The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work: Editor’s note: Final conclusions are assumed to be taken in coordination with RAN WGs. 1. A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality. e. The AIOTF may provide the following assistance information to RAN/UE Reader: - AIoT service type (e.g. Inventory, Command); - approximate number of AioT devices based on AF request; - approximate D2R message size based on AF request.

SA2 LS S2-2501241: Q1: Is it feasible for CN to provide the following information to the reader? - the service type of A-IoT (e.g. inventory, command) - whether the service is targeted for a single or multiple devices; - the approximate number of target devices (if available). SA2 answer: CN can provide the information listed above to the reader if available. Q2: Can there be a case where a D2R response is not expected for a R2D message for command service type? If so, can the CN indicate to the reader whether a message is expected in D2R direction in response to an R2D message for command service type (e.g., either explicitly or implicitly by indicating the response message size if available)? SA2 answer: No, a D2R response is always expected for a R2D message for command service type (e.g., the service data response for 'Read' service operation, and the operation result indication response for 'Write' or 'Disable' service operation).

- Proposal 9: The Inventory Assistacne Information IE also includes the information of “approximate number of target devices”and “expected D2R message size”.

- 2.2.3		Inventory Report Transfer IE in INVENTORY REPORT message

- In the INVENTORY REQUEST message, besides the Correlation Identifier IE, the AIOTF Identifier IE, the detailed information of the report should be included in the Inventory Report Transfer IE which is transparent to the AMF in case of indirect communication.

- From our understanding, this Inventory Request Transfer IE should at least be able to include a list of NAS PDUs (carries the Device ID from the device), and the report of reader id is subject to the discussion of agenda item 16.3.

- Proposal 10: The INVENTORY REPORT message also includes an Inventory Report Transfer IE, which is transparent to the AMF in case of indirect communication, and the Inventory Report Transfer IE contains a list of A-IoT NAS PDU (Device ID).

# Conclusion

In this contribution, we further discussed the A-IoT Inventory procedure over NG interface, and got the following proposals:

- Proposal 1: Introduce a Class 1 Inventory Request procedure, including INVENTORY REQUEST, INVENTORY RESPONSE and INVENTORY FAILURE messages

- Proposal 2: Introduce a Class 2 Inventory Report procedure, including INVENTORY REPORT message.

- Proposal 3: Include Correlation Identifier IE in all NGAP Inventory related messages.

- Proposal 4: In indirect communication, besides the Correlation Identifier IE, the AIOTF Identifier IE should be included in the all NGAP Inventory/Command related messages.

- Proposal 5: The INVENTORY REQUEST message also includes an Inventory Request Transfer IE, which is transparent to the AMF in case of indirect communication.

- Proposal 6: The Inventory Request Transfer IE contains at least the Device Identification for Paging IE, the Inventory Target information IE, and the Inventory Assistacne Information IE.

- Proposal 7: Define the Device Identification for Paging IE as CHOICE type, with three branches, first branch for single device inventory, second branch for a group device inventory, and third branch as NULL for all devices inventory.

- Proposal 8: The Inventory Target information IE includes a list of Inventory Area Identy and/or a list of Reader IDs.

- Proposal 9: The Inventory Assistacne Information IE also includes the information of “approximate number of target devices” and “expected D2R message size”.

- Proposal 10: The INVENTORY REPORT message also includes an Inventory Report Transfer IE, which is transparent to the AMF in case of indirect communication, and the Inventory Report Transfer IE contains a list of A-IoT NAS PDU (Device ID).

- The corresponding TPs to TS 38.410 BL CR and TS38.413 BL CR are provided in section 5 and 6.

# Reference

[1] 3GPP TR 38.769 V2.0.1, Study on solutions for ambient IoT (Internet of Things), 2024-12

- [2] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Dec. 2024

- [3] 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)

- [4] R2-2411232 Further LS on Assistance information from CN to the reader, Source: RAN2, To: SA2, Cc: RAN3

# TP for TS 38.410 BL CR

---------------Start of the First Change---------------

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AIOTF: as defined in TS 23.501 [8].

gNB: as defined in TS 38.300 [7].

NB-IoT: as defined in TS36.300 [11].

ng-eNB: as defined in TS 38.300 [7].

NG-RAN node: as defined in TS 38.300 [7].

UPF: as defined in TS 23.501 [8].

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

A-IoT	Ambient IoT

AIOTF	Ambient IoT Function

AMF	Access and Mobility Management Function

CIoT	Cellular IoT

CN	Core Network

DRX	Discontinuous Reception

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

NB-IoT	Narrow Band Internet of Things

NG-U	NG User plane interface

PTP	Point to Point

PTM	Point to Multipoint

QMC	QoE Measurement Collection

QoE	Quality of Experience

RIM	Remote Interference Management

SMF	Session Management Function

UP	User Plane

UPF	User Plane Function

---------------Start of the Next Change---------------

## 5.x	Inventory function

The Inventory function enables the 5GC to request the NG-RAN node to perform inventory over the A-IoT radio, and enables the NG-RAN node to report the outcome for the Inventory Session over the NG interface.

---------------Start of the Next Change---------------

## 6.x	Inventory Procedures

The following inventory procedures are used by the 5GC to request the NG-RAN node to perform inventory over the A-IoT radio, and to enable the NG-RAN node to report the outcome for the Inventory Session over the NG interface:

-	Inventory Request procedure;

-	Inventory Report procedure.

# TP for TS 38.413 BL CR

---------------Start of the First Change---------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

//Skip unchanged part

[57]	3GPP TS 23.527: "5G System; Restoration procedures".

[58]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[xx]	3GPP TS23.xxx: “Architecture support for Ambient power-enabled Internet of Things; Stage 2”

---------------Start of the Next Change---------------

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

ACL	Access Control List

A-IoT	Ambient IoT

AIOTF	Ambient IoT	 Function

AMF	Access and Mobility Management Function

AUN3	Authenticable Non-3GPP

CAG	Closed Access Group

CGI	Cell Global Identifier

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DC	Dual Connectivity

DL	Downlink

EPC	Evolved Packet Core

FN-RG	Fixed Network Residential Gateway

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber-Coax

IAB	Integrated Access and Backhaul

IOT	Internet of Things

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

N3IWF	Non 3GPP InterWorking Function

NB-IoT	Narrow Band Internet of Things

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NRPPa	NR Positioning Protocol Annex

---------------Start of the Next Change---------------

# 8	NGAP Procedures

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| MT Communication Handling | MT COMMUNICATION HANDLING REQUEST | MT COMMUNICATION HANDLING RESPONSE | MT COMMUNICATION HANDLING FAILURE |
| Broadcast Session Transport | BROADCAST SESSION TRANSPORT REQUEST | BROADCAST SESSION TRANSPORT RESPONSE | BROADCAST SESSION TRANSPORT FAILURE |
| Inventory Request | INVENTORY REQUEST | INVENTORY RESPONSE | INVENTORY FAILURE |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Session Release Required | BROADCAST SESSION RELEASE REQUIRED |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| RAN Paging Request | RAN PAGING REQUEST |
| Inventory Report | INVENTORY REPORT |

---------------Start of the Next Change---------------

### 8.xx.1	Inventory Request

#### 8.xx.1.1	General

The purpose of the Inventory Request procedure is to request the NG-RAN node to perform inventory over the A-IoT radio. The procedure uses non-UE associated signalling.

#### 8.xx.1.2	Successful Operation

Figure 8.xx.1.2-1: Inventory Request initiated by AIOTF directly, successful operation.

Figure 8.xx.1.2-2: Inventory Request initiated by AIOTF via AMF, successful operation.

The AIOTF initiates the procedure by sending an INVENTORY REQUEST message to the NG-RAN node. If the NG-RAN node accepts the request, the NG-RAN node responds with the INVENTORY RESPONSE message.

In case of Inventory Request initiated by AIOTF via AMF, the AMF shall include the AIOTF Identifier IE in the INVENTORY REQUEST message, and the NG-RAN node, shall include the same value of the AIOTF Identifier IE in the INVENTORY RESPONSE message.

When starting the requested inventory, the NG-RAN node shall, include the same value of the Correlation Identifier IE as received from the INVENTORY REQUEST message in the INVENTORY REPORT message.

#### 8.xx.1.3	Unsuccessful Operation

Figure 8.xx.1.3-1: Inventory Request initiated by AIOTF directly, unsuccessful operation.

Figure 8.xx.y1.3-2: Inventory Request initiated by AIOTF via AMF, unsuccessful operation.

If the NG-RAN node is not able to perform the inventory operation in any of the request area, it shall send the INVENTORY FAILURE message with an appropriate cause value, and include the same value of the Correlation Identifier IE as received from the INVENTORY REQUEST message in the INVENTORY REPORT message.

In case of Inventory Request initiated by AIOTF via AMF, the NG-RAN node, shall include the same value of the AIOTF Identifier IE as received from the INVENTORY REQUEST message in the INVENTORY FAILURE message.

#### 8.xx.1.4		Abnormal Conditions

Void.

### 8.xx.2	Inventory Report

#### 8.xx.2.1	General

The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide Inventory Report to the AIOTF directly, or via the AMF. The procedure uses non-UE associated signalling.

#### 8.xx.2.2	Successful Operation

Figure 8.xx.2.2-1: Inventory Report to the AIOTF directly

Figure 8.xx.2.2-1: Inventory Report to the AIOTF via AMF

The NG-RAN node initiates the procedure by sending an INVENTORY REPORT message for the Inventory service indicated by the Correlation Identifier IE.

In case of Inventory Report to the AIOTF via AMF, the NG-RAN node shall include in the INVENTORY REPORT message the same value of the AIOTF Identifier IE as received from the INVENTORY REQUEST message.

#### 8.xx.2.3	Abnormal Conditions

Void.

---------------Start of the Next Change---------------

### 9.2.x		A-IoT Messages

#### 9.2.x.1	INVENTORY REQUEST

This message is sent by the 5GC to trigger the NG-RAN node to perform inventory over the A-IoT radio.

Direction: AMF  NG-RAN node, or AIOTF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Correlation Identifier | M |  | 9.3.1.xx1 |  | YES | reject |
| AIOTF Identifier | O |  | 9.3.3.ee |  | YES | reject |
| Inventory Request Transfer | M |  | OCTET STRING | Containing the Inventory Request Transfer IE specified in subclause 9.3.x.1 | YES | reject |

#### 9.2.x.2	INVENTORY RESPONSE

This message is sent by the NG-RAN node to report the successful outcome of the request from the INVENTORY REQUEST message.

Direction: NG-RAN node  AMF, or NG-RAN node  AIOTF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Correlation Identifier | M |  | 9.3.1.xx1 |  | YES | reject |
| AIOTF Identifier | O |  | 9.3.3.ee |  | YES | reject |
| Inventory Response Transfer | O |  | OCTET STRING | Containing the Inventory Response Transfer IE specified in subclause 9.3.x.2 | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.3	INVENTORY FAILURE

This message is sent by the NG-RAN node to report the unsuccessful outcome of the request from the INVENTORY REQUEST message.

Direction: NG-RAN node  AMF, or NG-RAN node  AIOTF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Correlation Identifier | M |  | 9.3.1.xx1 |  | YES | reject |
| AIOTF Identifier | O |  | 9.3.3.ee |  | YES | reject |
| Inventory Failure Transfer | O |  | OCTET STRING | Containing the Inventory Failure Transfer IE specified in subclause 9.3.x.3 | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.4	INVENTORY REPORT

This message is sent by the NG-RAN node to report the outcome for an Inventory Session.

Direction: NG-RAN node  AMF, or NG-RAN node  AIOTF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Correlation Identifier | M |  | 9.3.1.xx1 |  | YES | reject |
| AIOTF Identifier | O |  | 9.3.3.ee |  | YES | reject |
| Inventory Report Transfer | O |  | OCTET STRING | Containing the Inventory Report Transfer IE specified in subclause 9.3.x.4 | YES | reject |

---------------Start of the Next Change---------------

### 9.3.1	Radio Network Layer Related IEs

#### 9.3.1.xx1	Correlation Identifier

This IE uniquely identifies an A-IoT service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation Identifier | M |  | OCTET STRING (SIZE(FFS)) | Encoded as defined in TS 23.003 [23]. |

#### 9.3.1.xx2	Inventory Target Information

This IE includes the target area for the A-IoT inventory.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Target Reader List |  | 0..1 |  |  |
| >Target Reader Item |  | 1..<maxnoofReaders> |  |  |
| >>Reader Identity | M |  | 9.3.3.zz |  |
| Target Inventory Area List |  | 0..1 |  |  |
| >Target Inventory Area Item |  | 1..<maxnoofInventoryAreas> |  |  |
| >>Inventory Area Identity | M |  | 9.3.3.dd |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers served by a gNB. Value is 256. |
| maxnoofInventoryAreas | Maximum no. of Inventory Areas. Value is 256. |

#### 9.3.1.xx3	Inventory Assistance Information

This IE includes the assistance information for the A-IoT Inventory Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Approximate Number of Target A-IoT Devices | O |  | (0..2047, …) |  |
| Estimate of Expected D2R Message Size | O |  | INTEGER (0..256, …) | Unit: bit. |

---------------Start of the Next Change---------------

#### 9.3.3.xx	Device Identification for Paging

This IE represents the Identity with which the A-IoT device(s) to be paged.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Device Identification For Paging | M |  |  |  |
| >Device Identity |  |  |  |  |
| >> Device Identity | M |  | Device Identification 9.3.3.yy |  |
| >Group Identity |  |  |  |  |
| >>Group Identity | M |  | Device Identification 9.3.3.yy |  |
| >All Devices |  |  | NULL |  |

#### 9.3.3.yy	Device Identification

This IE represents the A-IoT device identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Device Identity | M |  | OCTET STRING |  |

#### 9.3.3.zz	Reader Identity

This IE represents identifier of a Reader within a gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reader Identity | M |  | BIT STRING (SIZE(8)) |  |

#### 9.3.3.cc	Inventory Area Code

This IE uniquely identifies an Inventory Area Code.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inventory Area Code | M |  | OCTET STRING (SIZE(3)) |  |

#### 9.3.3.dd	Inventory Area ID

This IE uniquely identifies an Inventory Area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.3.5 |  |
| Inventory Area Code | M |  | 9.3.3.cc |  |

#### 9.3.3.ee	AIOTF Identifier

This IE uniquely identifies the AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIOTF Identitifer | M |  | FFS |  |

---------------Start of the Next Change---------------

### 9.3.x	AIOTF Related IEs

#### 9.3.x.1	Inventory Request Transfer

This IE is transparent to the AMF, if via AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Device Identification for Paging | M |  | 9.3.3.xx |  |
| Inventory Target information | O |  | 9.3.1.xx2 |  |
| Inventory Assistance Information | O |  | 9.3.1.xx3 |  |

#### 9.3.x.2	Inventory Response Transfer

This IE is transparent to the AMF, if via AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

#### 9.3.x.3	Inventory Failure Transfer

This IE is transparent to the AMF, if via AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

#### 9.3.x.4	Inventory Report Transfer

This IE is transparent to the AMF, if via AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Device ID to Report List |  | 1 |  |  |
| >Device ID to Report Item |  | 1..<maxnoofDeviceIDstoReport> |  |  |
| >>A-IoT NAS PDU | M |  | OCTET STRING |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDeviceIDstoReport | Maximum no. of Device IDs to report. Value is FFS. |

---------------End of the Change---------------