3GPP TSG-RAN WG3 #127	R3-250273

Athens, GR, 17th – 21st Feb 2025

Agenda Item:	11.2

Source:	Lenovo

Title: 		Way forward of per slice UE performance measurement

Document for:	Discussion and Approval

# Introduction

In this paper, we try to analyse and propose way forward of the per slice UE performance metric discussion.

# Discussion

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node. Turn WA to agreement: UE performance per Slice is reported from target node to source node. Companies have different understandings on if per slice per UE performance (e.g., throughput) is really supported by SA5 specification. Source node can request per UE per S-NSSAI granularity for UE performance feedback. There is no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node.

In the last RAN3 meeting, it was agreed that source node can request per UE per S-NSSAI granularity for UE performance feedback, including UL/DL UE throughput, packet delay, and DL packet loss. However, it seems very controversial how the per UE per S-NSSAI is measured and reported from target node to source node according to the offline discussion last time, w.r.t the following two aspects:

- Is per UE per S-NSSAI UE performance already defined in the specification?

  - Some companies believe SA5 spec already specified

  - Some companies understand differently.

Note: this confusion may be caused by the measurement name defined in 28.558, e.g., DRB.UEThpDl.SNSSAI

- In case per UE per S-NSSAI UE performance is not defined or is not good enough, is there a motivation to introduce any enhancement?

  - Some companies propose per QoS flow (group) level UE performance

  - Some companies don’t think any enhancement is needed.

R3-247785 SoD of CB AI based Slicing - Aspect#1: How UE performance per slice granularity can be discussed/decided by RAN3, but has some dependency on how UE performance per UE granularity is calculated which was discussed in Rel-18 correction and an LS to SA5 is triggered, e.g., - Or if it is an average over time among all packets via all DRBs within the same slice/UE, e.g., R3-247412 AvgTputDLUE-SA5=DV1+DV2∆T1+∆T2=5+10Mbit500+200ms≅21.43Mbps - Or if it is an average of DRB level metric within the same slice/UE, e.g., R3-247412 AvgTputDLUE-RAN3=TputDRB#1+TputDRB#22=10+502Mbps=30Mbps E///, Huawei, ZTE: SA5 has defined and supported the per slice per UE performance. RAN3 does not need to further discuss. Nokia, CATT: it is unclear from SA5 spec, how per slice per UE performance is calculated. What’s supported in SA5 spec now is per slice, but not per slice per UE. - Aspect#2: Issue raised by some companies is that: transferring UE performance per slice level may cause problem for the source gNB to correctly understand if UE performance is improved/degraded after handover, no matter how it is calculated according to Aspect#1. Same issue exists in Rel18 with UE level performance transfer. - Proponent companies propose to support per QoS flow (group) level UE performance transfer (measured per DRB eventually) to resolve the issue. - Opponent companies either don’t believe this is a big issue, or don’t believe per QoS flow (group) level UE performance measurement can resolve the issue neither.

RAN3 is suggested to firstly clarify if per UE per S-NSSAI throughput/delay/packet loss measurement is already specified e.g., in TS 28.558. Consult SA5 if needed.

If per UE per S-NSSAI throughput/delay/packet loss measurement is already specified, and if no enhancement is needed, no further discussion is needed in RAN3.

If per UE per S-NSSAI throughput/delay/packet loss measurement is not specified or not good enough, RAN3 can further discuss how to measure and report the per UE per S-NSSAI performance.

# Conclusion

Based on the discussion above, we observe:

No table of figures entries found.

Based on the discussion above, we propose:

Proposal 1	RAN3 is suggested to firstly clarify if per UE per S-NSSAI throughput/delay/packet loss measurement is already specified e.g., in TS 28.558. Consult SA5 if needed.

Proposal 2	If per UE per S-NSSAI throughput/delay/packet loss measurement is already specified, and if no enhancement is needed, no further discussion is needed in RAN3.

Proposal 3	If per UE per S-NSSAI throughput/delay/packet loss measurement is not specified or not good enough, RAN3 can further discuss how to measure and report the per UE per S-NSSAI performance.