3GPP TSG-RAN WG3 #127                                                 R3-250328

Athens, GR, 17th – 21st Feb, 2025

Agenda Item:	10.4

Source:	CATT

Title:	(TP for 37.340, 38.423, 36.423 and 38.413) SON enhancement for R18 leftover

Document for:	other

Introduction

According to the agreements achieved in last meeting, here we provide some analysis on MRO enhancement for MR-DC SCG failure and MHI/UHI for SCG deactivation/activation.

Discussion

  - MRO for MR-DC SCG failure

In previous RAN2 meeting, SCGFailureInformationNR message is enhanced as below:

Agreements - Add reporting of the following parameters for SCG failure report in EN-DC scenario: ⁻ For failedPSCellId and previousPSCellId: frequency and the PCI of the PSCell; ⁻ For timeSCGFailure: value range 0-1023; ⁻ For failureType: Reuse the legacy field. - perRA-InfoList

In last RAN3 meeting, timeSCGFailure IE has been included in SCG FAILURE INFORMATION REPORT message. For other new IEs included in SCGFailureInformationNR message, RAN3 needs to discuss whether it is needed to provide them from MN to SN.

For failure type, RAN2 decide to reuse legacy field which is defined in SCGFailureInformation message as below in TS38.331.

failureType                                    ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-ReconfigFailure,

srb3-IntegrityFailure, other-r16, spare1},

……

failureType-v1610                        ENUMERATED {scg-lbtFailure-r16, beamFailureRecoveryFailure-r16,

t312-Expiry-r16, bh-RLF-r16, beamFailure-r17, spare3, spare2, spare1} OPTIONAL

Obviously some failure type is not related to PSCell addition/change. For example, SCG failure due to srb3-IntegrityFailure has no relationship with mobility and should not trigger mobility optimization on SCG configuration. For other failure types, for example synchReconfigFailureSCG which have relationship with mobility, this kind of SCG failure should trigger mobility optimization on SCG configuration. So, failure type is needed for SN to perform MRO analysis.

Proposal 1: It is proposed to transfer failureType which is included in SCGFailureInformationNR message from MN to SN.

For perRA-InfoList, it could be used to detect RACH problem in SN, so it is also needed in target SN.

Proposal 2: It is proposed to transfer perRA-InfoList which is included in SCGFailureInformationNR message from MN to SN.

Here we continue discussing the forwarding mechanism in EN-DC.

First, MN could detect whether intra-SN PSCell change has been triggered in the last serving SN from SCGFailureInformationNR message. If previousPSCellId is one of the cells in source SN, there is no intra-SN PSCell occurs. On the contrary, if previousPSCellId is one of the cells in target SN, there is intra-SN PSCell occurs. So, the procedure on MN checking intra-SN PSCell change with SN is not necessary.

Proposal 3: It is proposed to not introduce the procedure on MN checking intra-SN PSCell change with SN.

For MN initiate PSCell change, MN selects PSCell. And MN generates execution condition, so, MN needs to optimize execution condition.

Observation 1: In case of MN initiate PScell change, MN may need to optimize PSCell selection for PSCell change and execution condition.

For SN initiate PScell change, source SN selects target SN and then target SN selects PSCell. So, for PSCell change to wrong PSCell failure type, both source SN and target SN may optimize target PCell selection. And source SN generates execution condition, so, source SN needs to optimize execution condition.

Observation 2: In case of SN initiate PScell change, source SN may need to optimize PSCell selection for PSCell change and execution condition.

For EN-DC and NGEN-DC case, the SCGFailureInformationNR message is defined in TS36.331 as below:

SCGFailureInformationNR message

-- ASN1START

SCGFailureInformationNR-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

scgFailureInformationNR-r15			SCGFailureInformationNR-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCGFailureInformationNR-r15-IEs ::=	SEQUENCE {

failureReportSCG-NR-r15				FailureReportSCG-NR-r15				OPTIONAL,

nonCriticalExtension					SCGFailureInformationNR-v1590-IEs	OPTIONAL

}

SCGFailureInformationNR-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension					OCTET STRING					OPTIONAL,

nonCriticalExtension						SEQUENCE {}					OPTIONAL

}

FailureReportSCG-NR-r15 ::=		SEQUENCE {

failureType-r15						ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-reconfigFailure,

srb3-IntegrityFailure, dummy},

measResultFreqListNR-r15				MeasResultFreqListFailNR-r15		OPTIONAL,

measResultSCG-r15						OCTET STRING						OPTIONAL,

...,

[[	locationInfo-r16				LocationInfo-r10						OPTIONAL,

logMeasResultListBT-r16			LogMeasResultListBT-r15					OPTIONAL,

logMeasResultListWLAN-r16		LogMeasResultListWLAN-r15				OPTIONAL,

failureType-v1610				ENUMERATED {t312-Expiry, scg-lbtFailure,

beamFailureRecoveryFailure, bh-RLF-r16,

beamFailure-r17,

spare3, spare2, spare1}	OPTIONAL

]]

}

measResultSCG Includes the NR MeasResultSCG-Failure IE as specified in TS 38.331 [82]. The field contains available results of measurements on NR frequencies the UE is configured to measure by the NR RRCConfiguration message.

The field measResultSCG is encoded in NR format hence LTE MN cannot decode it. Although measResultFreqListNR-r15 can be decoded by MN, it is so limited because it includes only SSB measurement result which may be better to be used for SN node selection. The measResultSCG which is configured by SN with more precise measurement information is used to select/optimize target PSCell. Note that, for legacy NR-DC PScell change, both MN and SN can decode measResultFreqList and measResultSCG-Failure, they use them for suitable PScell analyses.

Observation 3: For EN-DC and NGEN-DC case, MN cannot decode measResultSCG in SCGFailureInformationNR message but it shall be used for the next suitable PSCell selection.

Therefore, we propose NR SN takes the responsibility to select the next suitable PSCell based on measResultSCG in SCGFailureInformationNR message.

Proposal 4: NR SN selects the next suitable PSCell based on the measResultSCG in SCGFailureInformationNR message for both MN initiate PSCell change and SN initiate PSCell change.

Proposal 5: It is proposed to include the next suitable PSCell in SCG FAILURE TRANSFER message.

There is another issue on whether NR SN should provide PSCell change failure type to MN in case of MN initiated PScell change. If not providing PSCell change failure type to MN, MN should detect PSCell change failure type based on MN MRO configuration, for example, Tstore_UE_cntxt and threshold for detecting coverage hole. Manual work is needed to keep MRO configuration aligned between MN and SN. Otherwise, MN and SN may detect different PSCell failure type. For example, for different Tstore_UE_cntxt configured in MN and SN, SN may detect too late PSCell change failure type (SCG RLF occurs not shortly after successful PSCell change) while MN may detect to wrong cell PSCell change failure type (SCG RLF occurs shortly after successful PSCell change).

So, we slightly prefer NR SN providing PSCell change failure type from SN to MN.

Proposal 6: It is proposed for RAN3 to discuss whether including PSCell change failure type in SCG FAILURE TRANSFER message.

The MN performs the initial analysis when SCGFailureInformationNR is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case there is MN initialed PSCell change, MN forwards the SCG Failure Information Report message to the last serving SN which will response the next suitable PSCell to MN via SCG Failure Transfer. In case there is SN initialed inter-SN PSCell change, MN forwards the SCG Failure Information to the source SN. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

Proposal 7: It is proposed for RAN3 to discuss the above forwarding mechanism which is capture in TP for 37.340.

  - MHI/UHI enhancement for SCG deactivation/activation

RAN3 agree to support a network-based solution for enhancing PSCell activation/deactivation.

For network based solution, the Time in SCG activated state can be added in Last Visited PSCell Information. The time over 4096s is longer enough which means the DC is useful no matter the percentage it occupied in whole DC time.

Observation 4: SCG active time over 4096s is long enough to prove DC is needed, no matter what percentage of the whole DC time.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. | - |  |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | - |  |
| Time in SCG activated state | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in activated state in the cell in 1/10 seconds. If the UE stays in activated state in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | YES | ignore |

Companies also propose to add throughput in UHI. The throughput is strongly related to the service which may be very high in some time while very low in other time. Throughput cannot be treated as a criterion for configuring DC for UE. Therefore, as long as the activation time is long, even if the throughput on the SCG is low, it still can show that the SCG is useful.

Observation 5: Throughput is dynamic depend on different service. As long as the activation time is long, it can show that the SCG/DC is useful even the throughput on the SCG is low at some time.

Proposal 8: Add Time in SCG activated state IE in Last Visited PSCell Information.

Conclusions

MRO for MR-DC SCG failure:

Proposal 1: It is proposed to transfer failureType which is included in SCGFailureInformationNR message from MN to SN.

Proposal 2: It is proposed to transfer perRA-InfoList which is included in SCGFailureInformationNR message from MN to SN.

Proposal 3: It is proposed to not introduce the procedure on MN checking intra-SN PSCell change with SN.

Observation 1: In case of MN initiate PScell change, both MN and target SN may need to optimize PSCell selection for PSCell change. And MN may need to optimize execution condition.

Observation 2: In case of SN initiate PScell change, both source SN and target SN may need to optimize PSCell selection for PSCell change. And source SN needs to optimize execution condition.

Observation 3: For EN-DC and NGEN-DC case, MN cannot decode measResultSCG in SCGFailureInformationNR message but it shall be used for the next suitable PSCell selection.

Proposal 4: NR SN selects the next suitable PSCell based on the measResultSCG in SCGFailureInformationNR message for both MN initiate PSCell change and SN initiate PSCell change.

Proposal 5: It is proposed to include the next suitable PSCell in SCG FAILURE TRANSFER message.

Proposal 6: It is proposed for RAN3 to discuss whether including PSCell change failure type in SCG FAILURE TRANSFER message.

Proposal 7: It is proposed for RAN3 to discuss the above forwarding mechanism which is capture in TP for 37.340.

MHI/UHI enhancement for SCG deactivation/activation:

Observation 4: SCG active time over 4096s is long enough to prove that DC is needed, no matter which percentage it occupied in whole DC time.

Observation 5: Throughput is dynamic depend on different service. As long as the activation time is long, it can show that the SCG/DC is useful even the throughput on the SCG is low at some time.

Proposal 8: Add Time in SCG activated state IE in Last Visited PSCell Information.

TP for TS 37.340

- 10.18.2	PSCell change failure

One of the functions of self-optimization for PSCell change is to detect PSCell change failures that occur due to Too late PSCell change or Too early PSCell change, or Triggering PSCell change to wrong PSCell. These problems are defined as follows:

-	Too late PSCell change: an SCG failure occurs after the UE has stayed for a long period of time in the PSCell; a suitable different PSCell is found based on the measurements reported from the UE.

-	Too early PSCell change: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; source PSCell is still the suitable PSCell based on the measurements reported from the UE.

-	Triggering PSCell change to wrong PSCell: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; a suitable PSCell different with source PSCell or target PSCell is found based on the measurements reported from the UE.

In the definition above, the "successful PSCell change" refers to the UE state, namely the successful completion of the RA procedure.

For NR-DC, MN performs initial analysis to identify the node that caused the failure. The MN may use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN and stores the SCG Failure Information for the time needed to receive possible response from the last serving SN. If the failure is caused by a source SN, the MN forwards then the SCG Failure Information to the source SN. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

For EN-DC and NGEN-DC, the MN performs the initial analysis when SCGFailureInformationNR is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case there is MN initialed PSCell change, MN forwards the SCG Failure Information Report message to the last serving SN which will response the next suitable PSCell to MN via SCG Failure Transfer. In case there is SN initialed inter-SN PSCell change, MN forwards the SCG Failure Information to the source SN. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

TP for TS 38.423

- 8.3.17	SCG Failure Information Report

- 8.3.17.1	General

The purpose of the SCG Failure Information Report procedure is to provide SCG mobility related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

- 8.3.17.2	Successful Operation

Figure 8.3.17.2-1: SCG Failure Information Report, successful operation

The M-NG-RAN node initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to the S-NG-RAN node. Upon receiving the message, the S-NG-RAN node shall assume that a PSCell change failure event was detected.

The SCG FAILURE INFORMATION REPORT message may include:

-	the SN Mobility Information IE, if the SN Mobility Information IE was sent for the PSCell change procedure from the S-NG-RAN node;

-	the Source PSCell CGI IE, if the Source PSCell CGI IE was sent for the PSCell change procedure from the S-NG-RAN node.

If the SCG FAILURE INFORMATION REPORT message includes the Source PSCell CGI IE, the S-NG-RAN node shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the Failed PSCell CGI IE, the S-NG-RAN node shall, if supported, store the information and act as specified in TS 38.300 [9].

If received, the S-NG-RAN node uses the above information for SCG failure reason detection and optimisation.

If the SCG FAILURE INFORMATION REPORT message includes the Time SCG Failure IE, the S-NG-RAN node shall, if supported, store the information and act as specified in TS 38.300 [9].

If the SCG FAILURE INFORMATION REPORT message includes the Failure Type IE, the S-NG-RAN node shall, if supported, store the information and act as specified in TS 38.300 [9].

If the SCG FAILURE INFORMATION REPORT message includes the RA-InfoList IE, the S-NG-RAN node shall, if supported, store the information and act as specified in TS 38.300 [9].

- 8.3.17.3	Unsuccessful Operation

Not applicable.

- 8.3.17.4	Abnormal Conditions

Void.

- 8.3.18	SCG Failure Transfer

- 8.3.18.1	General

The purpose of the SCG Failure Transfer procedure is to indicate to the M-NG-RAN node that the root cause of the SCG failure may have occurred in the other nodes.

The procedure uses UE-associated signalling.

- 8.3.18.2	Successful Operation

Figure 8.3.18.2-1: SCG Failure Information Transfer, successful operation

S-NG-RAN node initiates the procedure by sending the SCG FAILURE TRANSFER message to M-NG-RAN node.

If the SCG FAILURE TRANSFER message includes the Next suitable PSCell IE, the M-NG-RAN node shall, if supported, store the information and act as specified in TS 38.300 [9].

If received, M-NG-RAN node uses the information according to TS 38.300 [9].

- 8.3.18.3	Unsuccessful Operation

Not applicable.

- 8.3.18.4	Abnormal Conditions

Void.

If received, the S-NG-RAN node uses the above information for SCG failure reason detection and optimisation.

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>

#### 9.1.2.29	SCG FAILURE INFORMATION REPORT

This message is sent by M-NG-RAN node to S-NG-RAN node to report a PSCell change failure event.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Source PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformation message or the SCGFailureInformationEUTRA message as defined in TS 38.331 [10] or the SCGFailureInformation message or the SCGFailureInformationNR message as defined in TS 36.331 [14] | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the PSCell change. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| CPAC Configuration | O |  | 9.2.2.103 |  | YES | ignore |
| Time SCG Failure | O |  | INTEGER (0..1023) | timeSCGFailure included in SCGFailureInformationNR message as defined in TS36.331 [9] | YES | ignore |
| Failure Type | O |  | FFS | Corresponds to information provided in the failureType contained in the SCGFailureInformationNR message as defined in TS 36.331 [9]. | YES | ignore |
| RA-InfoList | O |  | FFS | Corresponds to information provided in the perRAInfoList-r17 contained in the SCGFailureInformationNR message as defined in TS 36.331 [9]. | YES | ignore |

- 9.1.2.30	SCG FAILURE TRANSFER

This message is sent by the S-NG-RAN node to the M-NG-RAN node to indicate that the root cause of the SCG failure may have occurred in the other nodes.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Next suitable PSCell | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell which selected as the next suitable PSCell. | YES | ignore |

TP for TS 36.423

### 8.7.x	SCG Failure Information Report

#### 8.7.x.1	General

The purpose of the SCG Failure Information Report procedure is to provide SCG mobility related information to an en-gNB.

The procedure uses UE-associated signalling.

#### 8.7.x.2	Successful Operation

Figure 8.7.x.2-1: SCG Failure Information Report, successful operation

The MeNB initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to the en-gNB. Upon receiving the message, the en-gNB shall assume that a PSCell change failure event was detected.

The SCG FAILURE INFORMATION REPORT message may include:

-	the Source PSCell CGI IE, if the Source PSCell CGI IE was sent for the PSCell change procedure from the en-gNB.

If the SCG FAILURE INFORMATION REPORT message includes the Source PSCell CGI IE, the en-gNB shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the Failed PSCell CGI IE, the en-gNB shall, if supported, store the information and act as specified in TS 36.300 [15].

If the SCG FAILURE INFORMATION REPORT message includes the Time SCG Failure IE, the en-gNB shall, if supported, store the information and act as specified in TS 36.300 [15].

If the SCG FAILURE INFORMATION REPORT message includes the Failure Type IE, the en-gNB shall, if supported, store the information and act as specified in TS 36.300 [15].

If the SCG FAILURE INFORMATION REPORT message includes the RA-InfoList IE, the en-gNB shall, if supported, store the information and act as specified in TS 36.300 [15].

If received, the en-gNB uses the above information for SCG failure reason detection and optimisation.

#### 8.7.x.3	Unsuccessful Operation

Not applicable.

#### 8.7.x.4	Abnormal Conditions

Void.

### 8.7.y	SCG Failure Transfer

#### 8.7.y.1	General

The purpose of the SCG Failure Transfer procedure is to indicate to the MeNB that the root cause of the SCG failure may have occurred in the other nodes.

The procedure uses UE-associated signalling.

#### 8.7.y.2	Successful Operation

Figure 8.7.y.2-1: SCG Failure Transfer, successful operation

en-gNB initiates the procedure by sending the SCG FAILURE TRANSFER message to MeNB.

If the SCG FAILURE TRANSFER message includes the Next suitable PSCell IE, the MeNB node shall, if supported, store the information and act as specified in TS 36.300 [15].

If received, MeNB uses the information according to TS 36.300 [15].

#### 8.7.y.3	Unsuccessful Operation

Not applicable.

#### 8.7.y.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.4.x	SCG FAILURE INFORMATION REPORT

This message is sent by the MeNB to the en-gNB to report a PSCell change failure event.

Direction: MeNB  the en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE XnAP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Source PSCell CGI | O |  | NR CGI 9.2.111 | NR CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | NR CGI 9.2.111 | NR CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformationNR message as defined in TS 36.331 [9] | YES | ignore |
| Time SCG Failure | O |  | INTEGER (0..1023) | timeSCGFailure included in SCGFailureInformationNR message as defined in TS36.331 [9] | YES | ignore |
| Failure Type | O |  | FFS | Corresponds to information provided in the failureType contained in the SCGFailureInformationNR message as defined in TS 36.331 [9]. | YES | ignore |
| RA-InfoList | O |  | FFS | Corresponds to information provided in the perRAInfoList-r17 contained in the SCGFailureInformationNR message as defined in TS 36.331 [9]. | YES | ignore |

#### 9.1.4.y	SCG FAILURE TRANSFER

This message is sent by the en-gNB to the MeNB to indicate that the root cause of the SCG failure may have occurred in the other nodes.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE XnAP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Next suitable PSCell | O |  | 9.2.111 NR CGI | NR CGI of PSCell which selected as the next suitable PSCell. | YES | ignore |

TP for TS 38.413

- 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. | - |  |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | - |  |
| Time in SCG activated state | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in activated state in the cell in 1/10 seconds. If the UE stays in activated state in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | YES | ignore |

- 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

Unchanged part is omitted

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-TimeInSCG-ActivatedState,

maxnoofAllowedAreas,

Unchanged part is omitted

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID			NGRAN-CGI	 			OPTIONAL,

timeStay			INTEGER (0..40950),

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-TimeInSCG-ActivatedState					CRITICALITY ignore	EXTENSION TimeInSCG-ActivatedState		PRESENCE optional},

...

}

Unchanged part is omitted

-- T

TimeInSCG-ActivatedState ::= INTEGER (0..40950)

- 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

Unchanged part is omitted

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-TimeInSCG-ActivatedState								ProtocolIE-ID ::= xxx