3GPP TSG-RAN WG3#127	R3-250463

Athens, GR, 17-21 February 2025

Agenda Item:	11.4

Title:	Further discussions on Continuous MDT

Source:	Ericsson, InterDigital, Deutsche Telekom, FiberCop

Document for:	Discussion

# Introduction

During Rel-18, a number of agreements showing that Continuous MDT is a valuable and beneficial enhancement for the collection of data supporting AI/ML were taken and it has been agreed to include Continuous MDT collection targeting the same UE across RRC states within the objectives of the Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN WID for Rel-19 [1].

This paper continues the discussion on Continuous MDT and proposes solutions that are based on existing fields within the Trace Records in line with recent agreements in RAN3.

# Discussion

Discussions in RAN3 concerning Continuous MDT have relied on the fact that measurement reports for management based MDT could be complemented with a semi permanent identifier serving the purpose of correlating MDT measurement reports and letting the TCE understand that the reports were generated for the same data source (the same UE).

Observation 1: Continuous MDT discussions have so far been based on the possibility to introduce a semi-permanent identifier to the Trace Records containing management based MDT measurements, so to allow the TCE to determine that the measurements are collected by the same source

Up until recently, such semi-permanent identifier was believed to be the S-TMSI reported in the per UE measurements specified in TS28.558. However, SA5 confirmed that such identifier is not used for MDT measurements collected from the RAN.

To ensure the applicability of Continuous MDT solutions to legacy UEs, the following agreement was taken at RAN3-126:

In R19, introducing the Continuous MDT functionality with a solution being applicable to legacy UEs and not only to Rel-19 UEs.

And the following FFS was captured:

Compare the solutions based on this principle.

To provide a complete list of agreements, the following should be recalled from the agreements made at RAN3-125bis:

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

During RAN3-126 the group concluded that, while it would be beneficial for RAN3 to discuss solutions to enable continuous MDT, it is essential to involve in this discussion SA5, which is ultimately the group that owns Stage 2 description and OAM-related Stage 3 description for the MDT functionalities.

Because of this, the following was agreed:

Agree to send an LS to SA5 capturing e.g. problem description, solutions requirements, potential solutions (if agreeable). When to send the LS depends on convergence on the content of the LS.

Where an open point remains on the following:

Identification of requirements and description of problem statement, followed by LS drafting to be continued.

In order to reach identification of requirements, problem statement description and an understanding of the solutions that could be followed for Continuous MDT, we dedicate the following sections to a technical discussion on how to specify the crucial enablers for Continuous MDT.

  - Continuous MDT problems description and possible solution

The two following problems that need to be resolved for Continuous MDT are:

-	Problem A (measurement continuity): how to ensure that the same UE is selected for collection of MDT measurements during sessions in the same RRC state, across different RRC states and across RRC_Connected mobility.

-	Problem B (trace correlation): how to ensure that the TCE receiving the Trace Record files containing MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection process from the same data source (UE).

The agreement2 quoted in Section 2 imply that solutions for continuous MDT should:

- Reuse the Management Based MDT framework

- Rely on the OAM to configure the RAN with a Management Based MDT configuration to be used for Continuous MDT

- Be network based and avoid impacts on legacy UEs.

In the sections below we address the two main problems to be solved for Continuous MDT.

### Measurement continuity configuration

The main task here is to enable means to consistently select the UE across RRC state changes and across mobility between gNBs.

As per current agreements, the OAM is in charge of configuring management based MDT for continuous MDT. Such configuration is new and needs to be introduced by SA5. However, a straightforward way to achieve this is to rely on existing parameters and operations, such as the existing Trace References (TR) identifier. 
Indeed, the OAM could select a TR in a specific range or it could use a specific pre-defined Trace Reference prefix for the Trace Job corresponding to Continuous MDT. If continuous MDT is needed, a TR with one of the pre-defined values or prefix is sent from the OAM to the RAN as part of the TraceJob. In such a case, the receiving end of the TraceJob, the gNB, will understand from the TR prefix or from the TR value that it is a continuous MDT configuration.

With such a choice one would avoid specifying new trace jobs and the continuous MDT process would rely on existing MDT operation.

An example of applying such high level principle to the problem at hand is presented below, showing how such a simple way to reusing the existing identifiers in the Trace Record can achieve the purpose of UE selection continuity during RRC state changes:

  - The NG-RAN receives from the OAM a Management Based MDT configuration, including a specific Trace Reference ID. From such trace Reference ID the RAN deduces that the Management Based MDT configuration is for Continuous MDT.

  - The RAN selects a UE in RRC _Connected for the Continuous MDT measurement process. Continuous MDT measurements are collected while the Ue is in RRC_Connected.

  - While the UE is in RRC_CONNECTED, the NG-RAN would signal to the selected UE the continuous logged MDT configuration. The serving gNB would include in the logged MDT configuration the TR that indicates that the trace is for Continuous MDT, assuming that SA5 specified a range of TRs (e.g. via the allocated prefix) that is specific for Continuous MDT.

  - When the UE goes to RRC_IDLE/RRC_INACTIVE, it measures and logs measurements. These logs include the TR assigned by the OAM and the Trace Recording Session Reference (TRSR) assigned by the gNB.

  - When the UE goes to RRC_CONNECTED and it reports Logged MDT logs, such logs would include the Continuous MDT TR assigned by the OAM and the TRSR assigned by the gNB.

  - The receiving gNB understands that the UE has to be configured for Continuous MDT by checking the TR in the Logged MDT logs, which is specific for Continuous MDT.

  - For cases of UE mobility during RRC_CONNECTED and while a Continuous MDT process is running for the UE

    - Either the source gNB signals to the Target gNB a Continuous MDT TR, as part of the Handover Signalling, or the source gNB signals to the target gNB, as part of the Handover Signalling, an explicit “Continuous MDT flag”. In both cases, the target gNB understands that the UE has been configured with Continuous MDT at source gNB and it can select it again.

Proposal 1: RAN3 to agree to the following Continuous MDT problem statement:

Measurement continuity problem: how to ensure that the same UE is selected for collection of MDT measurements during sessions in the same RRC state, across different RRC states and across RRC_Connected mobility.

Trace Correlation Problem: how to ensure that the TCE receiving the Trace Record files containing MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection process from the same data source (UE).

Proposal 2: RAN3 to agree to the following principle to solve the measurement continuity problem, and to check its feasibility with SA5:

- The OAM provides to the RAN means to identify that the Management Based MDT process is for Continuous MDT, preferably reusing existing Trace Record information

Proposal 3: RAN3 to agree that Source gNB signals to Target gNB, as part of the Handover Signalling, an indication that the UE has been selected for Continuous MDT. Target gNB uses this information to re-select the UE for Continuous MDT.

It should be noted here that, as part of the continuous MDT configuration from OAM, the duration of the continuous MDT collection could also be included in the set of the provided Continuous MDT configuration parameters, otherwise it would not be possible for the Ng-RAN to understand when the Continuous MDT process should be terminated.

Proposal 4: RAN3 to agree that a parameter associated with the duration of the continuous MDT collection is to be provided by OAM and check feasibility with SA5.

### Correlation of Continuous MDT measurements

During the Rel-18 discussions it was acknowledged that Management Based MDT does not offer the possibility to merge Immediate and Logged MDT data collected by the same UE across different RRC states and across different serving gNBs. In order to do this an identifier would be needed that is consistently maintained across Management Based MDT data collection in different RRC states and across different gNBs, and that is associated to the collected MDT data.

The need of such identifier as a requirement for the realization of Continuous MDT has been raised to SA5 [2] and in its response SA5 is acknowledging the benefits of such requirement [3]. In particular:

“SA5 agree that there is a benefit to have a (temporary) identifier in the management-based MDT measurement report. This identity shall be unique within the lifetime of the measurement session. So, some data analytics function (e.g., AIML related function) can have a better understanding that the collected data sets are measured from the same source. Therefore, it would be good to provide some kind of identifier to indicate the measurement was done in the same or different UE.”

It is important to understand that a Continuous MDT correlation identifier would enable to achieve continuity of MDT measurements and resolve trace correlation.

The question at hand is how to select a proper Correlation ID that would be tagged to the Trace Records associated to the same Continuous MDT process and to the same UE.

To address this, it could be helpful to check the existing information carried out by the Trace Record Header. According to section 5.2.2 of TS32.423

### 5.2.2 Trace Record Header The trace record header contains the common fields as specified in the Table 5.2.2-1, in addition it may also contain vendor specific extensions. Table 5.2.2.1 : Common fields in the trace record header Trace Record Header field name; Description. timeStamp (M); Time stamp (in milliseconds since Epoch) of when the streaming trace record is produced internally in the Producer encoded as (64 bit integer). nfInstanceId (M); Unique id of the Producer NF instance that produced this trace record represented by a (String). nfType (M); Type of the Producer NF that produced this trace record represented by a (String). traceReference (M); Trace Reference (see clause 5.6 of 3GPP TS 32.422 [23]) (represented by a 3 bytes octet string). traceRecordingSessionReference (M); Trace Recording Session Reference (see clause 5.7 of 3GPP TS 32.422 [23]) represented by a (2 byte octet string. See Note 1.). traceRecordTypeId (M); Identifier of the trace record type (see clause 5.2.4 for details) represented by an ENUM with the following values: NORMAL TRACE_SESSION_START, TRACE_SESSION_STOP, TRACE_RECORDING_SESSION_START, TRACE_RECORDING_SESSION_STOP, TRACE_STREAM_HEARTBEAT. TRACE_RECORDING_SESSION_NOT_STARTED, TRACE_RECORDING_SESSION_DROPPED_EVENTS, TRACE_FILE_OPEN, TRACE_FILE_CLOSE, TRACE_FILE_ABNORMAL_CLOSED TRACE_RECORDING_SESSION_THROTTLED_START TRACE_RECORDING_SESSION_THROTTLED_STOP (See Note 2).. ranUeId (O); RAN defined id to represent a UE (8 byte octet string. See Note 3.). payloadSchemaURI (O); URI identifying the schema to be used in order to decode the payload represented by a (String. See Note 4.). globalGnbId (CM); Global gNB ID, as defined in [23]. Applied for trace reported by gNB-CU-CP, gNB-CU-UP, gNB-DU.. vendorExtension (O); Vendor-specific extension(s) represented by a (Arraylist of String. See Note 5.). - NOTE 1: The traceRecordingSessionReference must be present for the Streaming Trace Records with non-zero size payload where the payload carries data captured for a Trace Recording Session and in administrative messages related to a Trace Recording Session (e.g. "Trace Recording Session Start" or "Trace Recording Session Stop"). - NOTE 2: The traceRecordTypeId with value "NORMAL" is used for Streaming Trace Records that do not carry an administrative message. - NOTE 3: The ranUeId field is present in the trace record header when the identifier is supported by RAN. If RAN UE Id (see 3GPP TS 38.463 [25] and 38.473 [26]) has been captured in the traced signaling messages that value is used. - NOTE 4: The payloadSchemaURI is not required for Streaming Trace Records with payload of zero-size, or payload using common payload format (e.g. used to convey Streaming Trace administrative messages). - NOTE 5: The vendorExtension is typically a generic list of key-value pairs..

Conclusion 1: Some information, like the TR/TRSR, are always present in the trace records and could be exploited as correlation identifiers without the need to modify the Trace Recording header. Other information, like the RAN UE ID, are optionally present in the Trace Record and could also be considered as potential candidate for a correlation identifier.

It should also be noted here that, on top of the above information, and for Management Based Immediate MDT, the gNB shall include the CGI of the cell where Immediate MDT measurements were collected as part of the Trace Record. This is specified in TS32.422 section 4.1.1.9.2, as reported below:

For Immediate MDT when the gNB receives the MDT report from the UE in the RRC message the gNB shall capture it and put the UE’s serving cell CGI together with the MDT report from the UE to the trace record.

By the presence in the Trace Recording header of the TR, TRSR, CGI and time stamp, it could be argued that TCE can deduce that the Trace Records belong to the same UE by using a combination of such parameters. Said that, using the combination of many parameters to uniquely identify Trace Records belonging to the same source of data may result in a quite complex process. Nevertheless, RAN3 should ask SA5 for feedback about such solutions.

It is worth noting that deriving a correlation identifier from the parameters contained in the Trace Record has a number of advantages, among which the following:

- Trace Record parameters are already included in Trace Records by the gNB today. Reusing Trace Records parameters minimizes the impact on the current MDT design

- Reusing Trace Record parameters ensures avoiding impacts on the UE

- Reusing Trace Record parameters maintains the MDT functionality in the RAN and in the OAM. This avoids impacts on other systems. Making MDT depend on support from new systems/nodes creates further dependency and it generates higher impact

The above leads us to the following principle:

High level principle: for correlation of MDT reports, RAN3 to provide guidance to SA5 on what identifier could be used to correlate Continuous MDT measurement reports. Identifiers based on parameters contained in the Trace Record should be prioritised.

Proposal 5: RAN3 to agree to the following principle and to check its feasibility with SA5:

- For correlation of MDT reports, RAN3 to provide guidance to SA5 stating that identifiers based on parameters contained in the Trace Recording header should be prioritised as they minimise the impact on current functionalities and implementations.

One possible guidance RAN3 could send to SA5 concerning how to derive correlation identifiers could be based on reusing the TR, TRSR, RAN UE ID, CGI and time stamp contained in the Trace Record to enable the TCE to identify that the MDT data received come from the same data source.

Proposal 6: Send an LS to SA5 to check the viability of re-using TR, TRSR, time stamp, RAN UE ID and CGI combinations as candidates for continuous MDT correlation identifiers.

## AMF based solutions

During past RAN3 meetings, solutions were proposed where the Temporary Identifier for Continuous MDT report correlation is assigned and stored by the AMF. We believe that such solutions shall be avoided due increased signalling complexity, dependency and impact on the AMF, and shifting of MDT functionalities from the OAM/RAN to the CN. In particular. Consequences of such solutions could be as follows:

- There would be an impact on the AMF, and a dependency of the whole feature on AMF support: no AMF support, no feature.

- There would be a dependency on the AMF load and whether the AMD can keep a continuous MDT context active for the UE for a long time. If the AMF has issues with e.g. load, continuous MDT will fail.

- MDT is a process owned by the OAM, where the identifiers assigned to the MDT reports should be controlled by the OAM. If the AMF controls such identifiers, the process moves under AMF control, which implies a shift of functionalities from OAM to CN and that is not in line with the MDT functionality. Note that the OAM system is tightly coupled with the RAN for perfect interoperability, while the AMF is often from a different vendor than the RAN, creating potential interoperability problems for the whole feature.

- At change of AMF the identifier provided by the source AMF can no longer be used. Continuity of measurements is interrupted.

Proposal 7: Avoid the complexity and the functional changes caused by involving AMF and the Core Network in the Continuous MDT operation.

# Conclusions and Proposal

In this contribution the SA5 response LS is being discussed and the following proposals and conclusions were derived:

Observation 1: Continuous MDT discussions have so far been based on the possibility to introduce a semi-permanent identifier to the Trace Records containing management based MDT measurements, so to allow the TCE to determine that the measurements are collected by the same source

Proposal 1: RAN3 to agree to the following Continuous MDT problem statement:

Measurement continuity problem: how to ensure that the same UE is selected for collection of MDT measurements during sessions in the same RRC state, across different RRC states and across RRC_Connected mobility.

Trace Correlation Problem: how to ensure that the TCE receiving the Trace Record files containing MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection process from the same data source (UE).

Proposal 2: RAN3 to agree to the following principle to solve the measurement continuity problem, and to check its feasibility with SA5:

- The OAM provides to the RAN means to identify that the Management Based MDT process is for Continuous MDT, preferably reusing existing Trace Record information

Proposal 3: RAN3 to agree that Source gNB signals to Target gNB, as part of the Handover Signalling, an indication that the UE has been selected for Continuous MDT. Target gNB uses this information to re-select the UE for Continuous MDT.

Proposal 4: RAN3 to agree that a parameter associated with the duration of the continuous MDT collection is to be provided by OAM and check feasibility with SA5.

Conclusion 1: Some information, like the TR/TRSR, are always present in the trace records and could be exploited as correlation identifiers without the need to modify the Trace Recording header. Other information, like the RAN UE ID, are optionally present in the Trace Record and could also be considered as potential candidate for a correlation identifier.

Proposal 5: RAN3 to agree to the following principle and to check its feasibility with SA5:

- For correlation of MDT reports, RAN3 to provide guidance to SA5 stating that identifiers based on parameters contained in the Trace Recording header should be prioritised as they minimise the impact on current functionalities and implementations.

Proposal 6: Send an LS to SA5 to check the viability of re-using TR, TRSR, time stamp, RAN UE ID and CGI combinations as candidates for continuous MDT correlation identifiers.

Proposal 7: Avoid the complexity and the functional changes caused by involving AMF and the Core Network in the Continuous MDT operation.

An LS mirroring the proposals above is presented in [4].

# References

- RP-242385, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN.

R3-243941, Reply LS to SA5 on improved KPIs involving end-to-end data volume transfer time analytics, RAN3.

R3-245021, Reply LS on improved KPIs involving end-to-end data volume transfer time analytics, SA5.

R3-250464, [DRAFT] LS_to_SA5_Reply_LS_On_Imporved_KPIs, Ericsson (to be RAN3)