3GPP TSG-RAN WG3 Meeting #127	R3-250674

Athens, Greece, 17th – 21th Feb 2025

Title: 	(TP to TS36.410) Introduce S1 Removal to support Regenerative Payload in IoT NTN

Source: 	CMCC, ZTE, Samsung, CATT

Agenda item: 	15.2

Document Type:	Other

- Introduction

In the 3GPP TSG RAN Meeting #103, the work plan for Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 has been agreed in RP-240776:

The aim of this WI is enhancement of IoT-NTN with the following objectives:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN3 #125 meeting, the following is captured in chairlady’s notes:

RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later.

However in RAN#106, RP-243278 has been agreed to revise the WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3. A new objective for RAN3 has been added to:

  - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

2	Discussion

In this contribution, we will focus on discussing the enhancements for full eNB as regenerative payload, especially to address the feeder link switch over as needed.

In R19 NR NTN to support Regenerative Payload, it has been agreed to introduce NG Removal to avoid the unnecessary signals between gNB and 5GC. For the case of regenerative payload, when disconnecting from an AMF is required due to e.g. the satellite movement in a LEO constellation, the gNB on board the satellite may request the removal of the NG interface by initiating the NG Removal procedure toward the AMF.

In this way, satellite(gNB on board) can avoid storing too many UE contexts. It also avoids the un-necessary signal and data interworking over the feeder link, improves the efficiency of both satellite(gNB on board) and 5GC.

The same enhancement shall be introduced on S1 interface to support Regenerative Payload in IoT NTN.

Proposal 1: It is proposed to introduce S1 Removal functions initialized by eNB to support Regenerative Payload in IoT NTN. Add the function in TS36.410 as below TP.

3	Summary

In this contribution, we provide considerations on the IoT NTN in RAN3, following observations and proposals are made:

Proposal 1: It is proposed to introduce S1 Removal Procedures initialized by eNB to support Regenerative Payload in IoT NTN. Add the function in TS36.410 as below TP.

4	TP to TS 36.410

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

5.8	S1 interface management function

The error indication function is used by the eNB (respectively the MME) to indicate to the MME (respectively the eNB) that a logical error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the eNB and MME.

The S1 setup (respectively the eNB and MME configuration update) function allows to exchange (respectively update) application level data needed for the eNB and MME to interoperate correctly on the S1 interface.

The S1 removal function is used by eNB to initialize the removal S1 interfaces (with all application level configuration data) between MME.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>

5. Reference

[1] 	RP-240776, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2]       TR 23.700 -29, Study on integration of satellite components in the 5G architecture

[3]       TS 36.413, S1 application protocol (S1AP)

[4]       RAN3_126_agenda_202411221444_EOM, chairlady’s note