3GPP TSG RAN WG3 Meeting #127	R3-250016

Athens, Greece, 17 - 21 February, 2025

SA WG2 Meeting #SA2 166	S2-2412625

Orlando, USA, 18th Nov – 22nd Nov, 2024

Title:	LS on Location service of UEs served by MWAB

Response to:

Release:	Rel-19

Work Item:		VMR_Ph2

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact person:	Meng Li

raymond.limeng@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: TS 23.501 CR 5687

# 1	Overall description

While waiting for the response of S2-2405822 (LS on FS_VMR_Ph2 solution impacts to RAN) and S2-2407345 (LS on questions regarding FS_VMR_Ph2), SA2 would like to inform RAN3 that SA2 had progressed the work on location service of UEs served by MWAB in TS 23.273 v19.0.0 and TS 23.501 v19.1.0, and the related CRs are attached.

SA2 noted that such aspect may need the associating work in RAN3.

The potential impacts on RAN3 includes the following aspects as defined in clause 6.1.5 of TS 23.273:

-	TRP Information Exchange supported by MWAB-gNB in TS 38.455: "When the MWAB-gNB is integrated to the 5GS, the OAM triggers the LMF to perform TRP Information Exchange procedure. The LMF learns that a new integrated TRP at a MWAB-gNB is mobile and its UE ID (GPSI) of the MWAB-UE via a TRP information exchange towards the MWAB-gNB with the Cell ID of the TRP."

-	Additional ULI sent by the MWAB-gNB to the AMF in TS 38.413: when AMF is aware that the target UE is served by a MWAB based on the additional ULI, then "the AMF serving the target UE may indicate to the LMF that the serving cell is an MWAB (if applicable)".

-	Location and velocity information in TS 38.455: "If Network Assisted procedure is used, the MWAB-gNB may provide the MWAB-UE updated location and velocity information and the time obtained them to the LMF via NRPPa message as defined in TS 38.455."

And the following aspect as in the agreed CR 5687 of TS 23.501:

-	Additional ULI sent by the MWAB-gNB to the AMF in TS 38.413: "If serving cell change report is required by AMF, the MWAB-gNB shall report Additional ULI (and ULI) for the UE even when only serving cell of the MWAB-UE changes."

# 2	Actions

To RAN WG3

ACTION: 	SA2 kindly asks RAN WG3 to take the above information into account and provides feedback, if any.

# 3	Dates of next TSG SA WG2 meetings

S2-166-Ad Hoc-e           20 – 24, January 2025           Online

SA2#167	17– 21, February 2025	Athens