3GPP TSG RAN WG3 Meeting #127	R3-250021

Athens, Greece, 17 - 21 February, 2025

3GPP TSG-WG SA2 Meeting #166 	S2-2412876

Orlando, USA, November 18 – 22, 2024	(revision of S2-2412041, 12574, 12739)

# Title:	LS Reply on LP-WUS subgrouping

# Response to:	(S2-2411297/R2-2409226) LS on LP-WUS subgrouping

# Release:	Rel-19

# Work Item:	NR_LPWUS-Core

Source:	SA2

To:	RAN2, RAN3, CT1

Cc:

Contact Person:

Name:	Steven Wenham

Tel. Number:

E-mail Address:	steven DOT wenham AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR5824 (Rel-19 TS 23.501) and CR5131 (Rel-19 TS 23.502)

1. Overall Description:

SA2 thanks RAN2 on the progress and conclusion of using PEI subgrouping method as baseline for LP-WUS subgrouping as described in your LS in R2-2409226.

SA2 has agreed to use the existing CN based PEI subgrouping mechanism as the basis for the CN based LP-WUS subgrouping in the attached CRs for TS 23.501 and TS 23.502.

Additionally, SA2 discussed the following aspects:

1) 	SA2 noted that the name “LP-WUS” can be mis-understood, as SA2 understands that the Wake Up Signal is transmitted with high power to enable a large coverage area, and low power refers to a low power wake up receiver used in the mobile.

SA2 invites feedback on appropriate terminology and naming of the “LP-WUS” feature. SA2 will then align the terminology and name, based on the feedback.

2)	Initial Registration may be performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS) that analyses the UE’s UE Radio Access Capabilities to create the UE Radio Capability for Paging Information and send it to the AMF, see clause 5.4.4.3 of TS23.501. It will be important that LP-WUS related aspects of UE Radio Capability for Paging Information are not lost when Initial Registration is performed on a pre-Rel-19 gNB. If the LP-WUS aspects are lost, then there can be paging problems, when for example, paging is performed by a gNB that supports LP-WUS.

SA2 invites RAN2 and RAN3 to investigate this and provides solution if any issue is identified.

SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

2. Actions:

To RAN2, RAN3, CT1 group.

ACTION: 	SA2 asks RAN2, RAN3 and CT1 group to take the above into account and provide feedback if any.

ACTION: 	SA2 asks RAN3 and RAN2 to take the above into account and provide feedback on the terminology and name SA2 should use, and provide feedback on updating UE Radio Capability for Paging Information for the scenario mentioned above.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #166-Ad Hoc-e	Jan 20-24, 2025				Online

TSG-SA2 Meeting #167   	Feb 17 – 21, 2025				Athens, GR