3GPP TSG-RAN WG3 Meeting #93	R3-161966

Gothenburg, Sweden, 22 - 26 August 2016

Agenda item:	10.3.1

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Deployment options for NR and eLTE

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we provide a text proposal to resolve the following comeback related to R3-161769 [2]:

CB: # 28_TP_Clarification_alignements

- NG RAN definition/clarification/removal

- Be sure that nothing is missing if any removal

- eLTE eNB keep as it is … + build NG RAN reference on top

- Check reference non-standalone …

- Refine the FFS on CP EPC-gNB, pending to evolution of EPC against NG-core, clarified that no further analysis is foreseen

- Plenary alignment via Annex?

# 2	Text Proposal

Beginning of Text Proposal for TR 38.801

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP RP-160671: "New SID Proposal: Study on New Radio Access Technology".

[2]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[3]	3GPP TS 36.401: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture description".

[4]	3GPP TR 22.862: "FS_SMARTER - Critical Communications".

[5]	3GPP TR 38.913: "Study on Scenarios and Requirements for Next Generation Access Technologies".

[6]	3GPP TR 23.799: "Study on Architecture for Next Generation System".

[x]	RP-161266, "5G Architecture Options- Full Set". Joint RAN/SA Meeting, Busan, S. Korea, June 2016.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [2].

example: text used to clarify abstract rules by applying them literally.

eLTE eNB: The eLTE eNB is the evolution of eNB that supports connectivity to EPC and NG Core.

Network Slice: A Network Slice is a network created by the operator customized to provide an optimized solution for a specific market scenario which demands specific requirements with end to end scope as described in TR 23.799 [6].

Non-standalone NR: A deployment configuration where the gNB requires an LTE eNB as anchor for control plane connectivity to EPC, or an eLTE eNB as anchor for control plane connectivity to NGC.

Non-standalone Evolved E-UTRA: A deployment configuration where the eLTE eNB requires a gNB as anchor for control plane connectivity to NGC.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [2] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [2].

<ACRONYM>	<Explanation>

NGC	Next Generation Core

Next Change

## 6.3	Interfaces

Editor’s note: Intention is to capture protocol stacks and list of functions for each agreed interfaces in the RAN architecture.

### 6.3.1	General

The following options of [x] for providing NR access to suitably capable UEs should be considered in discussions on the RAN-CN interface, and the interface between (Evolved) E-UTRA and NR RAT.

Figure 6.3.1-1: Option 2

In Option 2, the gNB is connected to the NGC.

Figure 6.3.1-2: Options 3 and 3A

In Option /3A, the LTE eNB is connected to the EPC with Non-standalone NR.  The NR user plane connection to the EPC goes via the LTE eNB (Option 3) or directly (Option 3A).

Editor’s note: Terminology related to Option 3/3A can be further discussed, if needed.

Figure 6.3.1-3: Options 4 and 4A

In Option 4/4A, the gNB is connected to the NGC with Non-standalone Evolved E-UTRA.  The Evolved E-UTRA user plane connection to the NGC goes via the gNB (Option 4) or directly (Option 4A).

Figure 6.3.1-4: Option 5

In Option 5, the eLTE eNB is connected to the NGC.

Figure 6.3.1-5: Options 7 and 7A

In Option 7/7A, the eLTE eNB is connected to the NGC with Non-standalone NR.  The NR user plane connection to the NGC goes via the eLTE eNB (Option 7) or directly (Option 7A).

### 6.3.2	RAN-CN interface

#### 6.3.2.1	RAN-CN interface connectivity scenarios

In order to support the options described in section 6.3.1, the following scenarios for connectivity between a RAN consisting of E-UTRA, Evolved E-UTRA and NR, and a CN consisting of an NGC and an EPC should be considered in the discussions on RAN-CN interface definition. The connectivity scenario in figure 6.3.2.1-1 includes support for Option 3/3A, while figure 6.3.2.1-2 includes support for Option 2, Option 4/4A, Option 5, and Option 7/7A.

Figure 6.3.2.1-1: E-UTRA and NR connected to the EPC. The CP between EPC and gNB is FFS, but any further discussion in RAN3 is dependent on RAN Plenary decision.

Figure 6.3.2.1-2: Evolved E-UTRA and NR connected to the NGC (Note: In this scenario, eLTE eNB and gNB can be collocated.).

#### 6.3.2.2	General principles

The general principles for the specification of the NG1 interface are as follows:

-	the NG1 interface shall be open;

-	the NG1 interface shall support the exchange of signalling information between the NR BS and 5G CN;

-	from a logical standpoint, the NG1 is a point-to-point interface between an NR BS within the RAN and an 5G CN node. A point-to-point logical interface shall be feasible even in the absence of a physical direct connection between the NR BS and 5G CN;

-	the NG1 interface shall support contral plane and user plane separation;

-	the NG1 interface shall separate Radio Network Layer and Transport Network Layer;

-	the NG1 interface shall be future proof to fulfil different new requirements and support of new services and new functions;

-	the NG1 interface shall be decoupled with the possible RAN deployment variants.

NOTE 1:	The working assumption is that the interface between eLTE eNB and 5G CN is also NG1.

### 6.3.3	RAN internal interface

End of Text Proposal for TR 38.801

# 3	Conclusion

It is proposed to agree to the above Text Proposal for TR 38.801.

# References

- 3GPP TR 38.801 V0.2.0, Study on New Radio Access Technology; Radio Access Architecture and Interfaces, RAN3

- R3-161769, Deployment options for NR and eLTE, Nokia, Alcatel-Lucent Shanghai Bell