3GPP TSG-RAN WG3 Meeting #93	R3-162063

Gothenburg, Sweden, 22-26 August 2016

Source: 	AT&T

Title:	Text Proposal: Justification and Description with detail of Option 2 and 3

Agenda Item:	10.5.2

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide a text proposal to resolve the following comeback related to R3-161773:

Justification and description with detail of option 2 and 3

- include the pros/cons from R3-161628

- Provide FFS if needed

# Text Proposal

# -----------------------Start of Changes -----------------------

6.1.2.X   Justification and Detailed Description of Candidate Split Options

Option 2 - (PDCP/RLC) Split

Description:  In this split option, RRC, PDCP are in the central unit. RLC, MAC, physical layer and RF are in the distributed unit.

Benefits and Justification: This option will allow traffic aggregation from NR and eLTE transmission points to be centralized.  Additionally, it can facilitate the management of traffic load between NR and eLTE transmission points.   Fundamentals for achieving a PDCP-RLC split have already been standardized for LTE Dual Connectivity, alternative 3C. Therefore this split option should be the most straightforward option to standardize and the incremental effort required to standardize it should be relatively small.

Option 3 (High RLC/Low RLC) Split

Description: This option splits the RLC sublayer into High RLC and Low RLC sublayers such that for RLC Acknowledge Mode operation, the ARQ and packet ordering functions may be performed at the High RLC sublayer residing in the central unit, while the segmentation may be performed at the Low RLC sublayer residing in the distributed unit.

Benefits and Justification:

- Compared to the PDCP-RLC (Option 2) split, this option has the advantage of being more robust under non-ideal transport conditions because the ARQ and packet ordering is performed at the central unit

- This split option may also have better flow control across the split

- Centralization gains: ARQ located in the CU provides more centralization or pooling gains over Option 2

- The failure over transport network is also recovered using the end-end ARQ mechanism at CU. This provides protection for critical data and C-plane signaling

- DUs without functions of RLC can handle more connected mode UEs as there is no RLC state information stored and hence no need for UE context.

- Reduced processing and buffer requirements in DU due to absence of ARQ protocol

- Could be used over multiple radio legs of different DUs for higher reliability (U-Plane and C-Plane)

Cons

  - Comparatively, the split is more latency sensitive than the split with ARQ in DU, since re-transmissions are susceptible to transport network latency over a split transport network.

Overall, Option 3 where ARQ is located in CU provides significantly better pooling gains (packet processing) than Option 2. In addition, Option 2 requires larger packet buffers in DU. Therefore, it is beneficial to place ARQ function in CU according to the RAN function mapping shown in Option 3.

# -----------------------End of Changes -----------------------

# References

- R3-161773 Preferred NR Functional Splits and Considerations – AT&T

- R3-161628 Analysis of Higher-Layer functional split option – Nokia, Alcatel-Lucent Shanghai Bell