3GPP TSG-RAN WG3 Meeting # 96	R3-171785

Hangzhou, China, May 15 – 19, 2017

Agenda item:		10.2

Source:	Huawei.

Title:	TP for Correction to RAN Selection of CN entity

Document for:		pCR

# 1. Introduction

This contribution provides text proposal for TS 38.300 on RAN selection of CN entity.

START OF CHANGES

# X     RAN support of Network Slicing

## X.1	General Principles and Requirements

RAN selection of CN entity

-	For initial attach, the UE may provide assistance information to support the selection of an AMF. If available, NG RAN uses this information for routing the initial NAS to an AMF. If NR RAN is unable to select an AMF using this information or the UE does not provide any such information the NG RAN sends the NAS signalling to a default AMF.

-	For subsequent accesses, the UE provides a Temp ID, which is assigned to the UE by the 5GC, to enable the NG RAN to route the NAS message to the appropriate AMF as long as the Temp ID is valid (NG RAN is aware of and can reach the AMF which is associated with the Temp ID). Otherwise, the methods for initial attach applies.

END OF CHANGES