3GPP TSG-RAN3 #97	 R3-173045

Berlin, Germany, 21 – 25 Aug. 2017

Agenda Item:	10.10.1.4

Source:	China Telecom

Title:	TP for system information in disaggregated gNB

Document for:	pCR

Introduction

In the following we propose a pCR to capture the proposals in R3-173043.

pCR for TS 38.401

Start of Text Proposal for TS 38.401

- 10.3.2.2.4	System Information management function

Scheduling of system broadcast information is carried out in the gNB-DU. The gNB-DU is responsible for transmitting the system information according to the scheduling parameters available.

Editor’s note: How to support this function is FFS.

The parameters in the MIB message are controlled by the gNB-DU. The gNB-CU can retrieve the MIB message parameters that are configured by the gNB-DU by sending a specific request over the F1-C interface.

gNB-CU controls the MIB message transmission by configured MIB transmission parameters to gNB-DU over F1-C. gNB-DU generates, stores the MIB RRC message and transmits it to UEs over the air based on the gNB-CU MIB transmission configuration parameters.

The parameters in the SIB message are controlled by the gNB-CU.

For SIB messages, gNB-CU generats the SI RRC messages and transmits them to gNB-DU with SI transmission configuration parameters over F1-C, gNB-DU stores and transmits these RRC messages to UEs over the air based on the gNB-CU SI transmission configuration parameters.

NOTE: With reference to the system information parameters in LTE:

Under the gNB-CU control, the gNB-DU can provide the on-demand SIB messages to UEs in RRC_IDLE or RRC_INACTIVE without involving the gNB-CU.

End of Text Proposal for TS 38.401