3GPP TSG-RAN WG3 AH #2	Tdoc R3-172520

Qingdao, P. R. China, 27th – 29th June 2017

Agenda Item:	10.10.3

Source:	Ericsson

Title:	Xn handover in disaggregated gNB

Document for:	Discussion, Decision

# Introduction

During RAN3 #96 the procedures over the F1 interface were discussed. It was noted that the stage 2 procedures should be agreed first. Afterwards, we can start discussing the stage 3 procedures and the encoding of the messages. In this contribution, we discuss the stage 2 Xn handover procedure in a disaggregated gNB.

# Discussion

We consider the Xn handover procedure, which implies that the UE changes both the gNB-DU and gNB-CU. This is the most comprehensive handover procedure. The intra-gNB handover procedures can be logically derived from the procedure discussed in this contribution (e.g., the intra-CU inter-DU handover procedure). The call-flow for the Xn handover is shown in Fig. 1.

The following terminology is utilized to describe the entities involved in the procedure:

- S-gNB: source gNB

- T-gNB: target gNB

- S-DU: source gNB-DU

- S-CU: source gNB-CU

- T-DU: target gNB-DU

- T-CU: target gNB-CU

The call-flow can be logically divided into three phases.

- The first phase is the Xn handover preparation:

  - UE sends a RRC MEASUREMENT REPORT to S-DU. The event that triggers the measurement report depends on the UE measurement configuration.

  - S-DU employs the F1AP UL RRC TRANSFER message to forward the RRC measurement report to S-CU.

  - S-CU takes the handover decision, based on the content of the RRC measurement report. S-gNB (S-CU) sends the XnAP HANDOVER REQUEST message to T-gNB (T-CU), passing necessary information to prepare the handover at the target side.

  - T-CU performs admission control, creates a UE context, and identifies T-DU. T-CU configures the higher-layers, including security configuration and QoS-flows to DRB mapping. Then, T-CU sends the F1-AP CONTEXT SETUP REQUEST message to T-DU, including UE context information and CU-UL-TEID for data radio bearers.

  - T-DU performs admission control, configures the lower-layers, and creates a local UE context (including a C-RNTI for the UE). Then, T-DU sends the F1-AP CONTEXT SETUP RESPONSE message to T-CU, including lower-layers configuration, C-RNTI, and DU-DL-TEID for data radio bearers.

  - T-gNB (T-CU) sends the XnAP HANDOVER REQUEST ACKNOWLEDGE to S-gNB (S-CU). The XnAP HANDOVER REQUEST ACKNOWLEDGE message includes a transparent container to be sent to the UE as an RRC message to perform the handover (includes C-RNTI). The XnAP HANDOVER REQUEST ACKNOWLEDGE message may also include RNL/TNL information for the data forwarding tunnels (if applicable).

- Fig. 1: Xn handover in the disaggregated gNB deployment.

- The second phase is the Xn handover execution:

- S-CU sends the F1AP UE CONTEXT MODIFICATION REQUEST message to S-DU. The F1AP UE CONTEXT MODIFICATION REQUEST message includes a transparent container that carries the RRC message to perform handover, which was generated by T-CU and ciphered and integrity protected by S-CU. The F1AP UE CONTEXT MODIFICATION REQUEST message contains a specific field value to inform S-DU about the handover.

- S-DU sends the F1AP UE CONTEXT MODIFICATION RESPONSE message to S-CU to confirm that the handover is accepted.

- S-DU sends the RRC CONNECTION RECONFIGURATION MESSAGE to the UE.

- S-gNB (S-CU) sends the XnAP SN STATUS TRANFER message to T-gNB (T-CU) to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status.

- Data forwarding may start from S-gNB (S-CU) to T-gNB (T-CU) (if applicable).

- UE sends a RANDOM ACCESS preamble to T-DU. It may employ a dedicated RACH preamble if that was included in the RRC connection reconfiguration message.

- T-DU sends a RANDOM ACCESS RESPONSE to UE.

- UE sends the RRC CONNECTION CONFIGURATION COMPLETE to T-DU (include the C-RNTI to identify the UE).

- T-DU employs the F1AP UL RRC TRANSFER message to forward the RRC connection reconfiguration complete message to T-CU.

- T-gNB (T-CU) sends a NGAP PATH SWITCH REQUEST message to AMF to inform that the UE has changed cell. AMF should then contact the UPF to modify PDU session (not shown).

- AMF confirms the NGAP PATH SWITCH REQUEST message with the NGAP PATH SWITCH REQUEST ACKNOWLEDGE message.

- The last phase is the resource release in the source:

- TgNB (T-CU) sends the XnAP UE CONTEXT RELEASE message to S-gNB (S-CU). The XnAP UE CONTEXT RELEASE message informs S-CU that the handover was successful.

- S-CU sends the F1AP UE CONTEXT RELEASE message to S-DU, to allow T-DU to release the resources allocated to the UE.

# Conclusion

We conclude with the following proposals:

Proposal 1	RAN3 is kindly asked to agree with the proposed call-flow for Xn handover in disaggregated gNB deployment.

Proposal 2	RAN3 is kindly asked to agree with the TP in R3-172521 [1]

# References

R3-172521, “TP for Xn handover in disaggregated gNB”, Ericsson