3GPP TSG-RAN WG4 Meeting # 100-e						R4-2112016

Electronic Meeting, 16th – 27th August, 2021

Agenda item:			9.13.2

Source:	Samsung

Title:	NR-NTN calibration summary and observations

Document for:	Discussion

# Introduction

Since RAN4 99e meeting agreed the calibration activity to be held offline between 99e and 100e meeting, we had then initiated the offline discussion email thread and kicked off the calibration data submission and technical discussion. The email group has received calibration data from 10 companies, including Samsung, Qualcomm, CATT, THALES, Huawei, Xiaomi, ZTE, Nokia, Ericsson and FhG, and covers all NTN systems and NR (TN) systems.

This document would summarize this offline activity and share some of our observations for discussion in 100e meeting. Also, the calibration table is attached to this document as Annex 1.

Annex 1: R4-2112016-A1 NTN co-existence calibration

# Summary and observations

## NTN DL calibration

For NTN DL calibration, we have received the data from up to 10 companies for LEO-600, LEO-1200 and GEO, but only received 2 companies who provided HAPS DL data.

Figure NTN DL Rural (38.811) Calibration Data

Figure NTN DL Urban (38.811) Calibration Data

Observation 1:

For NTN DL coupling-loss,

- LEO-600, LEO-1200 and GEO’s DL coupling-loss are quite calibrated, the data from all companies are quite close and most variances are almost under 1.

- HAPS DL coupling-loss still needs more discussion to calibrate.

For NTN DL SINR,

- The differences of LEO-600, LEO-1200 and GEO SINR is from 1 or 2 companies, who did not consider the adjacent beam interference. Their SINR actually represents SNR value. And most other companies’ data are quite close.

- HAPS DL SINR data is better than its CL data, but still requires more discussion, since CL is mis-matching. The number of contributing companies are too small.

## NTN UL calibration

For NTN UL calibration, we also received up-to 10 companies’ data in LEO-600, LEO1200 and GEO, and up to 2 companies’ data in HAPS.

Figure NTN UL Rural (38.811) Calibration Data

Figure NTN UL Urban (38.811) Calibration Data

Observation 2:

For NTN UL coupling-loss,

- LEO-600, LEO-1200 and GEO’s UL coupling-loss are quite calibrated, the data from all companies are quite close and most variances are almost under 1.

- HAPS UL coupling-loss still needs more discussion to calibrate, and the number of contributing companies are too small.

For NTN UL SINR,

- Apart from 2~3 companies’ data, the LEO-600, LEO-1200 and GEO UL SINR data are quite close to each other. The assumptions alignment discussion is now on-going. Since the CL is quite close, SINR should be able to align once the understanding of the assumptions could be aligned.

- HAPS UL data only has 2 contributors, and the current data are quite diverse. It still requires more contributions to calibrate.

For NTN UL Tx power,

- LEO-600, LEO-1200 and GEO UL Tx power are the same from different contributors. Considering the CL is quite matched, the Tx power should not have any critical issues for calibration.

- HAPS UL tx power is still quite diverse, it is connected to the UL CL diversity.

## TN NR DL calibration

For TN NR DL, we have two different base station (BS) antenna types, one is AAS, and the other is non-AAS. We received up-to 6 companies’ data for AAS TN DL, and up-to 4 companies’ data for non-AAS DL case.

Figure TN NR DL AAS Calibration Data

Figure TN NR DL non-AAS Calibration Data

Observation 3:

The coupling-loss and SINR of TN has smaller issues than NTN. Most companies has already calibrated their simulator in previous 3GPP studies. In offline discussion, we found there are several different understandings of the assumptions, like ISD, Antenna pattern, polarization gain, etc. We believe this should has no critical issue once the assumptions are aligned.

## TN NR UL calibration

For TN NR UL, we have two different base station (BS) antenna types, one is AAS, and the other is non-AAS. We received up-to 6 companies’ data for AAS TN UL, and up-to 4 companies’ data for non-AAS UL case.

Figure TN NR UL AAS Calibration Data

Figure TN NR UL non-AAS Calibration Data

Observation 4: In offline discussion, we found there are several different understandings of the assumptions, like ISD, Antenna pattern, polarization gain, etc. We believe this should has no critical issue once the assumptions are aligned, since most companies has calibrated their simulator in previous 3GPP studies.

## NB-IoT calibration

We have not yet received any calibration data from any participating companies for NB-IoT scenario. Thus, we are not able to calibrate the NB-IoT system at this stage.

Observation 5: Since no contributions received for NB-IoT scenario, we are not able to calibrate this system at this stage.

# Conclusions

From the all the calibration efforts that the offline group has received so far, we have the following observations that would like to share to the meeting for further discussions.

Observation 1:

For NTN DL coupling-loss,

- LEO-600, LEO-1200 and GEO’s DL coupling-loss are quite calibrated, the data from all companies are quite close and most variances are almost under 1.

- HAPS DL coupling-loss still needs more discussion to calibrate.

For NTN DL SINR,

- The differences of LEO-600, LEO-1200 and GEO SINR is from 1 or 2 companies, who did not consider the adjacent beam interference. Their SINR actually represents SNR value. And most other companies’ data are quite close.

- HAPS DL SINR data is better than its CL data, but still requires more discussion, since CL is mis-matching. The number of contributing companies are too small.

Observation 2:

For NTN UL coupling-loss,

- LEO-600, LEO-1200 and GEO’s UL coupling-loss are quite calibrated, the data from all companies are quite close and most variances are almost under 1.

- HAPS UL coupling-loss still needs more discussion to calibrate, and the number of contributing companies are too small.

For NTN UL SINR,

- Apart from 2~3 companies’ data, the LEO-600, LEO-1200 and GEO UL SINR data are quite close to each other. The assumptions alignment discussion is now on-going. Since the CL is quite close, SINR should be able to align once the understanding of the assumptions could be aligned.

- HAPS UL data only has 2 contributors, and the current data are quite diverse. It still requires more contributions to calibrate.

For NTN UL Tx power,

- LEO-600, LEO-1200 and GEO UL Tx power are the same from different contributors. Considering the CL is quite matched, the Tx power should not have any critical issues for calibration.

- HAPS UL tx power is still quite diverse, it is connected to the UL CL diversity.

Observation 3:

The coupling-loss and SINR of TN has smaller issues than NTN. Most companies has already calibrated their simulator in previous 3GPP studies. In offline discussion, we found there are several different understandings of the assumptions, like ISD, Antenna pattern, polarization gain, etc. We believe this should has no critical issue once the assumptions are aligned.

Observation 4: In offline discussion, we found there are several different understandings of the assumptions, like ISD, Antenna pattern, polarization gain, etc. We believe this should has no critical issue once the assumptions are aligned, since most companies has calibrated their simulator in previous 3GPP studies.

Observation 5: Since no contributions received for NB-IoT scenario, we are not able to calibrate this system at this stage.

# Reference