3GPP TSG-RAN WG4 Meeting # 100-e	R4-2113769

# Electronic Meeting, August 16th – 27th, 2021

Title: 	Updated simulation results for NR-U PF3

Source: 	Huawei, HiSilicon

Agenda item:	6.1.1.7.4

Document for:	Discussion

Background

As per the provided simulation results [2] during last RAN4#99-e meeting, there is still large span among the simulation results for NR-U PUCCH format 3. We provide our updated our simulation results in this paper after double checking our results.

Simulation results

The updated simulation results are captured in Table 2-2 with simulation assumptions listed in Table 2-1 [1]:

Table 2-1: Simulation assumptions for NR-U PF3

| Parameter | Test 1 |
| --- | --- |
| Modulation order | QPSK |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Additional DM-RS configuration | No additional DM-RS |
| Number of symbols | 4 |
| The number of UCI information bits Note 1 | 4 bits |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Antenna configuration | 1T2R |
| Number of interlaces | 1 |
| Interlace index | 0 (Note 2) |
| Propagation conditions | TDLA30-10 Low |
| Index of OCC | Not configure |
| Length of OCC | Not configure |
| Cyclic shift index for DMRS | 0 |
| Test metric | SNR@Prob(ACK miss)≤10^(−2) |
| Note 1: The UCI information only contain HARQ-ACK and the bit pattern is [1 1 1 1]. Note 2: RBs 0, 10, 20,…,90 are allocated for 15kHz and RBs 0,5,10,…,45 are allocated for 30kHz. |  |

Table 2-2: Simulation results for NR-U PF3

| SCS | Ideal results | Impairment results |
| --- | --- | --- |
| 15KHz | -8.5dB | -6.5dB |
| 30kHz | -7.4dB | -5.4dB |

Conclusion

In this paper, we provide our updated our simulation results for NR-U PF3.

References

[1] R4-2108515, Way forward on NR-U BS demodulation requirements, RAN4#99-e, Huawei, HiSilicon.

[2] R4-2110507, Summary of ideal and impairment results for NR-U BS demodulation requirements, RAN4#99-e, Huawei, HiSilicon.