3GPP TSG-RAN WG4 Meeting #100-e                         	         R4-2114043

# Electronic meeting, 16th August – 27th August, 2021

Agenda item:	8.38.2

Source:	Huawei, HiSilicon

Title:	Text proposal for TR 37.827 to include DC_1A-5A_n78A

Document for:	Approval

# Introduction

This contribution is a text proposal for TR 37.827 to include DC_1A-5A_n78A according to the request in [1].

# Text Proposal

<Start of Text Proposal>

## 5.x	DC_1A-5A_n78A

### 5.x.1	Transmitter Characteristics

#### 5.x.1.1	Maximum Output Power

Table 5.x.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_aA_n78A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.x.1.2	Co-existence study

According to the PC3 DC_1A_5A-n78A study in 37.863-02-01, the Rx impacts are identified as below,

- Co-existence analysis for DC_1_n78 shows that there is IMD4 impact from DC_1_n78 UL to Band 1 and IMD5 impact from DC_1_n78 UL to Band 5 DL.

- Co-existence analysis for DC_5_n78 shows that there is IMD4 impact from DC_5_n78 UL to Band 5 DL and IMD3 impact from DC_5_n78 UL to Band 1 DL.

Thus, additional MSD for IMD 4 should be considered to mitigate the impact of the interference for PC2 DC_1A_5A-n78A combination.

### 5.x.2	Receiver Characteristics

#### 5.x.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.x.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_1A_5A-n78A are defined in table 5.x.2.2.1-1.

Table 5.x.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-5A_n78A | 1 | 1930 | 5 | 25 | 2120 | 19.2 | IMD4 |
|  | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | n78 | 3670 | 10 | 52 | 3670 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 5 | 844 | 5 | 25 | 889 | 19.2 | IMD4 |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1932 | 5 | 25 | 2122 | 27.0 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 52 | 3780 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 13.2 | IMD5 |
|  | n78 | 3405 | 10 | 52 | 3405 | N/A | N/A |

#### 5.x.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_1A_5A-n78A are the same as the Case A defined in table 5.x.2.1.1-1.

<End of Text Proposal>

# References

- RP-211172, Revised WID on PC2 EN-DC with x LTE band + y NR band (x= 2, 3, 4, y=1; x=1, 2, y=2)