3GPP TSG-RAN4 WG4 Meeting # 100-e	R4-2114950

- Electronic meeting, August. 16 – 27, 2021

Title:	[draft] LS on Scell dropping issue for CA

Response to:

Release:	Release 17

Work Item:	NR_RF_FR1_enh-Core

Source:	3GPP TSG RAN WG4

To:	RAN1

Cc:	RAN5

Contact Person:

- Name:	Zhangqian

- E-mail Address:	zhangqian67@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Reference:

[1] R4-2103410, “LS on Scell dropping”, RAN4 #98-e

1. Overall Description:

RAN #92-e tasked RAN4 to further discuss on Scell dropping issue. Discussion on Scell dropping of CA is triggered by a measurement problem found in MoP(maximum output power) of FR2 intra-band UL CA and later commented the same unexpected SCell dropping issue is found problematic also in field operation (R4-2112826). Meanwhile, it is also confirmed by RAN4 that the problem also exists for FR1 CA. The background and suggestion on CA measurements are captured in [1] and sent to RAN5.

In TS 38.213, the transmission priority rule for CA is specified based on physical channel, and also with: “In case of same priority order and for operation with carrier aggregation, the UE prioritizes power allocation for transmissions on the primary cell of the MCG or the SCG over transmissions on a secondary cell”. Thus, the UE is always required to drop the power on cell with lower priority when the configured power on cell with high priority occupies all power ability of the UE.

In order to advance the above discussions, RAN4 would like to ask RAN1 following questions:

Question 1: Whether UE drop Scell power according to the priority rule defined in 38.213 is considered as an issue from RAN1 perspective.

Question 2: Whether UE drop Scell power according to the priority rule defined in 38.213 has been addressed from 16 or 17? If not, what expected solution(s) are?

OPPO: the current wording is strong. We are not sure if RAN1 think it is issue or not.

Qualcomm: It should be addressed in RAN5. I do not think that we should ask RAN1 whether it is an issue.

Huawei: we share the similar view as Qualcomm. RAN4 and RAN5 have addressed the issue. There would be conflict between groups.

Qualcomm: we are not OK to ask RAN1. We do not work on different things across RAN1~5. I do not understand why we should ask RAN1.

OPPO: we need check whether RAN1 is working on the issue.

Question 3: If the problem above is solved in RAN4 specifications with solution by higher layer configuration, e.g. introduce additional UE-specific configuration of power limits on Pcmax,f,c for each CC to prevent SCell dropping (see e.g. R4-2112826 or R4-2114551 for details), is there any expected RAN1 spec impact or possible conflict with UE behaviour defined in RAN1 specifications?

2. Actions:

To: 3GPP TSG RAN WG1.

ACTION: RAN4 respectfully asks RAN1 to provide the answers to the above questions.

3. Date of Next TSG-RAN WG4 Meetings:

3GPP RAN4 #101-e		01 – 12 Nov 2021			      Electronic Meeting

3GPP RAN4 #101b-e 		17 – 25 Jan 2022		        	   TBD