3GPP TSG-RAN WG4 Meeting #100-e                      								        R4- 2113036

- Online, 16th August – 27th August, 2021

Source:	Vodafone

Title:	TP for TR 36.717-03-01: CA_3A-3A-8A-38A

Agenda Item:	11.2.3

Document for:	Approval

# Introduction

This contribution provides a text proposal on LTE CA band combination CA_3A-3A-8A-38A for TR 36.717-03-01 [1].

The TIB,c and RIB values provided mirror the values already present for CA_3-8-38 in 36101.

# Reference

- TR 36.717-03-01, Rel-17 LTE inter-band Carrier Aggregation for 3 bands DL with 1 band UL

# Text Proposal

----- Start of TP -----

## 5.x	CA_3A-3A-8A-38A

### 5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_3A-3A-8A-38A | - | 3 | See CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 70 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.x.2	∆TIB and ∆RIB values

For CA_3-3-8-38, the TIB,c and RIB,c values are shown in table 5.x.2-1 and table 5.x.2-2, respectively.

Table 5.x.2-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3-3-8-38 | 3 | 0.5 |
|  | 8 | 0.3 |
|  | 38 | 0.5 |

Table 5.x.2-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3-3-8-38 | 3 | 0 |
|  | 8 | 0 |
|  | 38 | 0 |

### 5.x.3	REFSENS requirements

No additional MSD required compared to fallbacks.

----- End of TP -----