3GPP TSG-RAN WG4 Meeting #100-e                      								        R4- 2113295

- Online, 16th August – 27th August, 2021

Source:	Vodafone

Title:	TP for TR 36.717-04-01: CA_1A-3A-7A-20A-38A

Agenda Item:	11.3.3

Document for:	Approval

# Introduction

This contribution provides a text proposal on LTE CA band combination CA_1A-3A-7A-20A-38A for TR 36.717-04-01 [1].

The TIB,c and RIB values provided are derived from a combination of the values already present for CA_1-3-7-20, CA_1-3-7-38 and CA_1-3-20-38 in 36101, and the values proposed in parallel TPs for the missing fallbacks.

# Reference

- TR 36.717-04-01, Rel-17 LTE inter-band Carrier Aggregation for x bands DL (x=4, 5) with 1 band UL

# Text Proposal

----- Start of TP -----

## 5.x	CA_1A-3A-7A-20A-38A

### 5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-7A-20A-38A1 | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Band 1, 3 or 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 5.x.2	∆TIB and ∆RIB values

For CA_1-3-7-20-38, the TIB,c and RIB,c values are shown in table 5.x.2-1 and table 5.x.2-2, respectively.

Table 5.x.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-20-38 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.3 |

Table 5.x.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-20-38 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |

### 5.x.3	REFSENS requirements

No additional MSD required compared to fallbacks.

----- End of TP -----