3GPP TSG-RAN WG4 Meeting #100-e	R4-2115341

Electronic Meeting, 16 – 27 August, 2021

Title:	LS on support of pre-configured MG

Response to:

Release:	Release 17

Work Item:	NR_MG_enh-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	Li Zhang, Xusheng Wei

E-mail Address:	zhangli164@huawei.com, xusheng.wei@vivo.com

Attachments: -

1. Overall Description:

For Rel-17 measurement gap (MG) enhancement WI, RAN4 has discussed support of pre-configured MG, and reached the following conclusions that both NW-controlled and autonomous pre-configured MG activation/deactivation are supported.

- The pre-configured MG activation/deactivation is triggered by the DCI/Timer based BWP switch - FFS if additional conditions for pre-configured MG activation/deactivation shall be considered - NW can control activation/deactivation of pre-configured MG for the specific BWP - Option 1: via its RRC configuration message - Option 2: via DCI or MAC configurations - Additional explicit rules for pre-configured MG autonomous activation/deactivation shall be defined for the case when signalling is not provided - UE capability on the support of NW-controlled and autonomous pre-configured MG activation/deactivation mechanisms can be further discussed

As to the configuration of pre-configured MG, RAN4 has further reached the following conclusion.

- The common configuration parameters of pre-configured MG (e.g. MGRP, MGL, etc) which are same as these of Rel-16 legacy MG can be configured by the similar way as the Rel-16 legacy MGs - The RRC parameter(s) used to differentiate pre-configured MG with the legacy MG are needed when pre-configured MG is being configured - The exact signalling can be designed by RAN2

RAN4 respectfully asks RAN2 to take the above information into account and define the signalling support for configuration of pre-configured MG.

2. Actions:

To RAN2:

RAN4 respectfully asks RAN2 to take the above information into account and define the signalling support for configuration of pre-configured MG.

3. Date of Next TSG-RAN4 Meetings:

TSG-RAN4 Meeting #101-e		  	    01 – 12 November, 2021