3GPP TSG-RAN4 Meeting #97-e	TDoc#

# Electronic Meeting, <Start_Date> - <End_Date>

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR on 38.101-1 Maintenance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 NR_newRAT-Core TEI16 5G_V2X_NRSL 5G_V2X_NRSL-Core NR_CADC_R16_2BDL_xBUL-Core NR_n41_LTE_41_coex-Core NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 16-11-2021 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | R4-2117103: Not enough margin to meet NS_38 and NS_39 requriement with the specified backoff based on the original simulation plots from RAN4#86, and there is ambiguity in the AMPR since regions A and B overlap for NS_38. R4-2117668: In the texts added in the last meeting, some clarifications and corrections are needed: - Applicability of 2 UL bands text and NS_xxU/100 should be made clear.. - Applicability texts for a single Band UL are duplicated. R4-2118971: As shown in the table A.3.1-1, number of PRB for CSI-RS for tracking is defined as the same size with the BWP size. And the CSI-RS periodicity is specified as follows. Now it is described in TS38.214 that it is not expected to configure with 2^ x 10 slots periodicity when CSI-RS resource size is larger than 52RBs. Thus it is not allowed to configure 2x1*10= 20 slot for example with SCS 30 kHz BW 100 MHz (BWP size = 273RBs). Also, the periodicity should be a multiple of 10 so as not to affect the DL RMC patterns. Extract from TS38.214 clause 5.1.6.1.1 the UE is not expected to be configured with the periodicity of slots if the bandwidth of CSI-RS resource is larger than 52 resource blocks. Extract from TS38.331 clause 6.3.2 CSI-ResourcePeriodicityAndOffset ::= CHOICE { slots4 INTEGER (0..3), slots5 INTEGER (0..4), slots8 INTEGER (0..7), slots10 INTEGER (0..9), slots16 INTEGER (0..15), slots20 INTEGER (0..19), slots32 INTEGER (0..31), slots40 INTEGER (0..39), slots64 INTEGER (0..63), slots80 INTEGER (0..79), slots160 INTEGER (0..159), slots320 INTEGER (0..319), slots640 INTEGER (0..639) } R4-2117100: In RAN4#100-e, the issue was raised that it is difficult to meet -50 dBm/MHz at 1475 - 1488 MHz as own Rx protection under the condition of channel bandwidths and those frequency offsets described in NOTE 42 for n74 in spurious emissions for UE co-existence table. n74 is assumed to implement a dual duplexer where lower duplexer is from 1427 - 1460MHz and upper duplexer is from 1440 - 1470 MHz, and the cause of this issue is that it is required to -50 dBm/MHz at 1475 - 1488 MHz even when a UE transmits in n74 with upper duplexer. 1) To address this issue, NOTE 42 will be modified as NOTE 42A so that -50 dBm/MHz at 1475 - 1488 MHz applies to UE transmitting in n74 with lower duplexer but does not apply to with upper duplexer. 2) Due to the modification of NOTE 42, missing protection requirements of -28dBm/MHz at 1475 – 1488MHz and of 35dBm/MHz at 1475.9 – 1510.9 MHz will be added in general requirements and NS_39 related requiremens since these were originally covered by NOTE 42. In addtion to above, 3) The upper edge of the frequency range of A-MPR for NS_38 for 5MHz will be modified from 1437.5 to 1434.5MHz since the frequency range between 1437.5 and 1434.5MHz is covered by NOTE 41 in UE co-existence table. 4) For EESS protection, the desciption of “This requirement shall be verified with UE transmission power of 15 dBm” will be added to NOTE 41 in UE co-existence table since it has already captured in additional requirement for NS_38 in Table 6.5.3.3.7-1. R4-2118967: Specifying parameter such as relative power error at the antenna port (logical port) is not appropriate, the specification at actual physical antenna ports makes more sense for RF related parameters. For FR2, the maximum allowable difference of relative phase and power errors requirements are specified at the physical antenna ports (corresponding to antenna connectors for FR1). Please see contribution R4-2118965 for more details R4-2119845: MOP requirements were missed in Table 6.2B.1.3-1 while there are more NR DC combos in Table 5.5B.1-1. Update: Thanks for ZTE’s check the proposed changes on the combos shall not belong to Rel-16. Thus the CR is revised accordingly R4-2119847: There are a few US inter-band CA combinations with Band n77 where the MSD test points for n77 either as UL aggressor carrier or DL victim carrier do not fall within US Band n77 frequency ranges. These MSD requirements would not be testable. R4-2118455: Correct the note in table 5.3.5-1 Note 9 is the same as note 5. R4-2119848: Based on the agreement in WF R4-2114977, RAN4 can still keep the related RF requirements with brackets and it will be replaced for the A-MPR and A-SEM requirements based on the FCC updating emission limits. And a Note could be considered that the requirements are pending update and test is not needed for NS_52. R4-2119849: Update the SRS-TxSwitch capability IE to align with 38.331. R4-2119850: Some errors should be corrected for UE maximum output power reduction. R4-2119291: In clause 5.4.2.3 the channel raster is defined for each band. For frequency bands with exactly two channel rasters the fifth paragraph defines the elegible SCS. In case of band n90, three different ΔFRaster are specified and the fifth paragraph does not seem to apply. The wording needs to be updated to fit for n90. R4-2119851: The relaxations ∆TRxSRS to Pcmax for SRS antenna switching only apply to power classes 2 and 3 and are limited in frequency to n79 and below. R4-2119497: Corrections needs in V2X PCmax |  |  |  |  |  |  |  |  |
| Summary of change: |  | R4-2117103: Modify the slope of the region A boundary from LCRB/2 to LCRB to cover all backoff >= 8dB to ensure power is = +15dBm to meet EESS requirement. Add the condition in region B (RBstart > -3.6MHz + LCRB) so that no ambiguity exists for AMPR for region A and region B. Modify the NS_39 RBend condition for 20MHz BW from 14.4MHz to 12.6MHz to match the original simulation plot where backoff is required. Revision 1 corrected the coversheet error of TS38.101-1 to 38.101 as well as corrects the LCRB inequality of NS_38 region A from “>” to “≥”. R4-2117668: Applicability of additional requirements are clarified and relevant text is added to NOTE 1 for NS_xxU/100. One of the duplicated text for a single Band UL is removed. One of the duplicated two band UL applicability text is removed. R4-2117100: - NOTE 42 will be modified as NOTE 42A so that -50 dBm/MHz at 1475 - 1488 MHz applies to UE transmitting in n74 with lower duplexer but does not apply to with upper duplexer. - Due to the modification of NOTE 42, missing protection requirements of -28dBm/MHz at 1475 – 1488MHz and of 35dBm/MHz at 1475.9 – 1510.9 MHz will be added in general requirements and NS_39 related requiremens - The upper edge of the frequency range of A-MPR for NS_38 for 5MHz will be modified from 1437.5 to 1434.5MHz. - For EESS protection, the desciption of “This requirement shall be verified with UE transmission power of 15 dBm” will be added to NOTE 41 in UE co-existence table R4-2118967: Change the requirements of Table 6.4D.4-1 [1] as being defined at the antenna connectors instead of as currently at the antenna ports. R4-2118971: CSI-RS periodicity is changed as follows. SCS15kHz : 10 slots -> 20 slots SCS30kHz : 20 slots -> 40 slots SCS60kHz : 40 slots -> 80 slots R4-2119845: Remove the combos that shall not existe in the Rel-16 in Table 5.5B.1-1 R4-2119847: 1. Modify US inter-band CA combinations with Band n77 where the MSD test points for n77 do not fall within US Band n77 frequency ranges or point the test configuration to the note stating “For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped.” 2. Correct CA_n5-n77 MSD values due to UL 5th harmonic for a few channel BWs. R4-2118455: Correct below 1. Change note 9 to void. 2. Change the superscript 9 to 5 for 5MHz of n40 and n50. R4-2119848: A Note is added that The ASE requirements for NS_52 will not be verified until the corresponding regulation release a formal rule for C-V2X emission limits. R4-2119849: Align SRS-TxSwitch capability IE with 38.331. R4-2119850: - Correct outer RB allocation value for MPR in Table 6.2.2-1. - Correct the description for CP-OFDM allocation condition. R4-2119291: The wording of the fifth paragraph is updated to fit for band n90. R4-2119851: Applicability of ∆TRxSRS is extended to power class 5 and for bands above n79 R4-2119497: - V2X PCmax applies V2X power class, PPowerclass moved to PPowerclass,V2X - None of the SL bands define ΔTC, that is removed from pcmax formula - ∆TIB,c Pcmax for V2X is referring ∆TIB,c definition in 6.2.4 but 6.2.4 does not define V2X concurrent operation. V2X operating bands for con-current operation is added in to 6.2.4 - V2X Pcmax in Configured transmitted power for V2X con-current operation is referring to 6.2E.4 but then its points to itself, changed it to 6.2E.4.1 which is Pcmax for V2X |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R4-2117103: UE cannot meet NS requirement R4-2117668: The relevant requirements remain unclear/duplicated. R4-2117100: For 1) and 2), it is difficult to implement n74 due to severe own Rx protection of -50 dBm/MHz at 1475 – 1488 MHz. For 3), the frequency range of general co-existence requirements with NOTE 41 and additional requirements for NS_38 are overlapping. For 4), the transmisson power condition for EESS protection is unclear in UE coexistence table. R4-2118967: Inconsistency due to the nature of the parameters and how it is specified in FR2. R4-2118971: It is not possible to configure CSI-RS periodicity in Table A.3.1-1 in TS 38.101-1 due to the limitation by TS 38.214 R4-2119845: The UE has no MOP requirements for the missing MOP combos R4-2119847: The US inter-band CA combinations with Band n77 where the MSD test points for n77 do not fall within US Band n77 frequency ranges are not testable. R4-2118455: Note 5 and Note 9 are duplicated. R4-2119848: The ASE requirements for NS_52 which is not aligned with current regulation may be verified. R4-2119849: SRS-TxSwitch capability IE is not aligned with 38.331. R4-2119850: The UE maximum output power reduction will be inaccurate. R4-2119291: It is unclear on how to correctly apply the SCS for n90. R4-2119851: Overly stringent requirements for power class 5 R4-2119497: V2X PCmax is erroneous and implemnetaition is not possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5, 5.4.2.3, 5.5B, 6.2.2, 6.2.3.9, 6.2.3.10, 6.2.4, 6.2A.1.3, 6.2A.3.1.3, 6.2D, 6.4D.4, 6.2E.4.1, 6.2E.4.2, 6.5.3.2, 6.5E.2.3.2, 7.3A.4, 7.3A.5, Annex A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R4-2117100: Note that the frequency range of 1440-1460MHz is covered by both lower and upper duplexer. The original problem was that -50dBm was extended to the upper duplexer range where the value defined in general is not feasible. Then, respecting the original thought, the range of 1440-1460MHz should support -50dBm/MHz protection requirements. As a result, it this leads to an assumption that the range of 1440-1460MHz should be covered by the lower duplexer. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |