3GPP TSG-RAN WG4 Meeting #102-e	R4-2204474

# Electronic Meeting, 21 Feb. 2022 - 03 Mar. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for idle mode UE meausrement capability in NTN. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add NTN specification into 38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new clauses 4.2B.1 and 4.2B.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN RRM specification for TR 38.133 is not made. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2B.1 and 4.2B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | No |  |  |  |  |  |  |  |  |

<<Start of Change>>

4.2B	Cell Re-selection in NTN

4.2B.1	Introduction

The cell reselection procedure allows the NTN UE to select a more suitable cell and camp on it.

When the NTN UE is in either Camped Normally state or Camped on Any Cell state on a cell, the NTN UE shall attempt to detect, synchronise, and monitor intra-frequency, inter-frequency and inter-RAT cells indicated by the serving cell. For intra-frequency and inter-frequency cells the serving cell may not provide explicit neighbour list but carrier frequency information and bandwidth information only. NTN UE measurement activity is also controlled by measurement rules defined in TS 38.304 [1], allowing the NTN UE to limit its measurement activity

4.2B.2	Requirements

4.2B.2.1	NTN UE measurement capability

For idle mode cell re-selection purposes, the NTN UE shall be capable of monitoring at least:

-	Intra-frequency carrier, and

-	Depending on UE capability, [TBD] NR inter-frequency carriers, and

-	Depending on UE capability, [TBD] FDD E-UTRA inter-RAT carriers, and

-	Depending on UE capability, [TBD] TDD E-UTRA inter-RAT carriers.

In addition to the requirements defined above, a NTN UE supporting E-UTRA measurements in RRC_IDLE state shall be capable of monitoring a total of at least [TBD] carrier frequency layers, which includes serving layer, comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD and NR layers.

<<End of Change>>